| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0053-v1 |
| --- | --- |

| Title: | AHG9: Signalling tile partitioning |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Shih-Ta Hsiang, Chia-Ming Tsai, Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {shih-ta.hsiang, chia-ming.tsai, yuwen.huang, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

- Abstract

This contribution proposes three high-level syntax modifications related to signalling tile partitioning of the coded picture, summarized as follows:

- The number of explicitly provided tile column widths or tile row heights is allowed to be equal to 0. When no_pic_partition_flag is equal to 0 and the coded picture contains only one tile, the proposed method can infer the tile column width and the tile row height to be equal to the picture width and the picture height, respectively, without signalling the syntax elements tile_column_width_minus1[ 0 ] and tile_row_height_minus1[ 0 ] for deriving the tile column width and the tile row height.

- pps_log2_ctu_size_minus5 is signalled in the picture parameter set (PPS) only when rect_slice_flag is equal to 0, single_slice_per_subpic_flag is equal to 0 and num_slices_in_pic_minus1 is greater than 0.

- loop_filter_across_tiles_enabled_flag is signalled only when the number of the tiles in the coded picture is greater than 1.

Proposed modifications

Proposal 1: Allowing not explicitly signalling any tile column width or row height

In Versatile Video Coding (VVC) Draft 8 as specified in [1], when the syntax element no_pic_partition_flag is equal to 0, the syntax elements num_exp_tile_columns_minus1 and num_exp_tile_rows_minus1 are signalled for deriving the number of the explicitly provided tile column widths and the number of explicitly provided tile row heights, respectively. The value of num_exp_tile_columns_minus1 shall be in the range of 0 to PicWidthInCtbsY − 1, inclusive. The value of num_exp_tile_rows_minus1 shall be in the range of 0 to PicHeightInCtbsY − 1, inclusive. When no_pic_partition_flag is equal to 0, at least one tile column width and one tile row height shall be explicitly signalled in the sequence parameter set (SPS).

In the proposed method, the number of explicitly provided tile column widths and the number of explicitly provided tile row heights are both allowed to be equal to 0. The syntax elements num_exp_tile_columns and num_exp_tile_rows replace the syntax elements num_exp_tile_columns_minus1 and num_exp_tile_rows_minus1 for specifying the number of explicitly provided tile column widths and the number of explicitly provided tile row heights, respectively. When the number of explicitly provided tile column widths or row heights is equal to 0, it is inferred that the tile column width or row height is equal to the picture width or height in units of coding tree blocks (CTBs). When there is no tiling in the coded picture, the proposed method only needs to spend two raw bins to signal that both num_exp_tile_columns and num_exp_tile_rows are equal to 0. It can be further inferred that the coded picture contains one tile with the tile column width and the tile row height equal to the picture width and the picture height in units of CTBs, respectively. In constrast, the current syntax in VVC Draft 8 will need to further signal two syntax elements tile_column_width_minus1[ 0 ] equal to PicWidthInCtbsY − 1 and tile_row_height_minus1[ 0 ] equal to PicHeightInCtbsY – 1 when there is no tiling in the coded picture.

Proposal 2: Conditionally signalling pps_log2_ctu_size_minus5

In VVC Draft 8, the syntax element pps_log2_ctu_size_minus5 is signalled in the PPS for deriving the luma CTB size. The information may be needed for derivation of the tile partitioning and the slice partitioing of the coded picture. It is a requirement of bitstream conformance that pps_log2_ctu_size_minus5 shall be equal to the value of the syntax element sps_log2_ctu_size_minus5 in the SPS. pps_log2_ctu_size_minus5 is signalled in the PPS such that the PPS can be parsed independently from the SPS.

However, information on the tile partitioning of a coded picture is needed for parsing the PPS only when the rectangular slice mode (rect_slice_flag=1) is used and the slice partitioning of the picture is not inferred from the subpicture partitioning of the picture (single_slice_per_subpic_flag = 0). In the proposed method, pps_log2_ctu_size_minus5 is signalled in the PPS only when rect_slice_flag is equal to 1, single_slice_per_subpic_flag is equal to 0, and num_slices_in_pic_minus1 is greater than 0.

Proposal 3: Conditionally signalling loop_filter_across_tiles_enabled_flag

The syntax element loop_filter_across_tiles_enabled_flag specifying whether in-loop filtering operations is applied across tile boundaries is relevant only for the coded pictures containing more than one tile. In the proposed method, loop_filter_across_tiles_enabled_flag is signalled conditioned on whether the number of the tiles in the coded picture is greater than 1.

Modifications to specification

The proposed modifications to VVC Draft 8 are written in red and provided below:

- 6.5.1 CTB raster scanning, tile scanning, and subpicture scanning processes

The variable NumTileColumns, specifying the number of tile columns, and the list colWidth[ i ] for i ranging from 0 to NumTileColumns − 1, inclusive, specifying the width of the i-th tile column in units of CTBs, are derived as follows:

if( num_exp_tile_columns = = 0 ) {

colWidth[0] = PicWidthInCtbsY
   		NumTileColumns = 1

} else {

remainingWidthInCtbsY = PicWidthInCtbsY
for( i = 0; i < num_exp_tile_columns_minus1 − 1; i++ ) {
	colWidth[ i ] = tile_column_width_minus1[ i ] + 1
	remainingWidthInCtbsY  −=  colWidth[ i ]
}
uniformTileColWidth = tile_column_width_minus1[ num_exp_tile_columns_minus1 − 1 ] + 1	
while( remainingWidthInCtbsY  >=  uniformTileColWidth ) {
	colWidth[ i++ ] = uniformTileColWidth
	remainingWidthInCtbsY  −=  uniformTileColWidth
}
if( remainingWidthInCtbsY > 0 )
	colWidth[ i++ ] = remainingWidthInCtbsY
NumTileColumns = i

}

The variable NumTileRows, specifying the number of tile rows, and the list RowHeight[ j ] for j ranging from 0 to NumTileRows − 1, inclusive, specifying the height of the j-th tile row in units of CTBs, are derived as follows:

if( num_exp_tile_rows = = 0 ) {

RowHeight [0] = PicHeightInCtbsY
   		NumTileRows = 1

} else {

remainingHeightInCtbsY = PicHeightInCtbsY
for( j = 0; j < num_exp_tile_rows_minus1 − 1; j++ ) {
	RowHeight[ j ] = tile_row_height_minus1[ j ] + 1
	remainingHeightInCtbsY  −=  RowHeight[ j ]
}
uniformTileRowHeight = tile_row_height_minus1[ num_exp_tile_rows_minus1 − 1 ] + 1	
while( remainingHeightInCtbsY  >=  uniformTileRowHeight ) {
	RowHeight[ j++ ] = uniformTileRowHeight
	remainingHeightInCtbsY  −=  uniformTileRowHeight
}
if( remainingHeightInCtbsY > 0 )
	RowHeight[ j++ ] = remainingHeightInCtbsY
NumTileRows = j

}

- 7.3.2.4 Picture parameter set RBSP syntax

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| pps_pic_parameter_set_id | ue(v) |
| …. |  |
| no_pic_partition_flag | u(1) |
| if( !no_pic_partition_flag ) { |  |
| pps_log2_ctu_size_minus5 | u(2) |
| num_exp_tile_columns_minus1 | ue(v) |
| num_exp_tile_rows_minus1 | ue(v) |
| for( i = 0; i <= num_exp_tile_columns_minus1 < num_exp_tile_columns; i++ ) |  |
| tile_column_width_minus1[ i ] | ue(v) |
| for( i = 0; i <= num_exp_tile_rows_minus1 < num_exp_tile_rows; i++ ) |  |
| tile_row_height_minus1[ i ] | ue(v) |
| MultipleTilesInPic = num_exp_tile_columns ¦ ¦ num_exp_tile_rows |  |
| if( NumTilesInPic > 1 MultipleTilesInPic ) |  |
| rect_slice_flag | u(1) |
| if( rect_slice_flag ) |  |
| single_slice_per_subpic_flag | u(1) |
| if( rect_slice_flag && !single_slice_per_subpic_flag ) { |  |
| num_slices_in_pic_minus1 | ue(v) |
| if( num_slices_in_pic_minus1 > 0 ) { |  |
| tile_idx_delta_present_flag | u(1) |
| if( MultipleTilesInPic ) |  |
| pps_log2_ctu_size_minus5 | u(2) |
| } |  |
| for( i = 0; i < num_slices_in_pic_minus1; i++ ) { |  |
| if( NumTileColumns > 1 ) |  |
| slice_width_in_tiles_minus1[ i ] | ue(v) |
| if( NumTileRows > 1 && ( tile_idx_delta_present_flag ¦ ¦ SliceTopLeftTileIdx[ i ] % NumTileColumns = = 0 ) ) |  |
| slice_height_in_tiles_minus1[ i ] | ue(v) |
| if( slice_width_in_tiles_minus1[ i ] = = 0 && slice_height_in_tiles_minus1[ i ] = = 0 && RowHeight[ SliceTopLeftTileIdx[ i ] / NumTileColumns ] > 1 ) { |  |
| num_exp_slices_in_tile[ i ] | ue(v) |
| for( j = 0; j < num_exp_slices_in_tile[ i ]; j++ ) |  |
| exp_slice_height_in_ctus_minus1[ i ][ j ] | ue(v) |
| i += NumSlicesInTile[ i ] − 1 |  |
| } |  |
| if( tile_idx_delta_present_flag && i < num_slices_in_pic_minus1 ) |  |
| tile_idx_delta[ i ] | se(v) |
| } |  |
| } |  |
| if( MultipleTilesInPic ) |  |
| loop_filter_across_tiles_enabled_flag | u(1) |
| loop_filter_across_slices_enabled_flag | u(1) |
| …. |  |
| } |  |

- 7.4.3.4 Picture parameter set RBSP semantics

num_exp_tile_columns_minus1 plus 1 specifies the number of explicitly provided tile column widths. The value of num_exp_tile_columns shall be in the range of 0 to PicWidthInCtbsY − 1, inclusive. When no_pic_partition_flag is equal to 1, the value of num_exp_tile_columns is inferred to be equal to 0.

num_exp_tile_rows_minus1 plus 1 specifies the number of explicitly provided tile row heights. The value of num_exp_tile_rows shall be in the range of 0 to PicHeightInCtbsY − 1, inclusive. When no_pic_partition_flag is equal to 1, the value of num_tile_rows is inferred to be equal to 0.

tile_column_width_minus1[ i ] plus 1 specifies the width of the i-th tile column in units of CTBs for i in the range of 0 to num_exp_tile_columns_minus1 − 1 − 2, inclusive, when num_exp_tile_columns is greater than 1. tile_column_width_minus1[ num_exp_tile_columns_minus1 − 1] is used to derive the width of the tile columns with index greater than or equal to num_exp_tile_columns_minus1 – 1 as specified in clause 6.5.1 when num_exp_tile_columns is greater than 0. The value of tile_column_width_minus1[ i ] shall be in the range of 0 to PicWidthInCtbsY − 1, inclusive. When not present, the value of tile_column_width_minus1[ 0 ] is inferred to be equal to PicWidthInCtbsY − 1.

tile_row_height_minus1[ i ] plus 1 specifies the height of the i-th tile row in units of CTBs for i in the range of 0 to num_exp_tile_rows_minus1 − 1  − 2, inclusive, when num_exp_tile_rows is greater than 1. tile_row_height_minus1[ num_exp_tile_rows_minus1 − 1 ] is used to derive the height of the tile rows with index greater than or equal to num_exp_tile_rows_minus1 – 1 as specified in clause 6.5.1 when num_exp_tile_rows is greater than 0. The value of tile_row_height_minus1[ i ] shall be in the range of 0 to PicHeightInCtbsY − 1, inclusive. When not present, the value of tile_row_height_minus1[ 0 ] is inferred to be equal to PicHeightInCtbsY − 1.

References

- B. Bross, J. Chen, and S. Liu, “Versatile Video Coding (Draft 8),” Document of Joint Video Experts Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-Q2001, 17th Meeting: Brussels, BE, 7–17 January 2020.

Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).