| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0055-v1 |
| --- | --- |

| Title: | AHG15: On referencing a non-existent scaling list |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chen-Yen Lai, Olena Chubach, Ching-Yeh Chen, Tzu-Der Chuang, Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {jenny.lai, olena.chubach, chingyeh.chen, peter.chuang, yuwen.huang, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

In this contribution, three methods to fix the issue of referencing a non-existent scaling list are presented. It is proposed to infer a non-existent scaling list to be the pre-defined matrix with all elements equal to 16 according to scaling_list_copy_mode_flag and scaling_list_pred_id_delta.

# Introduction

In the current VVC design [1], scaling list is signalled in adaptation parameter sets (APSs) which have corresponding aps_params_type equal to SCALING_APS. When decoding scaling lists, scaling_list_copy_mode_flag[id] and scaling_list_pred_mode_flag[id] are used to indicate methods of scaling list derivation for each scaling list with id from 0 to 27. Moreover, scaling_list_pred_id_delta[id] is used to indicate the predictor’s index of a corresponding scaling list with id.

For example, if scaling_list_copy_mode_flag[18] is equal to 1, the scaling list with id equal to 18 can be derived by directly copying from a previously decoded scaling list with index equal to 18 - scaling_list_pred_id_delta[18]. In another example, if scaling_list_copy_mode_flag[18] is equal to 1 and scaling_list_pred_id_delta[18] is equal to 0, this scaling list will be inferred as a flat matrix with all elements equal to 16.

At the last JVET meeting, a special handling of scaling lists for the case of 4:0:0 colour format was adopted. There, considering that both chroma components are not present in the current picture, scaling lists for chroma components will not be coded. To avoid signalling chroma scaling lists, an additional scaling_list_chroma_present_flag is added in the APS with aps_params_type equal to SCALING_APS. This flag is further used during the signalling of chroma scaling lists. It is a bitstream conformance requirement, that scaling_list_chroma_present_flag shall be equal to 0 when the colour format of current picture is equal to monochrome; otherwise, scaling_list_chroma_present_flag shall be equal to 1.

This adoption, while claiming to solve the issue of “signalling the scaling lists which are not used in 4:0:0”, creates another issue. Namely, referencing a non-existent scaling list, which is not defined in VVC Draft 8. This can happen if a non-existent chroma scaling list is used for prediction or is copied by the current coded scaling list.

# Proposed methods

In VVC Draft 8, when scaling_list_chroma_present_flag is equal to 0, no scaling list related syntax elements will be signalled for chroma components, including scaling_list_copy_mode_flag[id], scaling_list_pred_mode_flag[id], and scaling_list_pred_id_delta[id], and so on.

## Method 1

To minimize the modification of scaling list syntax in VVC Draft 8, it is proposed to determine a non-existent scaling list according to the existence of the scaling_list_copy_mode_flag[id] and scaling_list_pred_id_delta[id]. That is, when the scaling list related syntax elements are not present, scaling_list_copy_mode_flag[id] is inferred to be equal to 1, and scaling_list_pred_id_delta[id] is inferred to be equal to 0. Thus, the non-existent scaling list will be derived as a flat matrix with elements all equal to 16. To achieve that, it is proposed to add the following sentence marked with blue colour to the semantics of scaling_list_copy_mode_flag when it is not present:

scaling_list_copy_mode_flag[ id ] equal to 1 specifies that the values of the scaling list are the same as the values of a reference scaling list. The reference scaling list is specified by scaling_list_pred_id_delta[ id ]. scaling_list_copy_mode_flag[ id ] equal to 0 specifies that scaling_list_pred_mode_flag is present. When not present, the value of scaling_list_copy_mode_flag[ id ] is inferred to be equal to 1.

After this modification, when a referenced scaling list is not present in the APS, this scaling list can be derived by inferring it as a flat matrix with elements all equal to 16.

## Method 2

It is proposed to remove the scaling_list_chroma_present_flag from the VVC specification draft. Therefore, the scaling list signalling design will become the same as the one of VVC Draft 7, meaning all the scaling lists will be signalled, and there will be no issue when a non-existent scaling list is referenced during coding of the current scaling list.

## Method 3

It is proposed to limit the values of id which can be used for encoding a current scaling list when scaling_list_chroma_present_flag is equal to 0. Therefore, unavailable chroma scaling lists shall not be used for coding of the current scaling list. For this, it is proposed to add a following bitstream conformance requirement: When scaling_list_chroma_present_flag is equal to 0 and refId is not equal to 27, refId % 3 shall be equal to 2.

# Reference

[1] B. Bross, J. Chen, S. Liu, and Y.-K. Wang, “Versatile Video Coding (Draft 8),” Document of Joint Video Experts Team, JVET-Q2001.

[2] H. Zhang, X. Li, G. Li, L. Li, and S. Liu, “AHG15: Improvement for quantization matrix signalling,” Document of Joint Video Experts Team, JVET-Q0505.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).