| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0056-v2 |
| --- | --- |

| Title: | LFNST complexity reduction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tzu-Der Chuang, Man-Shu Chiang, Zhi-Yi Lin, Chih-Wei Hsu, Ching-Yeh Chen,Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {peter.chuang, man-shu.chiang, zhi-yi.lin, cw.hsu, chingyeh.chen, yuwen.huang, shawmin.lei}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

- Abstract

In VVC Draft 8, the low frequency non-separable transform (LFNST) is applied to luma only and not applied to chroma when the current coding unit (CU) is coded in single tree structure to reduce the LFNST complexity. However, in dual tree structure, the LFNST still can be applied to all three colour components, so the worst case LFNST complexity is not reduced. In this contribution, two methods are proposed to reduce the worst case LFNST complexity by only applying the LFNST to at most one colour component per CU. In the method-1, the chroma LFNST is disabled. The LFNST index is moved after the first nonzero luma transform block (TB). In the method-2, for chroma tree, the LFNST is only applied to the transformed coefficient of the first nonzero chroma TB. The LFNST index is moved after the first nonzero luma TB in single tree and luma tree, and moved after the first nonzero chroma TB in chroma tree. The results reportedly show 0.14%/1.18%/1.35% Y/Cb/Cr BD-rate under AI and 0.08%/0.28%/0.37% Y/Cb/Cr BD-rate under RA for method-1. For method-2, the results reportedly show 0.04%/0.38%/0.60% Y/Cb/Cr BD-rate under AI and 0.02%/0.13%/0.27% Y/Cb/Cr BD-rate under RA. It is asserted that 33% (for 4:2:0) / 66% (for 4:4:4) and 17% (for 4:2:0) / 33% (for 4:4:4) worst case LFNST complexity is reduced by method-1 and method-2, respectively, and issues of processing latency and coefficient buffering for LFNST in hardware decoding are solved.

Introduction

In VVC Draft 8 [1], the LFNST is only applied to luma when the current CU is coded in single tree structure to reduce the complexity [2]. However, it only reduces the complexity for single tree coded CU. The worst case LFNST complexity is still unchanged and happens when LFNST is applied to luma and both chroma components in dual tree coded CU. Meanwhile, the LFNST index and multiple transform selection (MTS) index are signalled at the end of CU. The inverse transform cannot be applied until the LFNST index and MTS index are decoded, which introduces issues of undesirable processing latency, additional coefficient buffering, and complex buffer sharing control in hardware decoders. In this contribution, two methods are proposed to further reduce the worst case LFNST complexity. In both methods, the LFNST can be applied to at most one colour component per CU. Moreover, the LFNST index is moved after the luma TB or the first nonzero chroma TB to solve the above mentioned hardware decoder issues.

Proposed methods

Method-1: Disable LFNST for chroma in both single tree and dual tree

In method-1, it is proposed to disable the LFNST for chroma components in not only single tree coded CU but also dual tree coded CU. In chroma tree coded CU, the LFNST index is inferred as 0. In this proposed method, since the LFNST and MTS are both applied to luma component only, the LFNST index and MTS index are moved after the first nonzero luma TB to solve the processing latency and coefficient buffering issues. The lfnstNotTsFlag (checking whether one of the TB in the current CU is coded in transform skip mode) is also removed. With the proposed method, the worst case LFNST complexity is reduced by 33% in 4:2:0 chroma format or 66% in 4:4:4 chroma format.

Method-2: Only apply LFNST to first nonzero chroma TB in chroma tree

In method-2, it is proposed to apply the LFNST to at most one colour component per CU. In single tree or luma dual tree coded CU, the LFNST is only applied to luma component. In chroma tree coded CU, the LFNST is only applied to the first nonzero chroma TB. For the joint coding of chroma residual mode (joint-cb-cr mode) in chroma tree, only one chroma TB is coded. The LFNST can be applied to the joint coding of chroma residual mode. In this proposed method, for single tree or luma tree coded CU, the LFNST index and MTS index are moved after the first nonzero luma TB. For chroma tree coded CU, the LFNST index is moved after the first nonzero chroma TB. The processing latency and coefficient buffering issues can be solved. The lfnstNotTsFlag is also removed. With the proposed method, the worst case LFNST complexity is reduced by 17% in 4:2:0 chroma format or 33% in 4:4:4 chroma format.

Experiment results

The proposed methods have been implemented on VTM8.0 [3]. Tables 1 and 2 show the results of the proposed method-1 and method-2 under the common test condition (CTC) [4], respectively. The results reportedly show 0.14%/1.18%/1.35% Y/Cb/Cr BD-rate under AI and 0.08%/0.28%/0.37% Y/Cb/Cr BD-rate under RA for method-1. For method-2, the results reportedly show 0.04%/0.38%/0.60% Y/Cb/Cr BD-rate under AI and 0.02%/0.13%/0.27% Y/Cb/Cr BD-rate under RA. Tables 3 and 4 show the results of the proposed method-1 and method-2 under the CTC but using single tree structure for I-slice (DualITree is set to be 0), respectively. The results reportedly show 0.07%/0.30%/0.37% Y/Cb/Cr BD-rate under AI and 0.06%/-0.02%/0.10% Y/Cb/Cr BD-rate under RA for method-1. For method-2, the results reportedly show 0.02%/0.11%/0.16% Y/Cb/Cr BD-rate under AI and 0.02%/0.08%/0.01% Y/Cb/Cr BD-rate under RA.

Table 1.  Results of the proposed method-1

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.04% | 1.55% | 1.96% | 95% | 99% |
| Class A2 | -0.04% | 1.55% | 1.62% | 92% | 97% |
| Class B | 0.13% | 1.10% | 1.45% | 96% | 101% |
| Class C | 0.28% | 0.72% | 0.95% | 93% | 101% |
| Class E | 0.26% | 1.19% | 0.82% | 95% | 102% |
| Overall | 0.14% | 1.18% | 1.35% | 94% | 100% |
| Class D | 0.25% | 0.88% | 0.98% | 93% | 102% |
| Class F | 0.22% | 0.49% | 0.44% | 97% | 101% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.06% | 0.45% | 0.46% | 101% | 103% |
| Class A2 | 0.02% | 0.45% | 0.59% | 101% | 101% |
| Class B | 0.05% | 0.63% | 0.79% | 101% | 101% |
| Class C | 0.18% | -0.41% | -0.41% | 101% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.08% | 0.28% | 0.37% | 101% | 101% |
| Class D | 0.19% | 0.23% | 0.44% | 101% | 102% |
| Class F | 0.25% | -0.24% | 0.17% | 101% | 101% |

Table 2.  Results of the proposed method-2

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.03% | 0.74% | 0.60% | 101% | 99% |
| Class A2 | 0.01% | 0.45% | 0.43% | 100% | 98% |
| Class B | 0.05% | 0.27% | 0.68% | 101% | 100% |
| Class C | 0.08% | 0.25% | 0.55% | 100% | 101% |
| Class E | 0.03% | 0.29% | 0.73% | 101% | 102% |
| Overall | 0.04% | 0.38% | 0.60% | 101% | 100% |
| Class D | 0.07% | 0.55% | 0.63% | 100% | 101% |
| Class F | 0.03% | 0.26% | 0.50% | 101% | 102% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.07% | 0.20% | 0.27% | 100% | 101% |
| Class A2 | 0.01% | 0.02% | 0.23% | 100% | 100% |
| Class B | 0.01% | 0.09% | 0.38% | 101% | 98% |
| Class C | 0.00% | 0.20% | 0.18% | 101% | 103% |
| Class E |  |  |  |  |  |
| Overall | 0.02% | 0.13% | 0.27% | 100% | 100% |
| Class D | 0.06% | 0.54% | 0.41% | 100% | 101% |
| Class F | 0.09% | -0.10% | 0.35% | 100% | 98% |

Table 3.  Results of the proposed method-1 in single tree structure for I-slice

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.15% | 0.20% | 98% | 103% |
| Class A2 | -0.02% | 0.38% | 0.44% | 98% | 100% |
| Class B | 0.04% | 0.39% | 0.49% | 98% | 101% |
| Class C | 0.18% | 0.36% | 0.47% | 97% | 101% |
| Class E | 0.11% | 0.12% | 0.15% | 99% | 101% |
| Overall | 0.07% | 0.30% | 0.37% | 98% | 101% |
| Class D | 0.22% | 0.54% | 0.74% | 98% | 102% |
| Class F | 0.12% | 0.45% | 0.42% | 100% | 101% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.05% | -0.06% | 0.24% | 100% | 98% |
| Class A2 | 0.03% | 0.10% | 0.16% | 99% | 97% |
| Class B | 0.03% | 0.24% | 0.35% | 101% | 98% |
| Class C | 0.15% | -0.39% | -0.36% | 101% | 102% |
| Class E |  |  |  |  |  |
| Overall | 0.06% | -0.02% | 0.10% | 100% | 99% |
| Class D | 0.15% | 0.02% | 0.37% | 101% | 102% |
| Class F | 0.17% | 0.05% | -0.01% | 100% | 99% |

Table 4.  Results of the proposed method-2 in single tree structure for I-slice

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.04% | 0.12% | 99% | 104% |
| Class A2 | -0.01% | 0.11% | 0.12% | 100% | 100% |
| Class B | 0.02% | 0.12% | 0.23% | 100% | 103% |
| Class C | 0.05% | 0.14% | 0.17% | 99% | 107% |
| Class E | 0.04% | 0.13% | 0.09% | 100% | 102% |
| Overall | 0.02% | 0.11% | 0.16% | 100% | 103% |
| Class D | 0.09% | 0.30% | 0.46% | 100% | 103% |
| Class F | 0.03% | 0.15% | 0.21% | 100% | 101% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.05% | -0.16% | 0.04% | 101% | 102% |
| Class A2 | 0.02% | 0.06% | -0.02% | 102% | 99% |
| Class B | 0.02% | 0.23% | 0.04% | 102% | 103% |
| Class C | 0.01% | 0.10% | -0.01% | 102% | 102% |
| Class E |  |  |  |  |  |
| Overall | 0.02% | 0.08% | 0.01% | 102% | 102% |
| Class D | 0.07% | 0.41% | 0.72% | 102% | 102% |
| Class F | 0.04% | 0.24% | 0.42% | 101% | 102% |

Conclusions

This contribution presents two methods to reduce the worst case LFNST complexity. In the proposed method-1, the chroma LFNST is disabled. In the proposed method-2, for chroma tree, the LFNST is only applied to the transformed coefficients of the first nonzero TB. The LFNST index is moved after the first nonzero luma TB or the first nonzero chroma TB to solve the processing latency and buffering issues. The results reportedly show 0.14%/1.18%/1.35% Y/Cb/Cr BD-rate under AI and 0.08%/0.28%/0.37% Y/Cb/Cr BD-rate under RA for method-1. For method-2, the results reportedly show 0.04%/0.38%/0.60% Y/Cb/Cr BD-rate under AI and 0.02%/0.13%/0.27% Y/Cb/Cr BD-rate under RA.

References

- B. Bross, J. Chen, and S. Liu, “Versatile Video Coding (Draft 8),” Document of Joint Video Experts Team, JVET-Q2001.

- H. E. Egilmez, A. Nalci, A. K. Ramasubramonian, M. Coban, V. Seregin, M. Karczewicz, T. Hashimoto, T. Chujoh, E. Sasaki, T. Ikai, “Combination of JVET-Q0106, JVET-0686 and JVET-Q0133 for LFNST signalling, latency reduction, and scaling process,” Document of Joint Video Experts Team, JVET-Q0784.

- VTM8.0, available at https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM

- F. Bossen, J. Boyce, X. Li, V. Seregin, and K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” Document of Joint Video Experts Team, JVET-N1010.

Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).