| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0058-v4 |
| --- | --- |

| Title: | AHG8/AHG9/AHG12: On the combination of RPR, subpictures, and scalability |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ye-Kui Wang1, Li Zhang1, Kai Zhang1 Zhipin Deng2 18910 University Center Lane San Diego, CA 92122, USA 2Zijinshumayuan 4th building F9 18 Zhongguancun Nansijie Haidian District, 100190, Beijing, China | Tel: Email: | yekui.wang@bytedance.com lizhang.idm@bytedance.com zhangkai.video@bytedance.com zhipin.deng@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

This contribution proposes the following changes related to the combination of RPR, subpictures, and scalability:

- Update the constraint on the combination of subpictures and scalability (in the semantics of subpic_treated_as_pic_flag[ i ]) as follows:

  - To impose cross-layer alignment restrictions on all layers in each dependency tree, independent of whether any of the layers is an output layer of an OLS.

  - To require cross-layer alignment of subpic_treated_as_pic_flag[ i ].

  - To not require cross-layer alignment of loop_filter_across_subpic_enabled_flag[ i ].

  - To not impose cross-layer alignment restrictions when sps_num_subpics_minus1 is equal to 0.

  - To clearly specify the applicable scope through the target set of AUs specified as follows: For each CLVS of a current layer referring to the SPS, let the target set of AUs targetAuSet be all the AUs starting from the AU containing the first picture of the CLVS in decoding order to the AU containing the last picture of the CLVS in decoding order, inclusive.

  - To impose cross-layer alignment restrictions only on the current layer and all the higher layers that depend on the current layer, while not on the higher layers that do not depend on the current layer or on the lower layers.

- Update the following RPR aspects related to the combination of RPR, subpictures, and scalability as follows:

  - Instead of having just one SPS flag for controlling RPR, use two SPS flags (ref_pic_resampling_enabled_flag and res_change_in_clvs_allowed_flag), one for controlling the use of RPR as a tool, and the other for controlling whether the picture resolution can change within the CLVS.

  - Consequently, also specify two general constraint flags, one for each of these two flags.

- In the decoding processes involving the clipping operations for treating subpicture boundaries in motion compensation and motion prediction as picture boundaries, the condition on when to apply the clipping operations is slightly updated such that the clipping operation is not applied when the reference picture is not split into multiple subpictures.

In -v2 of this contribution, the aspect on disallowing the collocated picture of a current picture to be an LTRP or ILRP was dropped.

In -v3 of this contribution, an editorial chanage was made to the proposed text changes for clarity improvement.

In -v4 of this contribution, the following two aspects were dropped as it was pointed out and confirmed during further offline discussion that the constraints unnecessarily limit the functionalities of RPR and scalability:

  - To require cross-layer alignment of the scaling window parameters (for pictures having the same spatial resolution) for the combination of SNR scalability and multiple subpictures per picture.

  - To require all pictures with the same resolution within a CLVS to have the same scaling window.

# Introduction

The latest VVC draft text includes the following constraints that basically disallows any other combination of subpictures and scalability with inter-layer prediction (ILP) than a restricted combination of subpictures and SNR scalability:

When subpic_treated_as_pic_flag[ i ] is equal to 1, it is a requirement of bitstream conformance that all of the following conditions are true for each output layer and its reference layers in an OLS that includes the layer containing the i-th subpicture as an output layer:

–	All pictures in the output layer and its reference layers shall have the same value of pic_width_in_luma_samples and the same value of pic_height_in_luma_samples.

–	All the SPSs referred to by the output layer and its reference layers shall have the same value of sps_num_subpics_minus1 and shall have the same values of subpic_ctu_top_left_x[ j ], subpic_ctu_top_left_y[ j ], subpic_width_minus1[ j ], subpic_height_minus1[ j ], and loop_filter_across_subpic_enabled_flag[ j ], respectively, for each value of j in the range of 0 to sps_num_subpics_minus1, inclusive.

–	All pictures in each access unit in the output layer and its reference layers shall have the same value of SubpicIdVal[ j ] for each value of j in the range of 0 to sps_num_subpics_minus1, inclusive.

We observed the following issues regarding the above constraints:

- We could not figure out a reason why whether a layer is an output layer should make a difference herein. Rather, we think the constraint should also apply when the layer containing the i-th subpicture is not an output layer of an OLS. The entire constraint should be specified in a manner that does not consider whether a layer is an output layer of an OLS.

- The requirement for the value of subpic_treated_as_pic_flag[ i ] to be aligned across layers should be included, as otherwise the extraction of the subpicture sequence with the same subpicture index across the layers won't be possible.

- The requirement for the value of loop_filter_across_subpic_enabled_flag[ i ] to be aligned across layers should be excluded, as regardless of the value of this flag, as long as the subpic_treated_as_pic_flag[ i ] is equal to 1 the subpicture sequence is extractable. Setting of the value of loop_filter_across_subpic_enabled_flag[ i ] should be left for the encoder to decide for trading-off the quality of single extractable subpicture sequences vs the quality of sets of extractable subpicture sequences, just as why the two flags are signalled independently from each other.

- The entire constraint should only apply when sps_num_subpics_minus1 is greater than 0, to avoid the cases of one subpicture per subpicture being covered by the constraint, unintentionally.

- The temporal scope, i.e., the set of AUs, in which the constraint applies is not clearly specified.

- The requirement for the value of each of the scaling window parameters scaling_win_left_offset, scaling_win_right_offset, scaling_win_top_offset, and scaling_win_bottom_offset to be aligned across layers should be included, to make sure RPR for inter-layer reference pictures (ILRPs) is not needed when there are multiple subpictures per picture, as the latter is not supported in the latest VVC design.

- It is unnecessary to disallow a layer to use multiple subpictures per picture while its reference layer has just one subpicture per picture. As we know, a typical subpicture-based viewport-dependent 360o video delivery scheme is shown in Figure 1, wherein a higher-resolution representation of the full video consists of subpictures, while a lower-resolution representation of the full video does not use subpictures and can be coded with less frequent random access points than the higher-resolution representation. The client receives the full video in the lower-resolution and for the higher-resolution video it only receives and decode the subpictures that cover the current viewport.

Figure 1. A typical subpicture-based viewport-dependent 360o video coding scheme

By allowing a layer to use multiple subpictures per picture while its reference layer has just one subpicture per picture, the coding scheme shown in Figure 2 would be allowed, where the only difference compared to the approach shown in Figure 1 is that ILP is allowed.

Figure 2. A typical subpicture-based viewport-dependent 360o video coding scheme based on subpictures and spatial scalability with ILP

# Proposal

## On the combination of subpictures and scalability

The following solutions are proposed to address the above issues:

- To address the 1st issue, the constraint is updated to impose cross-layer alignment restrictions on all layers in each dependency tree. A dependency tree contains a particular layer, all layers that have the particular layer as a reference layer, and all reference layers of the particular layer), independent of whether any of the layers is an output layer of an OLS.

- To address the 2nd issue, the constraint is updated to additionally impose a cross-layer alignment restriction on the value of subpic_treated_as_pic_flag[ i ].

- To address the 3rd issue, the constraint is updated to not impose a cross-layer alignment restriction on the value of loop_filter_across_subpic_enabled_flag[ i ].

- To address the 4th issue, the constraint is updated such that the constraint does not apply when sps_num_subpics_minus1 is equal to 0.

- To address the 5th issue, the constraint is updated to impose cross-layer alignment restrictions on pictures in a target set of AUs specified as follows: For each CLVS of a current layer referring to the SPS, let the target set of AUs targetAuSet be all the AUs starting from the AU containing the first picture of the CLVS in decoding order to the AU containing the last picture of the CLVS in decoding order, inclusive.

- To address the 6th issue, the constraint is updated to impose a cross-layer alignment restriction for the value of each of the scaling window parameters scaling_win_left_offset, scaling_win_right_offset, scaling_win_top_offset, and scaling_win_bottom_offset.

- To address the 7th issue, the constraint is updated to impose cross-layer alignment restrictions only on the current layer and all the higher layers that depend on the current layer, while not on the higher layers that do not depend on the current layer or on the lower layers.

The above updates clean up the exising constraint on the combination of subpicures and scalability in general, and allow the coding scheme shown in Figure 2 with the layers having the same sptial resolution, i.e., with SNR scalability.

## On the combination of RPR, subpictures, and scalability

To further enable the coding scheme shown in Figure 2 with the layers having different sptial resolutions, the following existing constraint that disalows the combination of RPR and subpictures needs to be updated:

When res_change_in_clvs_allowed_flag is equal to 1, the value of subpic_info_present_flag shall be equal to 0.

Note that the spatial scalability design in VVC relies on the RPR feature. In other words, whenever RPR is turned off, spatial scalability is disabled.

One way is to change to the semantics of the res_change_in_clvs_allowed_flag such that, regardless of the value of the flag resampling of an ILRP is allowed when decoding a picture. However, this approach would forfeit the purpose of the SPS flag res_change_in_clvs_allowed_flag and the general constraint flag no_res_change_in_clvs_constraint_flag to be able to fully disable RPR as a tool.

To address the above issue, the following are proposed:

- Instead of having just one SPS flag for controlling RPR, use two SPS flags (ref_pic_resampling_enabled_flag and res_change_in_clvs_allowed_flag), one for controlling the use of RPR as a tool, and the other for controlling whether the picture resolution can change within the CLVS.

- Consequently, also specify two general constraint flags, one for each of these two flags.

## On clipping operations for treating subpicture boundaries as picture boundaries in motion compensation and motion prediction

When subpic_treated_as_pic_flag[ i ] is equal to 1, the subpicture boundaries of the i-th subpicture would be treated as picture boundaries in motion compensation. Such treating is realized in the VVC draft text by applying certain clipping operations in the decoding processes related to motion compensation and motion prediction. However, for the coding scheme shown in Figure 2, since the lower layer is fully available to the decoder, not just the area corrsponding to the i-th subpicture, such clipping does not need to be applied in this case, to avoid unnecessary coding efficiency loss.

Therefore, in the following decoding processes involving the clipping operations for treating subpicture boundaries in motion compensation and motion prediction as picture boundaries, the condition on when to apply the clipping operations is slightly updated such that the clipping operation is not applied when the reference layer picture is not split into multiple subpictures:

- 8.5.2.11 Derivation process for temporal luma motion vector prediction

- 8.5.3.2.2 Luma sample bilinear interpolation process

- 8.5.5.3 Derivation process for subblock-based temporal merging candidates

- 8.5.5.4 Derivation process for subblock-based temporal merging base motion data

- 8.5.5.6 Derivation process for constructed affine control point motion vector merging candidates

- 8.5.6.3.2 Luma sample interpolation filtering process

- 8.5.6.3.3 Luma integer sample fetching process

- 8.5.6.3.4 Chroma sample interpolation process

However, when the collocated picture is disallowed to be an LTRP or ILRP, those clauses wherein the reference picture is the collocated picture, i.e., clauses 8.5.2.11, 8.5.5.3, 8.5.5.4, and 8.5.5.6, do not need to be updated.

## Proposed text changes

#### 7.3.2.3 Sequence parameter set syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| ... |  |
| ref_pic_resampling_enabled_flag | u(1) |
| if( ref_pic_resampling_enabled_flag ) |  |
| res_change_in_clvs_allowed_flag | u(1) |
| pic_width_max_in_luma_samples | ue(v) |
| pic_height_max_in_luma_samples | ue(v) |
| ... |  |
| } |  |

#### 7.4.3.3 Sequence parameter set RBSP semantics

...

ref_pic_resampling_enabled_flag equal to 1 specifies that one or more slices of pictures in the CLVS may refer to a reference picture with a different spatial resolution in an active entry of a reference picture list. ref_pic_resampling_enabled_flag equal to 0 specifies that no slice of pictures in the CLVS refers to a reference picture with a different spatial resolution in an active entry of a reference picture list.

NOTE 2 – When ref_pic_resampling_enabled_flag is equal to 1, for a current picture the reference picture with a different spatial resolution may either belong to the same layer or a different layer than the layer containing the current picture.

res_change_in_clvs_allowed_flag equal to 1 specifies that the picture spatial resolution may change within a CLVS referring to the SPS. res_change_in_clvs_allowed_flag equal to 0 specifies that the picture spatial resolution does not change within any CLVS referring to the SPS. When not present, the value of res_change_in_clvs_allowed_flag is inferred to be equal to 0.

...

subpic_treated_as_pic_flag[ i ] equal to 1 specifies that the i-th subpicture of each coded picture in the CLVS is treated as a picture in the decoding process excluding in-loop filtering operations. subpic_treated_as_pic_flag[ i ] equal to 0 specifies that the i-th subpicture of each coded picture in the CLVS is not treated as a picture in the decoding process excluding in-loop filtering operations. When not present, the value of subpic_treated_as_pic_flag[ i ] is inferred to be equal to sps_independent_subpics_flag.

When sps_num_subpics_minus1 is greater than 0 and subpic_treated_as_pic_flag[ i ] is equal to 1, for each CLVS of a current layer referring to the SPS, let targetAuSet be all the AUs starting from the AU containing the first picture of the CLVS in decoding order, to the AU containing the last picture of the CLVS in decoding order, inclusive, it is a requirement of bitstream conformance that all of the following conditions are true for the targetLayerSet that consists of the current layer and all the layers that have the current layer as a reference layer each output layer and its reference layers in an OLS that includes the layer containing the i-th subpicture as an output layer:

–	All pictures of the AUs in targetAuSet and the layers in targetLayerSet in the output layer and its reference layers shall have the same value of pic_width_in_luma_samples and the same value of pic_height_in_luma_samples.

–	All the SPSs referred to by the layers in targetLayerSet output layer and its reference layers shall have the same value of sps_num_subpics_minus1 and shall have the same values of subpic_ctu_top_left_x[ j ], subpic_ctu_top_left_y[ j ], subpic_width_minus1[ j ], subpic_height_minus1[ j ], and subpic_treated_as_pic_flag[ j ] loop_filter_across_subpic_enabled_flag[ j ], respectively, for each value of j in the range of 0 to sps_num_subpics_minus1, inclusive.

–	For each AU in targetAuSet, all All pictures of the layers in targetLayerSet in each access unit in the output layer and its reference layers shall have the same value of SubpicIdVal[ j ] for each value of j in the range of 0 to sps_num_subpics_minus1, inclusive.

...

#### 7.4.3.4 Picture parameter set RBSP semantics

...

scaling_window_explicit_signalling_flag equal to 1 specifies that the scaling window offset parameters are present in the PPS. scaling_window_explicit_signalling_flag equal to 0 specifies that the scaling window offset parameters are not present in the PPS. When ref_pic_resampling_enabled_flag res_change_in_clvs_allowed_flag is equal to 0, the value of scaling_window_explicit_signalling_flag shall be equal to 0.

scaling_win_left_offset, scaling_win_right_offset, scaling_win_top_offset, and scaling_win_bottom_offset specify the offsets that are applied to the picture size for scaling ratio calculation. When not present, the values of scaling_win_left_offset, scaling_win_right_offset, scaling_win_top_offset, and scaling_win_bottom_offset are inferred to be equal to pps_conf_win_left_offset, pps_conf_win_right_offset, pps_conf_win_top_offset, and pps_conf_win_bottom_offset, respectively.

The value of SubWidthC * ( scaling_win_left_offset + scaling_win_right_offset ) shall be less than pic_width_in_luma_samples, and the value of SubHeightC * ( scaling_win_top_offset + scaling_win_bottom_offset ) shall be less than pic_height_in_luma_samples.

...

#### 7.3.3.2 General constraint information syntax

| general_constraint_info( ) { | Descriptor |
| --- | --- |
| ... |  |
| no_ref_pic_resampling_constraint_flag | u(1) |
| no_res_change_in_clvs_constraint_flag | u(1) |
| ... |  |
| } |  |

#### 7.4.4.2 General constraint information semantics

...

no_ref_pic_resampling_constraint_flag equal to 1 specifies that ref_pic_resampling_enabled_flag shall be equal to 0. no_ref_pic_resampling_constraint_flag equal to 0 does not impose such a constraint.

no_res_change_in_clvs_constraint_flag equal to 1 specifies that res_change_in_clvs_allowed_flag shall be equal to 0. no_res_change_in_clvs_constraint_flag equal to 0 does not impose such a constraint.

...

#### 8.5.3.2.2	Luma sample bilinear interpolation process

...

The luma locations in full-sample units ( xInti, yInti ) are derived as follows for i = 0..1:

–	If subpic_treated_as_pic_flag[ CurrSubpicIdx ] is equal to 1 and sps_num_subpics_minus1 for the reference picture refPicLX is greater than 0, the following applies:

xInti = Clip3( SubpicLeftBoundaryPos, SubpicRightBoundaryPos, xIntL + i )	(640)

yInti = Clip3( SubpicTopBoundaryPos, SubpicBotBoundaryPos, yIntL + i )	(641)

–	Otherwise (subpic_treated_as_pic_flag[ CurrSubpicIdx ] is equal to 0 or sps_num_subpics_minus1 for the reference picture refPicLX is equal to 0), the following applies:

xInti = Clip3( 0, picW − 1, refWraparoundEnabledFlag ?
		ClipH( ( PpsRefWraparoundOffset ) * MinCbSizeY, picW, ( xIntL + i ) ) : xIntL + i )	(642)

yInti = Clip3( 0, picH − 1, yIntL + i )					(643)

...

#### 8.5.6.3.2	Luma sample interpolation filtering process

...

–	If subpic_treated_as_pic_flag[ CurrSubpicIdx ] is equal to 1 and sps_num_subpics_minus1 for the reference picture refPicLX is greater than 0, the following applies:

xInti = Clip3( SubpicLeftBoundaryPos, SubpicRightBoundaryPos, xInti )	(959)

yInti = Clip3( SubpicTopBoundaryPos, SubpicBotBoundaryPos, yInti )	(960)

–	Otherwise (subpic_treated_as_pic_flag[ CurrSubpicIdx ] is equal to 0 or sps_num_subpics_minus1 for the reference picture refPicLX is equal to 0), the following applies:

xInti = Clip3( 0, picW − 1, refWraparoundEnabledFlag ?
		ClipH( ( PpsRefWraparoundOffset ) * MinCbSizeY, picW, xInti ) : xInti )	(961)

yInti = Clip3( 0, picH − 1, yInti )						(962)

...

#### 8.5.6.3.3	Luma integer sample fetching process

...

The luma locations in full-sample units ( xInt, yInt ) are derived as follows:

–	If subpic_treated_as_pic_flag[ CurrSubpicIdx ] is equal to 1 and sps_num_subpics_minus1 for the reference picture refPicLX is greater than 0, the following applies:

xInt = Clip3( SubpicLeftBoundaryPos, SubpicRightBoundaryPos, xIntL )	(968)

yInt = Clip3( SubpicTopBoundaryPos, SubpicBotBoundaryPos, yIntL )	(969)

–	Otherwise (subpic_treated_as_pic_flag[ CurrSubpicIdx ] is equal to 0 or sps_num_subpics_minus1 for the reference picture refPicLX is equal to 0), the following applies:

xInt = Clip3( 0, picW − 1, refWraparoundEnabledFlag ?	(970)
		ClipH( ( PpsRefWraparoundOffset ) * MinCbSizeY, picW, xIntL ) : xIntL )

yInt = Clip3( 0, picH − 1, yIntL )						(971)

...

#### 8.5.6.3.4	Chroma sample interpolation process

...

–	If subpic_treated_as_pic_flag[ CurrSubpicIdx ] is equal to 1 and sps_num_subpics_minus1 for the reference picture refPicLX is greater than 0, the following applies:

xInti = Clip3( SubpicLeftBoundaryPos / SubWidthC, SubpicRightBoundaryPos / SubWidthC, xInti )	(977)

yInti = Clip3( SubpicTopBoundaryPos / SubHeightC, SubpicBotBoundaryPos / SubHeightC, yInti )	(978)

–	Otherwise (subpic_treated_as_pic_flag[ CurrSubpicIdx ] is equal to 0 or sps_num_subpics_minus1 for the reference picture refPicLX is equal to 0), the following applies:

xInti = Clip3( 0, picWC − 1, refWraparoundEnabledFlag ? ClipH( xOffset, picWC, xInti ) :	(979)
		xIntC + i − 1 )

yInti = Clip3( 0, picHC − 1, yInti )						(980)

...

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).