| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0064-v1 |
| --- | --- |

| Title: | AHG9: Signalling of scaling list control |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ye-Kui Wang, Li Zhang, Kai Zhang 8910 University Center Lane San Diego, CA 92122, USA. | Tel: Email: | yekui.wang@bytedance.com lizhang.idm@bytedance.com zhangkai.video@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

This contribution proposes the following changes on signalling of scaling list control:

- The PH flag ph_explicit_scaling_list_enabled_flag is replaced with a 2-bit ph_explicit_scaling_list_mode_idc, with 3 modes specified: disabled (mode 0), used for all slices (mode 1), and enabled (mode 2). In mode 1, the explicit scaling list is used for all slices of a picture, and no scaling list signalling is needed in the SH.

- Move the flag scaling_matrix_for_lfnst_disabled_flag from the scaling_list_data( ) syntax to the SPS.

# Introduction

We observed the following issues with the current VVC design regarding scaling list control:

- When ph_explicit_scaling_list_enabled_flag is equal to 1, in most cases the explicit scaling list would be used for all slices of the picture. However, in the current VVC design, for the case when the explicit scaling list is enabled for all slices of a picture, not only ph_explicit_scaling_list_enabled_flag needs to be equal to 1, but the slice flag slice_explicit_scaling_list_used_flag with the value 1 needs to be signalled for each slice as well.

- The explicit scaling list was designed to determine the appropriate levels of quantization required to reduce unwanted video compression artifact. Therefore, it is typically dependent on the color component, block dimension and prediction mode, such as Intra/Inter/IBC. In VVC, even different types of transform matrixes (including primary transforms/secondary transforms) are supported, the explicit scaling lists are not signalled separately according to transform types. However, only for blocks using LFNST (a secondary transform), an on/off control flag is signalled in the APS. Since LFNST is only applied to intra coded blocks, the explicit scaling list applied to coded blocks using LFNST has to follow the explicit scaling list signalled for intra coded blocks. While it might be useful to turn on/off the use of explicit scaling list for intra coded blocks depending on whether LFNST is in use, we don't see a reason for the encoder to turn it on for some pictures in a CLVS and turn it off for the other pictures in the same CLVS. Therefore, the flag should be signalled in the SPS instead.

# Proposal

## Summary of the proposal

The following changes are proposed to address the above issues:

- The PH flag ph_explicit_scaling_list_enabled_flag is replaced with a 2-bit ph_explicit_scaling_list_mode_idc, with 3 modes specified: disabled (mode 0), used for all slices (mode 1), and enabled (mode 2). In mode 1, the explicit scaling list is used for all slices of a picture, and no scaling list signalling is needed in the SH.

- Move the flag scaling_matrix_for_lfnst_disabled_flag from the scaling_list_data( ) syntax to the SPS.

## Proposed text changes

#### 7.3.2.3	Sequence parameter set RBSP syntax

| seq_parameter_set_rbsp( ) { | Descriptor |  |
| --- | --- | --- |
| ... |  |  |
| sps_lfnst_enabled_flag | u(1) |  |
| ... |  |  |
| sps_explicit_scaling_list_enabled_flag | u(1) |  |
| if( sps_lfnst_enabled_flag && sps_explicit_scaling_list_enabled_flag ) |  |  |
| scaling_matrix_for_lfnst_disabled_flag | u(1) |
| ... |  |  |
| } |  |  |

#### 7.4.3.3	Sequence parameter set RBSP semantics

...

sps_lfnst_enabled_flag equal to 1 specifies that lfnst_idx may be present in intra coding unit syntax. sps_lfnst_enabled_flag equal to 0 specifies that lfnst_idx is not present in intra coding unit syntax.

...

sps_explicit_scaling_list_enabled_flag equal to 1 specifies that the use of an explicit scaling list, which is signalled in a scaling list APS, in the scaling process for transform coefficients when decoding a slice is enabled for the CLVS. sps_explicit_scaling_list_enabled_flag equal to 0 specifies that the use of an explicit scaling list in the scaling process for transform coefficients when decoding a slice is disabled for the CLVS.

scaling_matrix_for_lfnst_disabled_flag equal to 1 specifies that scaling matrices are not applied to blocks coded with LFNST. scaling_matrix_for_lfnst_disabled_flag equal to 0 specifies that the scaling matrices may be applied to the blocks coded with LFNST. When not present, the value of scaling_matrix_for_lfnst_disabled_flag is inferred to be equal to 1.

...

#### 7.3.2.7	Picture header structure syntax

| picture_header_structure( ) { | Descriptor |
| --- | --- |
| ... |  |
| if( sps_explicit_scaling_list_enabled_flag ) { |  |
| ph_explicit_scaling_list_enabled_flag | u(1) |
| if( ph_explicit_scaling_list_enabled_flag ) |  |
| ph_explicit_scaling_list_mode_idc | u(2) |
| if( ph_explicit_scaling_list_mode_idc > 0 ) |  |
| ph_scaling_list_aps_id | u(3) |
| } |  |
| ... |  |
| } |  |

#### 7.4.3.7	Picture header structure semantics

...

ph_explicit_scaling_list_enabled_flag equal to 1 specifies that the use of the explicit scaling list, which is signalled in the referenced scaling list APS (i.e., the APS with aps_params_type equal to SCALING_APS and adaptation_parameter_set_id equal to ph_explicit_scaling_list_aps_id), in the scaling process for transform coefficients when decoding a slice is enabled for the picture. ph_explicit_scaling_list_enabled_flag equal to 0 specifies that the use of an explicit scaling list in the scaling process for transform coefficients when decoding a slice is disabled for the picture. When not present, the value of ph_explicit_scaling_list_enabled_flag is inferred to be equal to 0.

ph_explicit_scaling_list_mode_idc equal to 0 specifies that use of an explicit scaling list in the scaling process for transform coefficients when decoding a slice is disabled for the picture. ph_explicit_scaling_list_mode_idc equal to 1 specifies that the explicit scaling list, which is signalled in the referenced scaling list APS (i.e., the APS with aps_params_type equal to SCALING_APS and adaptation_parameter_set_id equal to ph_explicit_scaling_list_aps_id), is used in the scaling process for transform coefficients when decoding all slices in the picture. ph_explicit_scaling_list_mode_idc equal to 2 specifies that use of the explicit scaling list in the scaling process for transform coefficients when decoding a slice is enabled for the picture. The value of ph_explicit_scaling_list_mode_idc shall be less than 3. When not present, the value of ph_explicit_scaling_mode_idc is inferred to be equal to 0.

ph_scaling_list_aps_id specifies the adaptation_parameter_set_id of the scaling list APS. The TemporalId of the APS NAL unit having aps_params_type equal to SCALING_APS and adaptation_parameter_set_id equal to ph_scaling_list_aps_id shall be less than or equal to the TemporalId of the picture associated with PH.

...

#### 7.3.2.21	Scaling list data syntax

| scaling_list_data( ) { | Descriptor |
| --- | --- |
| scaling_matrix_for_lfnst_disabled_flag | u(1) |
| scaling_list_chroma_present_flag | u(1) |
| ... |  |
| } |  |  |

#### 7.4.3.21	Scaling list data semantics

scaling_matrix_for_lfnst_disabled_flag equal to 1 specifies that scaling matrices are not applied to blocks coded with LFNST. scaling_matrix_for_lfnst_disabled_flag equal to 0 specifies that the scaling matrices may apply to the blocks coded with LFNST.

scaling_list_chroma_present_flag equal to 1 specifies that chroma scaling lists are present in scaling_list_data( ). scaling_list_chroma_present_flag equal to 0 specifies that chroma scaling lists are not present in scaling_list_data( ). It is a requirement of bitstream conformance that scaling_list_chroma_present_flag shall be equal to 0 when ChromaArrayType is equal to 0, and shall be equal to 1 when ChromaArrayType is not equal to 0.

...

#### 7.3.7.1	General slice header syntax

| slice_header( ) { | Descriptor |
| --- | --- |
| ... |  |
| if( ph_explicit_scaling_list_enabled_flag ) |  |
| if( ph_explicit_scaling_list_mode_idc = = 2 ) |  |
| slice_explicit_scaling_list_used_flag | u(1) |
| ... |  |
| } |  |

#### 7.4.8.1 General slice header semantics

...

slice_explicit_scaling_list_used_flag equal to 1 specifies that the explicit scaling list, which is signalled in the referenced scaling list APS with aps_params_type equal to SCALING_APS and adaptation_parameter_set_id equal to ph_scaling_list_aps_id, is used in the scaling process for transform coefficients when decoding of the current slice. slice_explicit_scaling_list_used_flag equal to 0 specifies that no explicit scaling list is not used in the scaling process for transform coefficients when decoding of the current slice. When not present, the value of slice_explicit_scaling_list_used_flag is inferred to be equal to ( ph_explicit_scaling_list_mode_idc  = =  1 ) ? 1: 0.

...

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).