| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0067-v1 |
| --- | --- |

| Title: | AHG8/AHG9: On the derivation of PictureOutputFlag |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ye-Kui Wang (Bytedance) 8910 University Center Lane San Diego, CA 92122, USA Miska M. Hannuksela (Nokia) Nokia Technologies, Tampere, Finland | Tel: Email: | yekui.wang@bytedance.com miska.hannuksela@nokia.com |
| Source: | Bytedance Inc., Nokia |  |  |

_____________________________

# Abstract

The current text for the derivation of the variable PictureOutputFlag normatively specifies a specific picture output behavior for an AU when the picture of the only output layer is not present (due to e.g. loss or layer down-switching). However, that piece of specification assertedly has multiple issues.

This contribution proposes some changes to address the asserted issues, by either keeping the picture output behavior normatively specified but changed with the asserted issues fixed, or only describing it in a NOTE while in the specified normative picture output behavior the value of PictureOutputFlag for a current picture is set equal to 0 whenever the current picture does not belong to an output layer.

# Introduction

The current text for the derivation of the variable PictureOutputFlag normatively specifies a specific picture output behavior for an AU when the picture of the only output layer is not present (due to e.g. loss or layer down-switching). However, that piece of specification has the following issues:

- The derivation involves the use of the PictureOutputFlag of a picure in the same AU as the current picture and in a higher layer than the current picture. However, for a picture picA that has nuh_layer_id greater than that of the current picture, when PictureOutputFlag of the current picture is being derived, PictureOutputFlag of picA has not yet been derived.

- For example, assuming there are two layers in the bitstream of the OLS and only the higher layer is an output layer, and in a particular AU auA the picture of the higher layer has pic_output_flag equal to 0. At the decoder side the higher-layer picture of auA is not present, while the lower-layer picture of auA is present and has pic_output_flag equal to 1. Then the value of PictureOutputFlag of the lower-layer picture of auA would be set equal to 1. However, when an OLS has only one output layer and a picture of the output layer has pic_output_flag equal to 0, it should be interpreted that the encoder (and the content provider) did not want a picture to be output for the AU containing that picture.

- For another example, assuming there are three or more layers in the bitstream of the OLS and only the top layer is an output layer. At the decoder side the top layer picture of the current AU is not present, while two or more pictures of the lower layers for the current AU are present and these pictures have pic_output_flag equal to 1, then for this AU more than one picture would be output. However, this is problematic as there is only one output layer for the OLS, thus outputting of only one picture was expected by the encoder (and the content provider).

- Same as the OLS mode 0 (when ols_mode_idc is equal to 0), the OLS mode 2 (when ols_mode_idc is equal to 2) can also specify only one output layer. However, the behavior of outputting a lower layer picture for an AU when the output layer picture (which is also the highest layer picture) is not present is currently only specified for mode 0 but not for mode 2. This is not a choice of a consistent design.

This contribution tries to address the above issues, with two options provided.

# Proposal

## Option 1: Leave the specific picture output behavior to implementations

In this option, to address the above issues, it is proposed that, instead of normatively specifying the specific picture output behavior for an AU when the picture of the only output layer is not present, that behavior is only described in a NOTE, and in the specified normative picture output behavior, the value of PictureOutputFlag for a current picture is set equal to 0 whenever the current picture does not belong to an output layer.

In the proposed text changes, the PictureOutputFlag deriation for RASL pictures, GDR pictures and pictures associated with GDR pictures in an output layer remains unchanged technically.

The changed texts for this option are based on the latest VVC text in JVET-Q2001-vE/v15, with added parts highlighted in turquoise and deleted parts highlighted in yellow strikethrough in red fonts.

#### 7.4.3.2	Video parameter set RBSP semantics

...

ols_mode_idc equal to 0 specifies that the total number of OLSs specified by the VPS is equal to vps_max_layers_minus1 + 1, the i-th OLS includes the layers with layer indices from 0 to i, inclusive, and for each OLS only the highest layer in the OLS is an output layer.

ols_mode_idc equal to 1 specifies that the total number of OLSs specified by the VPS is equal to vps_max_layers_minus1 + 1, the i-th OLS includes the layers with layer indices from 0 to i, inclusive, and for each OLS all layers in the OLS are output layers.

ols_mode_idc equal to 2 specifies that the total number of OLSs specified by the VPS is explicitly signalled and for each OLS the output layers are explicitly signalled and other layers are the layers that are direct or indirect reference layers of the output layers of the OLS.

...

#### 7.4.3.7 Picture header structure semantics

[Note: The changes here are just for defining the term recovering picture to be coneniently used in other places, the use of which is simpler editorially than using the variable derived here, thus the variable is made to be a local variable instead.]

...

recovery_poc_cnt specifies the recovery point of decoded pictures in output order.

When the current picture is a GDR picture, the variable recoveryPointPocVal is derived as follows:

recoveryPointPocVal = PicOrderCntVal + recovery_poc_cnt	(81)

If the current picture is a GDR picture that is associated with the PH, and there is a picture picA that follows the current GDR picture in decoding order in the CLVS that has PicOrderCntVal equal to recoveryPointPocVal the PicOrderCntVal of the current GDR picture plus the value of recovery_poc_cnt, the picture picA is referred to as the recovery point picture. Otherwise, the first picture in output order that has PicOrderCntVal greater than recoveryPointPocVal the PicOrderCntVal of the current picture plus the value of recovery_poc_cnt in the CLVS is referred to as the recovery point picture. The recovery point picture shall not precede the current GDR picture in decoding order. The pictures that are associated with the current GDR picture and have PicOrderCntVal less than recoveryPointPocVal are referred to as the recovering pictures of the GDR picture. The value of recovery_poc_cnt shall be in the range of 0 to MaxPicOrderCntLsb − 1, inclusive.

When the current picture is a GDR picture, the variable RpPicOrderCntVal is derived as follows:

RpPicOrderCntVal = PicOrderCntVal + recovery_poc_cnt	(81)

NOTE 2 – When gdr_enabled_flag is equal to 1 and PicOrderCntVal of the current picture is greater than or equal to recoveryPointPocVal RpPicOrderCntVal of the associated GDR picture, the current and subsequent decoded pictures in output order are exact match to the corresponding pictures produced by starting the decoding process from the previous IRAP picture, when present, preceding the associated GDR picture in decoding order.

...

#### 8.1.2 Decoding process for a coded picture

...

–	The variable PictureOutputFlag of the current picture is set as derived as follows:

–	If sps_video_parameter_set_id is greater than 0 and the current layer is not an output layer (i.e., nuh_layer_id is not equal to OutputLayerIdInOls[ TargetOlsIdx ][ i ] for any value of i in the range of 0 to NumOutputLayersInOls[ TargetOlsIdx ] − 1, inclusive), or one of the following conditions is true, PictureOutputFlag is set equal to 0:

–	tThe current picture is a RASL picture and NoOutputBeforeRecoveryFlag of the associated IRAP picture is equal to 1.

–	gdr_enabled_flag is equal to 1 and tThe current picture is a GDR picture with NoOutputBeforeRecoveryFlag equal to 1 or a recovering picture of a GDR picture with NoOutputBeforeRecoveryFlag equal to 1. [Note: Removed of "gdr_enabled_flag is equal to 1" is just an editorial simplifiaiton, as when gdr_enabled_flag is equal to 0 there is no GDR picture.]

–	gdr_enabled_flag is equal to 1, the current picture is associated with a GDR picture with NoOutputBeforeRecoveryFlag equal to 1, and PicOrderCntVal of the current picture is less than RpPicOrderCntVal of the associated GDR picture. [Note: This removed item is covered, without technical change, by the added part in the immediately above item, in a simpler way.]

–	sps_video_parameter_set_id is greater than 0, ols_mode_idc is equal to 0 and the current AU contains a picture picA that satisfies all of the following conditions:

–	PicA has PictureOutputFlag equal to 1.

–	PicA has nuh_layer_id nuhLid greater than that of the current picture.

–	PicA belongs to the output layer of the OLS (i.e., OutputLayerIdInOls[ TargetOlsIdx ][ 0 ] is equal to nuhLid).

–	sps_video_parameter_set_id is greater than 0, ols_mode_idc is equal to 2, and ols_output_layer_flag[ TargetOlsIdx ][ GeneralLayerIdx[ nuh_layer_id ] ] is equal to 0. [Note: This removed item is covered, without technical change, by the added part after "If".]

–	Otherwise, PictureOutputFlag is set equal to pic_output_flag.

NOTE – In an implementation, the decoder may output a picture not belong to an output layer. For example, when there is only one output layer while in an AU the picture of the output layer is not available, e.g., due to a loss or layer down-switching, the decoder may set PictureOutputFlag set equal to 1 for the picture that has the highest value of nuh_layer_id among all pictures of the AU available to the decoder and having pic_output_flag equal to 1, and set PictureOutputFlag equal to 0 for all other pictures of the AU available to the decoder.

...

## Option 2: Still normatively specify the specific picture output behavior with bug fixes

In this option, the specific picture output behavior for an AU when the picture of the only output layer is not present is still normatively specified, but with the issues discussed above fixed.

The changed texts for this option are relative to text in option 1, with added parts highlighted in turquoise, and deleted parts highlighted in yellow strikethrough in red fonts.

#### 7.4.3.2	Video parameter set RBSP semantics

...

ols_mode_idc equal to 0 specifies that the total number of OLSs specified by the VPS is equal to vps_max_layers_minus1 + 1, the i-th OLS includes the layers with layer indices from 0 to i, inclusive, and for each OLS only the highest layer in the OLS is an output layer.

ols_mode_idc equal to 1 specifies that the total number of OLSs specified by the VPS is equal to vps_max_layers_minus1 + 1, the i-th OLS includes the layers with layer indices from 0 to i, inclusive, and for each OLS all layers in the OLS are output layers.

ols_mode_idc equal to 2 specifies that the total number of OLSs specified by the VPS is explicitly signalled and for each OLS the output layers are explicitly signalled and other layers are the layers that are direct or indirect reference layers of the output layers of the OLS.

NOTE – When an OLS contains only one output layer and no picture of the output layer is present in an AU, a picture with the highest value of nuh_layer_id among the pictures with pic_output_flag equal to 1 in the AU is output.

...

#### 8.1.2 Decoding process for a coded picture

...

–	The variable PictureOutputFlag of the current picture is derived as follows:

–	If sps_video_parameter_set_id is greater than 0 and the current layer is not an output layer (i.e., nuh_layer_id is not equal to OutputLayerIdInOls[ TargetOlsIdx ][ i ] for any value of i in the range of 0 to NumOutputLayersInOls[ TargetOlsIdx ] − 1, inclusive), or one of the following conditions is true, PictureOutputFlag is set equal to 0:

–	The current picture is a RASL picture and NoOutputBeforeRecoveryFlag of the associated IRAP picture is equal to 1.

–	The current picture is a GDR picture with NoOutputBeforeRecoveryFlag equal to 1 or a recovering picture of a GDR picture with NoOutputBeforeRecoveryFlag equal to 1.

–	Otherwise, PictureOutputFlag is set equal to pic_output_flag.

NOTE – In an implementation, the decoder may output a picture not belong to an output layer. For example, when there is only one output layer while in an AU the picture of the output layer is not available, e.g., due to a loss or layer down-switching, the decoder may set PictureOutputFlag set equal to 1 for the picture that has the highest value of nuh_layer_id among all pictures of the AU available to the decoder and having pic_output_flag equal to 1, and set PictureOutputFlag equal to 0 for all other pictures of the AU available to the decoder.

–	When sps_video_parameter_set_id is greater than 0, there is only one output layer, the picture of the output layer (which has to be the top layer when there is only one output layer) is not present in the current AU, and the current picture is the last picture present in the current AU, the values of PictureOutputFlag of all pictures present in the current AU are updated as follows: PictureOutputFlag is set equal to 1 for the picture that has the highest value of nuh_layer_id among all pictures of the AU available to the decoder and having pic_output_flag equal to 1, and set equal to 0 for all other pictures of the current AU available to the decoder.

...

# Patent rights declaration(s)

Bytedance Inc. and Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).