| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0071-v1 |
| --- | --- |

| Title: | AHG12: Some cleanups on subpicture signalling |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Zhipin Deng1, Ye-Kui Wang2, Li Zhang2 Kai Zhang 1Zijinshumayuan 4th building F9 18 Zhongguancun Nansijie Haidian District, 100190, Beijing, China 28910 University Center Lane San Diego, CA 92122, USA | Tel: Email: | zhipin.deng@bytedance.com yekui.wang@bytedance.com lizhang.idm@bytedance.com zhangkai.video@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

This contribution proposes the following changes on subpicture signalling:

- Condition sps_independent_subpics_flag on "sps_num_subpics_minus1 > 0". Accordingly, when sps_independent_subpics_flag is not present, it is inferred to be equal to 1.

- Infer subpic_treated_as_pic_flag[ i ] to be equal to 1 when not present.

- Infer loop_filter_across_subpic_enabled_pic_flag[ i ] to be equal to 0 when not present.

- Infer single_slice_per_subpic_flag to be equal to 1 when no_pic_partition_flag is equal to 1.

# Introduction

In the current draft text, the following issues on subpicture signalling were observed:

- The syntax element sps_independent_subpics_flag is signalled even when there is only one subpicture. For a clean syntax design, we believe there is no need to signal sps_independent_subpics_flag when sps_num_subpics_minus1 is equal to 0, as when there is only one subpicture in each picture of the CLVS, the subpicture is by definition a fully independent subpicture.

Moreover, when sps_independent_subpics_flag is not present (i.e., when subpic_info_present_flag equal to 0), it is inferred to be equal to 0. However, when subpic_info_present_flag equal to 0, subpicture information is not present and there is only one subpicture, the value of sps_independent_subpics_flag should be inferred to be equal to 1 instead, as again the subpicture in this case is by definition a fully independent subpicture.

- When the SPS syntax element subpic_treated_as_pic_flag[ i ] is not present, it is inferred to be equal to sps_independent_subpics_flag. subpic_treated_as_pic_flag[ i ] is not present in the following three cases: i) subpic_info_present_flag is equal to 0; ii) subpic_info_present_flag is equal to 1 and sps_num_subpics_minus1 is equal to 0; and iii) subpic_info_present_flag is equal to 1, sps_num_subpics_minus1 is greater than 0, and sps_independent_subpics_flag is equal to 1.

For case i), subpic_info_present_flag equal to 0 specifies that subpicture information is not present and there is only one subpicture in each picture of the CLVS. Furthermore, according to the current text, when subpic_info_present_flag is equal to 0, sps_independent_subpics_flag would not be signalled and it is inferred to be equal to 0. Consequently, subpic_treated_as_pic_flag[ i ] is inferred to be equal to sps_independent_subpics_flag, which is 0. However, subpic_treated_as_pic_flag[ i ] should be inferred to be equal to 1 when there is only one subpicture in each picture, as in this case boundaries of the subpicture are not only treated as picture boundaries, they are picture boundaries.

For case ii), subpic_info_present_flag equal to 1 and sps_num_subpics_minus1 equal to 0 specify that, the subpicture information is present and there is only one subpicture in each picture of the CLVS. In this case, according to the current text, sps_independent_subpics_flag would still be signalled. And consequently, subpic_treated_as_pic_flag[ i ] is inferred to be equal to sps_independent_subpics_flag, which can be 0. Again, in this case, subpic_treated_as_pic_flag[ i ] should be inferred to be equal to 1.

For case iii), the current text is correct. In this case, there is more than one subpicture and sps_independent_subpics_flag is signalled to be equal to 1, subpic_treated_as_pic_flag[ i ] is inferred to be equal to sps_independent_subpics_flag which is signalled as 1.

- When the SPS syntax element loop_filter_across_subpic_enabled_flag[ i ] is not present, it is inferred to be equal to 1 − sps_independent_subpics_flag. However, similar to item 2 above, loop_filter_across_subpic_enabled_flag[ i ] is not present in three cases: i) subpic_info_present_flag is equal to 0; ii) subpic_info_present_flag is equal to 1 and sps_num_subpics_minus1 is equal to 0; and iii) subpic_info_present_flag is equal to 1, sps_num_subpics_minus1 is greater than 0, and sps_independent_subpics_flag is equal to 1.

For case i), sps_independent_subpics_flag is inferred to be equal to 0, and loop_filter_across_subpic_enabled_flag[ i ] is inferred to be equal to 1 − sps_independent_subpics_flag, which is 1. However, loop_filter_across_subpic_enabled_flag[ i ] should be inferred to be equal to 0 when there is only one subpicture in each picture, as the boundaries of the subpicture are picture boundaries and while there is anyway no in-loop filtering applied across picture boundaries.

Case ii) and iii) are similar as in item 2 with similar conclusions.

- When the PPS syntax element single_slice_per_subpic_flag is not present, it is inferred to be equal to 0. single_slice_per_subpic_flag is not present in two cases: i) no_pic_partition_flag is equal to 1, and ii) no_pic_partition_flag is equal to 0 and rect_slice_flag is equal to 0.

For case i), no_pic_partition_flag equal to 1 specifies that no picture partitioning is applied to each picture referring to the PPS, therefore, there is only one slice in each picture, and consequently, there is only one subpicture in each picture and there is only one slice in each subpicture. Therefore, in this case single_slice_per_subpic_flag should be inferred to be equal 1.

For case ii), since rect_slice_flag is equal to 0, an inferred value of single_slice_per_subpic_flag is not needed.

# Proposal

The following changes are proposed to address the above issues:

- To address the 1st issue, it is proposed to condition the signalling of sps_independent_subpics_flag on "sps_num_subpics_minus1 > 0".

Accordingly, with this modification, sps_independent_subpics_flag is not present in two cases: i) subpic_info_present_flag is equal to 0, and ii) subpic_info_present_flag is equal to 1 and sps_num_subpics_minus1 is equal to 0. In either case, there is only one subpicture in each picture of the CLVS.

Therefore, it is proposed to infer the value of sps_independent_subpics_flag to be equal to 1 when not present.

- To address the 2nd issue, it is proposed to infer subpic_treated_as_pic_flag[ i ] to be equal to 1 when not present.

- To address the 3rd issue, it is proposed to infer loop_filter_across_subpic_enabled_flag[ i ] to be equal to 0 when not present.

- To address the 4th issue, it is proposed to infer single_slice_per_subpic_flag to be equal to 1 when no_pic_partition_flag is equal to 1.

# Proposed spec text changes

Spec text changes for all the above proposals are provided below.

The proposed text changes are relative to the latest VVC draft text. The most relevant parts that have been added or modified are highlighted in turquoise, and the most relevant removed parts are highlighted in yellow strikethrough in red fonts.

The syntax structure seq_parameter_set_rbsp( ) is changed as follows:

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| sps_num_subpics_minus1 | ue(v) |
| if( sps_num_subpics_minus1 ) |  |
| sps_independent_subpics_flag | u(1) |
| … |  |

…

sps_independent_subpics_flag equal to 1 specifies that all subpicture boundaries in the CLVS are treated as picture boundaries and there is no loop filtering across the subpicture boundaries. sps_independent_subpics_flag equal to 0 does not impose such a constraint. When not present, the value of sps_independent_subpics_flag is inferred to be equal to 0 1.

…

subpic_treated_as_pic_flag[ i ] equal to 1 specifies that the i-th subpicture of each coded picture in the CLVS is treated as a picture in the decoding process excluding in-loop filtering operations. subpic_treated_as_pic_flag[ i ] equal to 0 specifies that the i-th subpicture of each coded picture in the CLVS is not treated as a picture in the decoding process excluding in-loop filtering operations. When not present, the value of subpic_treated_as_pic_flag[ i ] is inferred to be equal to sps_independent_subpics_flag 1.

…

loop_filter_across_subpic_enabled_flag[ i ] equal to 1 specifies that in-loop filtering operations may be performed across the boundaries of the i-th subpicture in each coded picture in the CLVS. loop_filter_across_subpic_enabled_flag[ i ] equal to 0 specifies that in-loop filtering operations are not performed across the boundaries of the i-th subpicture in each coded picture in the CLVS. When not present, the value of loop_filter_across_subpic_enabled_pic_flag[ i ] is inferred to be equal to 1 - sps_independent_subpics_flag 0.

…

And the semantics of single_slice_per_subpic_flag is changed as follows:

single_slice_per_subpic_flag equal to 1 specifies that each subpicture consists of one and only one rectangular slice. single_slice_per_subpic_flag equal to 0 specifies that each subpicture may consist of one or more rectangular slices. When no_pic_partition_flag is equal to 1 not present, the value of single_slice_per_subpic_flag is inferred to be equal to 01.

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).