| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0072-v2 |
| --- | --- |

| Title: | AHG9: On signalling of deblocking control |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Zhipin Deng1, Ye-Kui Wang2, Li Zhang2 Kai Zhang2, Jizheng Xu2 1Zijinshumayuan 4th building F9 18 Zhongguancun Nansijie Haidian District, 100190, Beijing, China 28910 University Center Lane San Diego, CA 92122, USA | Tel: Email: | zhipin.deng@bytedance.com yekui.wang@bytedance.com lizhang.idm@bytedance.com zhangkai.video@bytedance.com xujizheng@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

It is asserted that the deblocking control mechanism in the latest VVC text is pretty complicated, not straightforward, not easy to understand, and consequently prone to errors. This contribution proposes to change the deblocking signalling based on a 2-bit deblocking mode indicator in the PPS, summarized as follows:

- Replace the three PPS flags for deblocking signalling with a 2-bit deblocking mode indicator that specifies the following four modes: a) deblocking fully disabled and not used for all slices; b) deblocking used for all slices using 0-valued β and tC offsets; c) deblocking used for all slices using β and tC offsets explicitly signalled in the PPS; and d) deblocking further controlled at either picture or slice level.

- The two flags in PH/SH are renamed to be ph/slice_deblocking_filter_used_flag and ph/slice_deblocking_parameters_override_flag, with the use flag specifying whether deblocking is used for the current picture/slice, and the override flag specifying whether the β and tC offsets are overridden by the values signalled in the PH/SH.

# Introduction

It is asserted that the deblocking control mechanism in the latest VVC text is pretty complicated, not straightforward, and not easy to understand, and consequently prone to errors. We have spent quite some effort to figure out the following summary of the current deblocking control mechanism, as shown in Table 1, up to the PH level signalling (without adding the SH level signalling herein should be good enough for the summary of the different modes and for the discussion below):

Table 1. Summary of the current deblocking control mechanism

| # possible case |  | #1 | #2 | #3 | #4 | #5 | #6 | #7 | #8 | #9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PPS | deblocking_filter_control_present_flag | 1 | 0 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
|  | deblocking_filter_override_enabled_flag | 0 | - | 0 | 1 | 1 | 1 | 1 | 1 | 1 |
|  | pps_deblocking_filter_disabled_flag | 1 | - | 0 | 0 | 0 | 0 | 1 | 1 | 1 |
|  | pps β/tc | - | - | pps β/tc | pps β/tc | pps β/tc | pps β/tc | - | - | - |
|  | dbf_info_in_ph_flag | - | - | - | 0/1 | 0/1 | 0/1 | 0/1 | 0/1 | 0/1 |
| PH | ph_deblocking_filter_override_flag | - | - | - | 0 | 1 | 1 | 0 | 1 | 1 |
|  | ph_deblocking_filter_disabled_flag | - | - | - | - | 0 | 1 | - | 1 | 0 |
|  | ph β/tc | - | - | - | - | ph β/tc | - | - | - | ph β/tc |

* case #1: deblocking fully disabled and not used for all slices

case #2: deblocking used for all slices using 0-valued β and tC offsets

case #3: deblocking used for all slices using β and tC offsets explicitly signalled in the PPS

case #4: deblocking further controlled at picture level, and used for the current picture using offsets from PPS

case #5: deblocking further controlled at picture level, and used for the current picture using offsets signalled in PH

case #6: deblocking further controlled at picture level, and disabled for the current picture

case #7: deblocking firstly disabled at PPS and then disabled (again!) at picture level

case #8: deblocking firstly disabled at PPS and then disabled (again!) at picture level

case #9: deblocking firstly disabled at PPS and then enabled at picture level

Our observation is that case #1 to #6 summarized above are the main functionalities to be achieved (in cases #4 to #6 the deblocking is further controlled at either picture or slice level). Cases #7 to #9 are all weird: disabled and disabled again or disabled and then enabled, while cases #7 and #8 do the same weird thing twice. We think that these weird cases are not necessary and probably was not intended to be there, there were likely just some bad side-products when juggling around all the flags for the design.

Speaking of the current design being difficult to understand and prone to errors, here are some example issues we observed:

- According to the current text, even if the deblocking filter is disabled in the PPS, it can be enabled in the PH or SH. For example, as shown in Table 1 case #9, if pps_deblocking_filter_disabled_flag is firstly signalled to be equal to 1, and deblocking_filter_override_enabled_flag is also signalled to be equal to 1, it indicates that the deblocking filter is disabled at PPS and it also allows the deblocking filter enable/disable control being overridden in the PH or SH. Then dbf_info_in_ph_flag is signalled subsequently and the PH syntax element ph_deblocking_filter_disabled_flag might be signalled to be equal to 0 which ultimately enables the deblocking filter for slices associated with the PH. In such case, the deblocking is eventually enabled at PH regardless that it has been disabled at the higher level (e.g., PPS). Such design logic is unique in VVC text, and it is quite different from the design logic of other coding tools (e.g., ALF, SAO, LMCS, TMVP, WP, and etc.) as normally when a coding tool is disabled at a higher layer (e.g., SPS, PPS), then it is disabled completely at lower layers (e.g., PH, SH).

Furthermore, the current definition of pps_deblocking_filter_disabled_flag is like “pps_deblocking_filter_disabled_flag equal to 1 specifies that the operation of deblocking filter is not applied for slices referring to the PPS in which slice_deblocking_filter_disabled_flag is not present...”. However, according to the current syntax table, even though pps_deblocking_filter_disabled_flag is equal to 1 and slice_deblocking_filter_disabled_flag is not present, the operation of deblocking filter would still be applied in case of ph_deblocking_filter_disabled_flag is present and signalled to be equal to 0. Therefore, the current definition of pps_deblocking_filter_disabled_flag is not correct.

- Moreover, according to the current text, if both the PPS syntax elements deblocking_filter_override_enabled_flag and pps_deblocking_filter_disabled_flag are equal to 1, it specifies that deblocking is disabled in PPS and the control of deblocking filter is intended to be overridden in the PH or SH. However, the subsequent PH syntax elements ph_deblocking_filter_override_flag and ph_deblocking_filter_disabled_flag might be still signalled to be equal to 1, as in Table 1 case #8, which turns out that the resulted overriding process doesn’t change anything (e.g., deblocking remains disabled in the PH/SH) but just use unnecessary bits for meaningless signalling.

- In addition, according to the current text, when the SH syntax element slice_deblocking_filter_override_flag is not present, it is inferred to be equal to ph_deblocking_filter_override_flag. However, besides implicit or explicit signalling in the PPS, the deblocking parameters can only be signalled either in PH or SH according to dbf_info_in_ph_flag, but never both. Therefore, when dbf_info_in_ph_flag is true, the intention is to allow to signal the overriding deblocking filter parameters in the PH. In this case, if the PH override flag is true and the SH override flag is not signalled but inferred to be equal to the PH override flag, additional deblocking filter parameters will still be signalled in the SH which is conflicting with the intention.

# Proposal

To make the deblocking control signalling simple and straightforward to understand while keeping all the main functionalities, and at the same time resolve the errors in the semantics etc., we propose to use a 2-bit mode indicator in the PPS to signal the deblocking filter mode instead of using three PPS flags. The proposed deblocking control signalling is summarized as follows:

- Replace the three PPS flags for deblocking signalling with a 2-bit deblocking mode indicator that specifies the following four modes: a) deblocking fully disabled and not used for all slices; b) deblocking used for all slices using 0-valued β and tC offsets; c) deblocking used for all slices using β and tC offsets explicitly signalled in the PPS; and d) deblocking further controlled at either picture or slice level.

- The two flags in PH/SH are renamed to be ph/slice_deblocking_filter_used_flag and ph/slice_deblocking_parameters_override_flag, with the use flag specifying whether deblocking is used for the current picture/slice, and the override flag specifying whether the β and tC offsets are overridden by the values signalled in the PH/SH.

With the proposal, the main functionalities through the 6 cases are achieved as shown in Table 2.

Table 2. Summary of the proposed deblocking control mechanism (up to the PH level signalling)

| # possible case |  | #1 | #2 | #3 | #4 | #5 | #6 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| PPS | deblocking_filter_mode_idc | 0 | 1 | 2 | 3 | 3 | 3 |
|  | pps beta/tc | - | - | pps β/tc | pps β/tc | pps β/tc | pps β/tc |
|  | dbf_info_in_ph_flag | - | - | - | 0/1 | 0/1 | 0/1 |
| PH | ph_deblocking_filter_used_flag | - | - | - | 1 | 1 | 0 |
|  | ph_deblocking_parameters_override_flag | - | - | - | 0 | 1 | - |
|  | ph beta/tc | - | - | - | - | ph β/tc | - |

* case #1: deblocking fully disabled and not used for all slices

case #2: deblocking used for all slices using 0-valued β and tC offsets

case #3: deblocking used for all slices using β and tC offsets explicitly signalled in the PPS

case #4: deblocking further controlled at picture level, and used for the current picture using offsets from PPS

case #5: deblocking further controlled at picture level, and used for the current picture using offsets signalled in PH

case #6: deblocking further controlled at picture level, and disabled for the current picture

# Proposed spec text changes

The proposed text changes are relative to the latest VVC draft text. The most relevant parts that have been added or modified are highlighted in turquoise, and the most relevant removed parts are highlighted in yellow strikethrough in red fonts.

The syntax structure pic_parameter_set_rbsp( ) is changed as follows:

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| pps_pic_parameter_set_id | ue(v) |
| … |  |
| deblocking_filter_control_present_flagmode_idc | u(12) |
| if(deblocking_filter_control_present_flagmode_idc > 1 ) { |  |
| deblocking_filter_override_enabled_flag | u(1) |
| pps_deblocking_filter_disabled_flag | u(1) |
| if( !pps_deblocking_filter_disabled_flag ) { |  |
| pps_beta_offset_div2 | se(v) |
| pps_tc_offset_div2 | se(v) |
| pps_cb_beta_offset_div2 | se(v) |
| pps_cb_tc_offset_div2 | se(v) |
| pps_cr_beta_offset_div2 | se(v) |
| pps_cr_tc_offset_div2 | se(v) |
| } |  |
| } |  |
| rpl_info_in_ph_flag | u(1) |
| if( deblocking_filter_override_enabled_ flagmode_idc = = 3 ) |  |
| dbf_info_in_ph_flag | u(1) |
| rpl_info_in_ph_flag |  |
| … |  |

...

deblocking_filter_mode_idc equal to 0 specifies that the deblocking filter is not applied for any slice referring to the PPS. deblocking_filter_mode_idc equal to 1 specifies that the deblocking filter is applied for all slices referring to the PPS, using 0-valued deblocking parameter offsets for β and tC. deblocking_filter_mode_idc equal to 2 specifies that the deblocking filter is applied for all slices referring to the PPS, using deblocking parameter offsets for β and tC explicitly signalled in the PPS. deblocking_filter_mode_idc equal to 3 specifies that whether the deblocking filter is applied for a slice referring to the PPS is controlled by parameters present either in the PH or the slice header of the slice.

deblocking_filter_control_present_flag equal to 1 specifies the presence of deblocking filter control syntax elements in the PPS. deblocking_filter_control_present_flag equal to 0 specifies the absence of deblocking filter control syntax elements in the PPS.

deblocking_filter_override_enabled_flag equal to 1 specifies the presence of ph_deblocking_filter_override_flag in the PHs referring to the PPS or slice_deblocking_filter_override_flag in the slice headers referring to the PPS. deblocking_filter_override_enabled_flag equal to 0 specifies the absence of ph_deblocking_filter_override_flag in PHs referring to the PPS or slice_deblocking_filter_override_flag in slice headers referring to the PPS. When not present, the value of deblocking_filter_override_enabled_flag is inferred to be equal to 0.

pps_deblocking_filter_disabled_flag equal to 1 specifies that the operation of deblocking filter is not applied for slices referring to the PPS in which slice_deblocking_filter_disabled_flag is not present. pps_deblocking_filter_disabled_flag equal to 0 specifies that the operation of the deblocking filter is applied for slices referring to the PPS in which slice_deblocking_filter_disabled_flag is not present. When not present, the value of pps_deblocking_filter_disabled_flag is inferred to be equal to 0.

dbf_info_in_ph_flag equal to 1 specifies that deblocking filter information is present in the PH syntax structure and not present in slice headers referring to the PPS that do not contain a PH syntax structure. dbf_info_in_ph_flag equal to 0 specifies that deblocking filter information is not present in the PH syntax structure and may be present in slice headers referring to the PPS that do not contain a PH syntax structure. When not present, the value of dbf_info_in_ph_flag is inferred to be equal to 0.

...

And the syntax structure picture_header_structure( ) is changed as follows:

| picture_header_structure( ) { | Descriptor |
| --- | --- |
| gdr_or_irap_pic_flag | u(1) |
| … |  |
| if( deblocking_filter_override_enabled_flagmode_idc = = 3 && dbf_info_in_ph_flag) { |  |
| ph_deblocking_filter_overrideused_flag | u(1) |
| if( ph_deblocking_filter_overrideused_flag ) { |  |
| ph_deblocking_filter_disabledparameters_override_flag | u(1) |
| if( !ph_deblocking_filter_disabledparameters_override_flag ) { |  |
| ph_beta_offset_div2 | se(v) |
| ph_tc_offset_div2 | se(v) |
| ph_cb_beta_offset_div2 | se(v) |
| ph_cb_tc_offset_div2 | se(v) |
| ph_cr_beta_offset_div2 | se(v) |
| ph_cr_tc_offset_div2 | se(v) |
| } |  |
| } |  |
| } |  |
| … |  |

...

ph_deblocking_filter_used_flag equal to 1 specifies that the deblocking filter is applied for the slices in the current picture. ph_deblocking_filter_used_flag equal to 0 specifies that the deblocking filter is not applied for the slices in the current picture. When not present, the value of ph_deblocking_filter_used_flag is inferred to be equal to ( deblocking_filter_mode_idc > 0 ).

ph_deblocking_filterparameters_override_flag equal to 1 specifies that deblocking parameters are present in the PH. ph_deblocking_filterparameters_override_flag equal to 0 specifies that deblocking parameters are not present in the PH.When not present, the value of ph_deblocking_filter_override_flag is inferred to be equal to 0.

ph_deblocking_filter_disabled_flag equal to 1 specifies that the operation of the deblocking filter is not applied for the slices associated with the PH. ph_deblocking_filter_disabled_flag equal to 0 specifies that the operation of the deblocking filter is applied for the slices associated with the PH. When ph_deblocking_filter_disabled_flag is not present, it is inferred to be equal to pps_deblocking_filter_disabled_flag.

...

And the syntax structure slice_header ( ) is changed as follows:

| slice_header( ) { | Descriptor |
| --- | --- |
| picture_header_in_slice_header_flag | u(1) |
| … |  |
| if( deblocking_filter_override_enabled_flagmode_idc = = 3 && !dbf_info_in_ph_flag ) |  |
| slice_deblocking_filter_overrideused_flag | u(1) |
| if( slice_deblocking_filter_overrideused_flag ) { |  |
| slice_deblocking_filter_disabledparameters_override_flag | u(1) |
| if( !slice_deblocking_filter_disabledparameters_override_flag ) { |  |
| slice_beta_offset_div2 | se(v) |
| slice_tc_offset_div2 | se(v) |
| slice_cb_beta_offset_div2 | se(v) |
| slice_cb_tc_offset_div2 | se(v) |
| slice_cr_beta_offset_div2 | se(v) |
| slice_cr_tc_offset_div2 | se(v) |
| } |  |
| } |  |
| … |  |

...

slice_deblocking_filter_used_flag equal to 1 specifies that the deblocking filter is applied for the current slice. slice_deblocking_filter_used_flag equal to 0 specifies that the deblocking filter is not applied for the current slice. When not present, the value of slice_deblocking_filter_used_flag is inferred to be equal to ph_deblocking_filter_used_flag.

slice_deblocking_filterparameters_override_flag equal to 1 specifies that deblocking parameters are present in the slice header. slice_deblocking_filterparameters_override_flag equal to 0 specifies that deblocking parameters are not present in the slice header.When not present, the value of slice_deblocking_filter_override_flag is inferred to be equal to ph_deblocking_filter_override_flag0.

slice_deblocking_filter_disabled_flag equal to 1 specifies that the operation of the deblocking filter is not applied for the current slice. slice_deblocking_filter_disabled_flag equal to 0 specifies that the operation of the deblocking filter is applied for the current slice. When slice_deblocking_filter_disabled_flag is not present, it is inferred to be equal to ph_deblocking_filter_disabled_flag.

...

And the decoding process of deblocking filter process is changed as follows:

8.8.3 Deblocking filter process

8.8.3.1 General

The deblocking filter process is applied to all coding subblock edges and transform block edges of a picture, except the following types of edges:

- Edges that are at the boundary of the picture,

- Edges that coincide with the boundaries of a subpicture with subpicture index subpicIdx and loop_filter_across_subpic_enabled_flag[ subpicIdx ] is equal to 0,

- Edges that coincide with the virtual boundaries of the picture when VirtualBoundariesPresentFlag is equal to 1,

- Edges that coincide with tile boundaries when loop_filter_across_tiles_enabled_flag is equal to 0,

- Edges that coincide with slice boundaries when loop_filter_across_slices_enabled_flag is equal to 0,

- Edges that coincide with upper or left boundaries of slices with slice_deblocking_filter_useddisabled_flag equal to 10,

- Edges within slices with slice_deblocking_filter_useddisabled_flag equal to 10,

- Edges that do not correspond to 4×4 sample grid boundaries of the luma component,

- Edges that do not correspond to 8×8 sample grid boundaries of the chroma component,

- Edges within the luma component for which both sides of the edge have intra_bdpcm_luma_flag equal to 1,

- Edges within the chroma components for which both sides of the edge have intra_bdpcm_chroma_flag equal to 1,

- Edges of chroma subblocks that are not edges of the associated transform unit.

The edge type, vertical or horizontal, is represented by the variable edgeType as specified in Table 42.

Table 42 – Name of association to edgeType

| edgeType | Name of edgeType |
| --- | --- |
| 0 (vertical edge) | EDGE_VER |
| 1 (horizontal edge) | EDGE_HOR |

When slice_deblocking_filter_useddisabled_flag of the current slice is equal to 01, the following applies:

- The variable treeType is set equal to DUAL_TREE_LUMA.

- The vertical edges are filtered by invoking the deblocking filter process for one direction as specified in clause 8.8.3.2 with the variable treeType, the reconstructed picture prior to deblocking, i.e., the array recPictureL and the variable edgeType set equal to EDGE_VER as inputs, and the modified reconstructed picture after deblocking, i.e., the array recPictureL as outputs.

- The horizontal edge are filtered by invoking the deblocking filter process for one direction as specified in clause 8.8.3.2 with the variable treeType, the modified reconstructed picture after deblocking, i.e., the array recPictureL and the variable edgeType set equal to EDGE_HOR as inputs, and the modified reconstructed picture after deblocking, i.e., the array recPictureL as outputs.

- When ChromaArrayType is not equal to 0, the following applies:

- The variable treeType is set equal to DUAL_TREE_CHROMA

- The vertical edges are filtered by invoking the deblocking filter process for one direction as specified in clause 8.8.3.2 with the variable treeType, the reconstructed picture prior to deblocking, i.e., the arrays recPictureCb and recPictureCr, and the variable edgeType set equal to EDGE_VER as inputs, and the modified reconstructed picture after deblocking, i.e., the arrays recPictureCb and recPictureCr as outputs.

- The horizontal edge are filtered by invoking the deblocking filter process for one direction as specified in clause 8.8.3.2 with the variable treeType, the modified reconstructed picture after deblocking, i.e., the arrays recPictureCb and recPictureCr, and the variable edgeType set equal to EDGE_HOR as inputs, and the modified reconstructed picture after deblocking, i.e., the arrays recPictureCb and recPictureCr as outputs.

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).