| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0077-v1 |
| --- | --- |

| Title: | Chroma deblocking parameters based on chroma coding tools |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jizheng Xu1, Li Zhang1, Ye-Kui Wang1 Kai Zhang1, Zhipin Deng2 18901 University Center Ln San Diego, CA 92122, USA 2Zijinshumayuan 4th building F9 18 Zhongguancun Nansijie Haidian District, 100190, Beijing, China | Email: | xujizheng@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

This contribution proposes the following changes on signalling of chroma deblocking parameters:

- Add a PPS flag pps_chroma_deblocking_params_present_flag, which is required to be equal to 0 when ChromaArrayType is equal to 0.

- Skip the signalling of the Cb/Cr deblocking tc/beta offsets in PPS/PH/SH when pps_chroma_deblocking_params_present_flag is equal to 0.

# Introduction

In the current VVC design, signalling of pps_cb_beta_offset_div2, pps_cb_tc_offset_div2, pps_cr_beta_offset_div2, pps_cr_tc_offset_div2, ph_cb_beta_offset_div2, ph_cb_tc_offset_div2, ph_cr_beta_offset_div2, ph_cr_tc_offset_div2, slice_cb_beta_offset_div2, slice_cb_tc_offset_div2, slice_cr_beta_offset_div2 and slice_cr_tc_offset_div2 does not depend on whether chroma tools are applied or not. It is proposed to signal those syntax elements only when chroma tools are applied.

# Proposed methods

We propose to add one flag, named pps_chroma_deblocking_params_present_flag in PPS and conditionally signal the chroma deblocking parameters in PPS/PH/SH based on the flag.

The proposed text changes are as following, with newly added marked in turquoise.

7.3.2.4 Picture parameter set RBSP syntax

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … | ue(v) |
| deblocking_filter_control_present_flag | u(1) |
| if( deblocking_filter_control_present_flag ) { |  |
| pps_chroma_deblocking_params_present_flag | u(1) |
| deblocking_filter_override_enabled_flag | u(1) |
| pps_deblocking_filter_disabled_flag | u(1) |
| if( !pps_deblocking_filter_disabled_flag ) { |  |
| pps_beta_offset_div2 | se(v) |
| pps_tc_offset_div2 | se(v) |
| if( pps_chroma_deblocking_params_present_flag ) { |  |
| pps_cb_beta_offset_div2 | se(v) |
| pps_cb_tc_offset_div2 | se(v) |
| pps_cr_beta_offset_div2 | se(v) |
| pps_cr_tc_offset_div2 | se(v) |
| } |  |
| } |  |
| } |  |
| … |  |

7.3.2.7 Picture header structure syntax

| picture_header_structure( ) { | Descriptor |
| --- | --- |
| …… |  |
| if( deblocking_filter_override_enabled_flag && dbf_info_in_ph_flag ) { |  |
| ph_deblocking_filter_override_flag | u(1) |
| if( ph_deblocking_filter_override_flag ) { |  |
| ph_deblocking_filter_disabled_flag | u(1) |
| if( !ph_deblocking_filter_disabled_flag ) { |  |
| ph_beta_offset_div2 | se(v) |
| ph_tc_offset_div2 | se(v) |
| if( pps_chroma_deblocking_params_present_flag ) { |  |
| ph_cb_beta_offset_div2 | se(v) |
| ph_cb_tc_offset_div2 | se(v) |
| ph_cr_beta_offset_div2 | se(v) |
| ph_cr_tc_offset_div2 | se(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| if( picture_header_extension_present_flag ) { |  |
| ph_extension_length | ue(v) |
| for( i = 0; i < ph_extension_length; i++) |  |
| ph_extension_data_byte[ i ] | u(8) |
| } |  |
| } |  |

7.3.7.1 General slice header syntax

| slice_header( ) { | Descriptor |
| --- | --- |
| …… |  |
| if( deblocking_filter_override_enabled_flag && !dbf_info_in_ph_flag ) |  |
| slice_deblocking_filter_override_flag | u(1) |
| if( slice_deblocking_filter_override_flag ) { |  |
| slice_deblocking_filter_disabled_flag | u(1) |
| if( !slice_deblocking_filter_disabled_flag ) { |  |
| slice_beta_offset_div2 | se(v) |
| slice_tc_offset_div2 | se(v) |
| if( pps_chroma_deblocking_params_present_flag ) { |  |
| slice_cb_beta_offset_div2 | se(v) |
| slice_cb_tc_offset_div2 | se(v) |
| slice_cr_beta_offset_div2 | se(v) |
| slice_cr_tc_offset_div2 | se(v) |
| } |  |
| } |  |
| } |  |
| …… |  |

7.4.3.4 Picture parameter set RBSP semantics

…

pps_chroma_deblocking_params_present_flag equal to 1 specifies that chroma deblocking related syntax elements are present in the PPS RBSP syntax structure. pps_chroma_deblocking_params_present_flag equal to 0 specifies that chroma deblocking related syntax elements are not present in the PPS RBSP syntax structure. When ChromaArrayType is equal to 0, the value of pps_chroma_deblocking_params_present_flag shall be equal to 0.

…

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).