| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, AT, 15–24 April 2020 | Document: JVET-R0080-v1 |
| --- | --- |

| Title: | AHG12: On signaling of tile and slice |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yong-Uk Yoon Dohyeon Park Jihoon Do (ETRI) Jae-Gon Kim (KAU) | Tel: Email: | +82-2-300-0414 yuyoon@kau.kr dhpark@kau.kr jhdo@etri.re.kr jgkim@kau.ac.kr |
| Source: | Korea Aerospace University |  |  |

_____________________________

- Abstract

VVC (Draft 8) [1] includes signaling for tile and slices in PPS.

This contribution proposes two modifications on signaling of tile and slice information in the PPS as follows.

- Proposal 1: It is proposed to change the condition for signaling the syntax element of tile_idx_delta_present_flag. When the value of num_slices_in_pic_minus1 is greater than 1 instead of 0, the syntax element of tile_idx_delta_present flag is signaled.

- Proposal 2: It is proposed to replace the syntax element of num_exp_tile_columns_minus1 and num_exp_tile_rows_minus1 as num_exp_tile_columns and num_exp_tile_rows, respectively. When a picture is not partitioned into multiple tiles in rows or columns, the value of num_exp_tile_columns or num_exp_tile_rows is signaled as 0. Then, the syntax element of tile_column_width_minus1 or tile_rows_height_minus1 is not signaled and inferred to be equal to PicWidthInCtbsY-1 or PicHeightInCtbsY-1.

Proposal 1

In VVC Draft 8, the syntax element of tile_idx_delta_present_flag indicates whether the syntax element of tile_idx_delta[ i ] is present or not in the PPS. When the value is equal to 0, all rectangular slices in pictures are specified in raster order and no need to signal the syntax element of tile_idx_delta[i] for slice.

In the case that the number of slices in picture is equal to 2, all rectangular slices in pictures are always in raster order so that the syntax of tile_idx_delta_present_flag doesn’t need to be signaled. So, in this contribution, the syntax elements of tile_idx_delta_present_flag is inferred as 0 when the value of num_slices_in_pic_minus1 is smaller than 2. The proposed syntax changes are shown highlighted below compared with VVC Draft 8.

- 7.3.2.4 Picture parameter set RBSP syntax

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| if( NumTilesInPic > 1 ) |  |
| rect_slice_flag | u(1) |
| if( rect_slice_flag ) |  |
| single_slice_per_subpic_flag | u(1) |
| if( rect_slice_flag && !single_slice_per_subpic_flag ) { |  |
| num_slices_in_pic_minus1 | ue(v) |
| if( num_slices_in_pic_minus1 > 0 1 ) |  |
| tile_idx_delta_present_flag | u(1) |
| for( i = 0; i < num_slices_in_pic_minus1; i++ ) { |  |
| if( NumTileColumns > 1 ) |  |
| slice_width_in_tiles_minus1[ i ] | ue(v) |
| if( NumTileRows > 1 && ( tile_idx_delta_present_flag ¦ ¦ SliceTopLeftTileIdx[ i ] % NumTileColumns = = 0 ) ) |  |
| slice_height_in_tiles_minus1[ i ] | ue(v) |
| if( slice_width_in_tiles_minus1[ i ] = = 0 && slice_height_in_tiles_minus1[ i ] = = 0 && RowHeight[ SliceTopLeftTileIdx[ i ] / NumTileColumns ] > 1 ) { |  |
| num_exp_slices_in_tile[ i ] | ue(v) |
| for( j = 0; j < num_exp_slices_in_tile[ i ]; j++ ) |  |
| exp_slice_height_in_ctus_minus1[ i ][ j ] | ue(v) |
| i += NumSlicesInTile[ i ] − 1 |  |
| } |  |

On semantics, tile_idx_delta_present_flag is not present and inferred to be equal to 0, when the num_slices_in_pic_minus1 is smaller than 2.

Proposal 2

In VVC Draft 8, no_pic_partition_flag indicates whether picture partitioning is applied or not. In other words, no_pic_partition_flag equal to 0 specifies each picture may be partitioned into more than one tile or slice. When the syntax element of no_pic_partition_flag equal to 0, the syntax elements of num_exp_tile_columns_minus1 and num_exp_tile_rows_minus1 are signaled to indicate how many tiles are partitioned in a picture, and then the syntax elements of tile_column_width_minus1 and tile_rows_height_minus1 are signaled for each tile column and row, respectively.

When a picture is partitioned into multiple tiles horizontally or vertically only, for non-partitioned side, the syntax element of num_exp_tile_(columns or rows)_minus1 is always signaled as 0 and then the syntax element of tile_(columns or rows)_(width or height)_minus1 which means the width or height of the a tile in units of CTBs is signaled. But in such case, the syntax elements for size of tile is not necessary to be signaled and can be inferred.

In this contribution, the syntax elements of num_exp_tile_columns_minus1 and num_exp_tile_rows_minus1 is replaced as num_exp_tile_columns and num_exp_tile_rows, respectively. When the num_exp_tile_columns or num_exp_tile_rows is equal to 0, tile_column_width_minus1[ i ] or tile_row_height_minus1[ i ] is inferred as PicWidthInCtbsY-1 and PicHeightInCtbsY-1. The proposed syntax changes, the associated semantics changes, and the changed scanning processes are shown highlighted below compared with VVC Draft 8.

- 7.3.2.4 Picture parameter set RBSP syntax

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| no_pic_partition_flag | u(1) |
| if( !no_pic_partition_flag ) { |  |
| pps_log2_ctu_size_minus5 | u(2) |
| num_exp_tile_columns_minus1 | ue(v) |
| num_exp_tile_rows_minus1 | ue(v) |
| num_exp_tile_columns | ue(v) |
| num_exp_tile_rows | ue(v) |
| for( i = 0; i <= num_exp_tile_columns_minus1 num_exp_tile_columns; i++ ) |  |
| tile_column_width_minus1[ i ] | ue(v) |
| for( i = 0; i <= num_exp_tile_rows_minus1 num_exp_tile_rows; i++ ) |  |
| tile_row_height_minus1[ i ] | ue(v) |
| … |  |

The proposed modifications on semantics are shown below comapred with VVC Draft 8. The syntax element of num_exp_tile_columns_minus1 and num_exp_tile_rows_minus1 are replaced as num_exp_tile_columns and num_exp_tile_rows, respectively. When the value of num_exp_tile_columns or num_exp_tile_rows is equal to 0, tile_column_width_minus1[ i ] or tile_row_height_minus1[ i ] is not signaled and inferred.

- 7.4.3.4 Picture parameter set RBSP semantics

num_exp_tile_columns_minus1 plus 1 specifies the number of explicitly provided tile column widths. The value of num_exp_tile_columns_minus1 shall be in the range of 0 to PicWidthInCtbsY − 1, inclusive. When no_pic_partition_flag is equal to 1, the value of num_exp_tile_columns_minus1 is inferred to be equal to 0.

num_exp_tile_rows_minus1 plus 1 specifies the number of explicitly provided tile row heights. The value of num_exp_tile_rows_minus1 shall be in the range of 0 to PicHeightInCtbsY − 1, inclusive. When no_pic_partition_flag is equal to 1, the value of num_tile_rows_minus1 is inferred to be equal to 0.

num_exp_tile_columns specifies the number of explicitly provided tile column widths. The value of num_exp_tile_columns shall be in the range of 0 to PicWidthInCtbsY − 1, inclusive. When num_exp_tile_columns is equal to 0, 	tile_column_width_minus1[ i ] is not signaled and inferred as PicWidthInCtbsY. When no_pic_partition_flag is equal to 1, the value of num_exp_tile_columns is inferred to be equal to 0.

num_exp_tile_rows specifies the number of explicitly provided tile row heights. The value of num_exp_tile_rows shall be in the range of 0 to PicHeightInCtbsY − 1, inclusive. When num_exp_tile_rows is equal to 0, 	tile_row_height_minus1[ i ] is not signaled and inferred as PicHeightInCtbsY. When no_pic_partition_flag is equal to 1, the value of num_exp_tile_rows is inferred to be equal to 0.

6.5.1	CTB raster scanning, tile scanning, and subpicture scanning processes

The variable NumTileColumns, specifying the number of tile columns, and the list colWidth[ i ] for i ranging from 0 to NumTileColumns − 1, inclusive, specifying the width of the i-th tile column in units of CTBs, are derived as follows:

remainingWidthInCtbsY = PicWidthInCtbsY

if( num_exp_tile_columns == 0)

{

NumTileColumns = 0

colWidth[ 0 ] = PicWidthInCtbsY

}

else

{

for( i = 0; i < num_exp_tile_columns ; i++ ) {

colWidth[ i ] = tile_column_width_minus1[ i ] + 1

remainingWidthInCtbsY  −=  colWidth[ i ]

}

uniformTileColWidth = tile_column_width_minus1[ num_exp_tile_columns ] + 1

while( remainingWidthInCtbsY  >=  uniformTileColWidth ) {

colWidth[ i++ ] = uniformTileColWidth

remainingWidthInCtbsY  −=  uniformTileColWidth

}

if( remainingWidthInCtbsY > 0 )

colWidth[ i++ ] = remainingWidthInCtbsY

NumTileColumns = i

}

The variable NumTileRows, specifying the number of tile rows, and the list RowHeight[ j ] for j ranging from 0 to NumTileRows − 1, inclusive, specifying the height of the j-th tile row in units of CTBs, are derived as follows:

remainingHeightInCtbsY = PicHeightInCtbsY

if( num_exp_tile_rows == 0)

{

NumTileRows = 0

RowHeight [ 0 ] = PicHeightInCtbsY

}

else

{

for( i = 0; i < num_exp_tile_rows; i++ ) {

RowHeight [ i ] = tile_row_height_minus1[ i ] + 1

remainingHeightInCtbsY  −=  RowHeight [ i ]

}

uniformTileRowHeight = tile_row_height_minus1[ num_exp_tile_rows ] + 1

while( remainingHeightInCtbsY  >=  uniformTileRowHeight ) {

RowHeight [ i++ ] = uniformTileRowHeight

remainingHeightInCtbsY  −=  uniformTileRowHeight

}

if( remainingHeightInCtbsY > 0 )

RowHeight[ i++ ] = remainingHeightInCtbsY

NumTileRows = i

}

References

- B. Bross, J. Chen, S. Liu and Y.-K. Wang, “Versatile Video Coding (Draft 8),” JVET-Q2001, Jan. 2020.

Patent rights declaration(s)

Korea Aerospace University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).