| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0082-v1 |
| --- | --- |

| Title: | AHG9: Byte alignment modifications |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Zhi Zhang Martin Pettersson Mitra Damghanian Jack Enhorn Jacob Ström Rickard Sjöberg | Email: | zhi.a.zhang@ericsson.com martin.m.pettersson@ericsson.com mitra.damghanian@ericsson.com jack.enhorn@ericsson.com jacob.strom@ericsson.com rickard.sjoberg@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

This contribution contains three proposals related to two identical syntax tables rbsp_trailing_bits( ) and byte_alignment( ) in the current version of VVC.

Proposal 1 is an asserted cleanup to use the syntax element alignment_bit_equal_to_one to indicate the last bit in the arithmetic decode terminate process.

Proposal 2 replaces the byte_alignment( ) last in the slice header (SH) by the contents of the byte_alignment( ) syntax structure, but without including the leading bit equal to 1. The proponents claim that this removes the only use of byte_alignment( ) in the specification and that the leading bit equal to 1 has no use.

Proposal 3 removes the syntax table byte_alignment( ) since it is claimed to never be used due to proposal 2.

# Introduction

In the current version of VVC, JVET-Q2001-vE, the two syntax tables rbsp_trailing_bits( ) and byte_alignment( ) are identical as shown in Table 1. The syntax element rbsp_stop_one_bit in rbsp_trailing_bits( ) is used to indicate the last bit of string of data bits (SODB). It is also used to indicate the last bit in the arithmetic decode terminate process. However, the syntax element alignment_bit_equal_to_one in byte_alignment( ) is defined but not used anywhere in the current version of VVC.

| RBSP trailing bits syntax | Byte alignment syntax |
| --- | --- |
| rbsp_trailing_bits( ) {; Descriptor. rbsp_stop_one_bit; f(1). while( !byte_aligned( ) ). rbsp_alignment_zero_bit; f(1). }. | byte_alignment( ) {; Descriptor. alignment_bit_equal_to_one; f(1). while( !byte_aligned( ) ). alignment_bit_equal_to_zero; f(1). }. |

Table 1. rbsp_trailing_bits( ) and byte_alignment( )

The slice header (SH) syntax table ends with byte_alignement( ) to make the SH byte aligned. However, when the last bit of SODB in the SH is the last bit of a byte-aligned byte, it is not useful to signal 8 additional bits (0x80) in the bitstream.

# Proposals

## Proposal 1: Use the syntax element alignment_bit_equal_to_one to indicate the last bit in arithmetic decode terminate process

The changes for proposal 1 are aligned with the HEVC text. The HEVC text (Rec. ITU-T H.265 11/2019) regarding “Decoding process for binary decisions before termination” is as follows:

9.3.4.3.5 Decoding process for binary decisions before termination

Inputs to this process are bits from slice segment data and the variables ivlCurrRange and ivlOffset.

Outputs of this process are the updated variables ivlCurrRange and ivlOffset, and the decoded value binVal.

This decoding process applies to decoding of end_of_slice_segment_flag, end_of_subset_one_bit and pcm_flag corresponding to ctxTable equal to 0 and ctxIdx equal to 0. Figure 9-9 shows the flowchart of the corresponding decoding process, which is specified as follows:

First, the value of ivlCurrRange is decremented by 2. Then, the value of ivlOffset is compared to the value of ivlCurrRange and then the following applies:

– If ivlOffset is greater than or equal to ivlCurrRange, the variable binVal is set equal to 1, no renormalization is carried out, and CABAC decoding is terminated. The last bit inserted in register ivlOffset is equal to 1. When decoding end_of_slice_segment_flag, this last bit inserted in register ivlOffset is interpreted as rbsp_stop_one_bit. When decoding end_of_subset_one_bit, this last bit inserted in register ivlOffset is interpreted as alignment_bit_equal_to_one.

– Otherwise (ivlOffset is less than ivlCurrRange), the variable binVal is set equal to 0 and renormalization is performed as specified in clause 9.3.4.3.3.

Proposed changes on top of the VVC draft (JVET-Q2001-vE) are shown below with added text in yellow and removed text in red strike-through.

9.3.4.3.5 Decoding process for binary decisions before termination

Inputs to this process are bits from slice data and the variables ivlCurrRange and ivlOffset.

Outputs of this process are the updated variables ivlCurrRange and ivlOffset, and the decoded value binVal.

This decoding process applies to decoding of end_of_slice_one_bit, end_of_tile_one_bit, and end_of_subset_one_bit corresponding to ctxTable equal to 0 and ctxIdx equal to 0. Figure 19 shows the flowchart of the corresponding decoding process, which is specified as follows:

First, the value of ivlCurrRange is decremented by 2. Then, the value of ivlOffset is compared to the value of ivlCurrRange and further steps are specified as follows:

–	If ivlOffset is greater than or equal to ivlCurrRange, the variable binVal is set equal to 1, no renormalization is carried out, and CABAC decoding is terminated. The last bit inserted in register ivlOffset is equal to 1. When decoding end_of_slice_one_bit, this last bit inserted in register ivlOffset is interpreted as rbsp_stop_one_bit. When decoding end_of_tile_one_bit, and end_of_subset_one_bit, this last bit inserted in register ivlOffset is interpreted as alignment_bit_equal_to_one rbsp_stop_one_bit.

–	Otherwise (ivlOffset is less than ivlCurrRange), the variable binVal is set equal to 0 and renormalization is performed as specified in subclause 9.3.4.3.3.

## Proposal 2: Remove the byte_alignment( ) in the slice header (SH)

We propose to remove the byte_alignment( ) in the SH. When the last bit of SODB in the SH is not the last bit of a byte-aligned byte, one or more zero-valued bits are present to achieve byte alignment. Note that we propose to remove the leading bit equal to 1 that is present in byte_alignment( ).

Proposed changes on top of the VVC draft (JVET-Q2001-vE) are shown below with added text in yellow and removed text in red strike-through.

| slice_header( ) { | Descriptor |
| --- | --- |
| … | … |
| if( slice_header_extension_present_flag ) { |  |
| slice_header_extension_length | ue(v) |
| for( i = 0; i < slice_header_extension_length; i++) |  |
| slice_header_extension_data_byte[ i ] | u(8) |
| } |  |
| byte_alignment( ) |  |
| while( !byte_aligned( ) ) |  |
| alignment_bit_equal_to_zero /* equal to 0 */ | f(1) |
| } |  |

## Proposal 3: Remove one of the identical syntax table

Based on proposal 2, given that the syntax table byte_alignment( ) is only used in the slice_data( ), we propose to remove the syntax table byte_alignment( ) from the VVC specification. It is proposed to let the syntax table rbsp_trailing_bits( ) replace the byte_alignment( ) in slice_data( ), and let the syntax element rbsp_stop_one_bit indicate the last bit in the arithmetic decode terminate process.

Proposed changes on top of the VVC draft (JVET-Q2001-vE) are shown below with added text in yellow and removed text in red strike-through.

7.3.2.15 RBSP trailing bits syntax

| rbsp_trailing_bits( ) { | Descriptor |
| --- | --- |
| rbsp_stop_one_bit /* equal to 1 */ | f(1) |
| while( !byte_aligned( ) ) |  |
| rbsp_alignment_zero_bit /* equal to 0 */ | f(1) |
| } |  |

7.3.2.16 Byte alignment syntax

| byte_alignment( ) { | Descriptor |
| --- | --- |
| alignment_bit_equal_to_one /* equal to 1 */ | f(1) |
| while( !byte_aligned( ) ) |  |
| alignment_bit_equal_to_zero /* equal to 0 */ | f(1) |
| } |  |

7.3.7.1 General slice header syntax

| slice_header( ) { | Descriptor |
| --- | --- |
| … | … |
| if( slice_header_extension_present_flag ) { |  |
| slice_header_extension_length | ue(v) |
| for( i = 0; i < slice_header_extension_length; i++) |  |
| slice_header_extension_data_byte[ i ] | u(8) |
| } |  |
| byte_alignment( ) |  |
| while( !byte_aligned( ) ) |  |
| rbsp_alignment_zero_bit /* equal to 0 */ | f(1) |
| } |  |

7.3.10.1 General slice data syntax

| slice_data( ) { | Descriptor |
| --- | --- |
| FirstCtbRowInSlice = 1 |  |
| for( i = 0; i < NumCtusInCurrSlice; i++ ) { |  |
| … |  |
| } |  |
| coding_tree_unit( ) |  |
| if( i = = NumCtusInCurrSlice − 1 ) |  |
| end_of_slice_one_bit /* equal to 1 */ | ae(v) |
| else if( CtbAddrX = = CtbToTileColBd[ CtbAddrX + 1 ] − 1 ) { |  |
| if( CtbAddrY = = CtbToTileRowBd[ CtbAddrY + 1 ] − 1 ) { |  |
| end_of_tile_one_bit /* equal to 1 */ | ae(v) |
| rbsp_trailing_bits( )byte_alignment( ) |  |
| } else if( entropy_coding_sync_enabled_flag ) { |  |
| end_of_subset_one_bit /* equal to 1 */ | ae(v) |
| rbsp_trailing_bits( )byte_alignment( ) |  |
| } |  |
| FirstCtbRowInSlice = 0 |  |
| } |  |
| } |  |
| } |  |

7.4.3.15 RBSP trailing bits semantics

rbsp_stop_one_bit shall be equal to 1.

rbsp_alignment_zero_bit shall be equal to 0.

7.4.3.16 Byte alignment semantics

alignment_bit_equal_to_one shall be equal to 1.

alignment_bit_equal_to_zero shall be equal to 0.

9.3.4.3.5 Decoding process for binary decisions before termination

Inputs to this process are bits from slice data and the variables ivlCurrRange and ivlOffset.

Outputs of this process are the updated variables ivlCurrRange and ivlOffset, and the decoded value binVal.

This decoding process applies to decoding of end_of_slice_one_bit, end_of_tile_one_bit, and end_of_subset_one_bit corresponding to ctxTable equal to 0 and ctxIdx equal to 0. Figure 19 shows the flowchart of the corresponding decoding process, which is specified as follows:

First, the value of ivlCurrRange is decremented by 2. Then, the value of ivlOffset is compared to the value of ivlCurrRange and further steps are specified as follows:

–	If ivlOffset is greater than or equal to ivlCurrRange, the variable binVal is set equal to 1, no renormalization is carried out, and CABAC decoding is terminated. The last bit inserted in register ivlOffset is equal to 1. When decoding end_of_slice_one_bit, end_of_tile_one_bit, and end_of_subset_one_bit, this last bit inserted in register ivlOffset is interpreted as rbsp_stop_one_bit.

–	Otherwise (ivlOffset is less than ivlCurrRange), the variable binVal is set equal to 0 and renormalization is performed as specified in subclause 9.3.4.3.3.

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).