| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0087-v1 |
| --- | --- |

| Title: | AHG12: Modification of subpicture information in slice header |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Name(s) Woong Lim, Gun Bang | Email: | woong.lim@etri.re.kr, gbang@etri.re.kr |
| Source: | ETRI |  |  |

_____________________________

- Abstract

This contribution proposes to modify syntax elements related to subpicture in slice header. VVC slice header refers to subpic_info_present_flag signaled in SPS and parsing dependency may exist. To remove the dependency, slice level subpicture-related syntax element, slice_subpic_info_present_flag, is proposed.

Introduction

In the current VVC specification [1], the syntax structure of slice header and semantic of subpic_info_present_flag in SPS are shown below.

| slice_header( ) { | Descriptor |
| --- | --- |
| picture_header_in_slice_header_flag | u(1) |
| if( picture_header_in_slice_header_flag ) |  |
| picture_header_structure( ) |  |
| if( subpic_info_present_flag ) |  |
| slice_subpic_id | u(v) |
| … |  |
| } |  |

subpic_info_present_flag equal to 1 specifies that subpicture information is present for the CLVS and there may be one or more than one subpicture in each picture of the CLVS. subpic_info_present_flag equal to 0 specifies that subpicture information is not present for the CLVS and there is only one subpicture in each picture of the CLVS.

When res_change_in_clvs_allowed_flag is equal to 1, the value of subpic_info_present_flag shall be equal to 0.

NOTE 3 – When a bitstream is the result of a sub-bitstream extraction process and contains only a subset of the subpictures of the input bitstream to the sub-bitstream extraction process, it might be required to set the value of subpic_info_present_flag equal to 1 in the RBSP of the SPSs.

Proposal

The proposed simple specification change (yellow highlighted) is shown below.

| slice_header( ) { | Descriptor |
| --- | --- |
| picture_header_in_slice_header_flag | u(1) |
| if( picture_header_in_slice_header_flag ) |  |
| picture_header_structure( ) |  |
| slice_subpic_info_present_flag | u(1) |
| if( slice_subpic_info_present_flag ) |  |
| slice_subpic_id | u(v) |
| … |  |
| } |  |

slice_subpic_info_present_flag shall be equal to subpic_info_present_flag.

Reference

[1] B. Bross, J. Chen, S. Liu, Y. Wang, “Versatile Video Coding (Draft 8)”, JVET-Q2001, 17th meeting, Brussels, BE, January 2020.

Patent rights declaration(s)

ETRI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).