| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0090 |
| --- | --- |

| Title: | AHG9: On Video Usability Information |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Virginie Drugeon | Tel: Email: | +49-173-6192228 virginie.drugeon@eu.panasonic.com |
| Source: | Panasonic |  |  |

_____________________________

# Abstract

In the current specification text, the parsing of the VUI syntax structure has a dependency on two external variables (GeneralProgressiveSourceFlag and GeneralInterlacedSourceFlag), which is undesirable. It is suggested to signal these two external variables as syntax elements within the VUI syntax structure. Currently the corresponding information is signalled within the general_constraint_info syntax structure. It is suggested to either move this information from the general_constraint_info syntax structure to the VUI syntax structure, or at least repeat the information within the VUI syntax structure.

# Introduction

The information contained in the VUI (Video Usability Information) syntax structure is not required for decoding the luma and chroma samples of the video signal. However, some of the information is essential for proper rendering of the signal on a display and is becoming even more essential in today’s world.

Signalling chroma sample location for progressive content has become essential now that recommendations BT. 2020 and BT. 2100 use the type 2 location, instead of the type 0 location used for previous formats. Unfortunately, in bitstreams conforming to previous video coding standards, this information tended to be forgotten. One reason may have been that the name of the fields (chroma_sample_loc_type_top_field and chroma_sample_loc_type_bottom_field) tended to give the impression that this was an information relevant for interlaced content only. In addition, it is unclear how the display should react to different values of these two fields for progressive content.

The group has therefore decided during previous discussions about JVET-Q0042 [1] to have only one chroma location type field for progressive content, conditioned based on the two flags general_progressive_source_flag and general_interlaced_source_flag. Unfortunately, these two flags are currently signalled within the general_constraint_info syntax structure, therefore creating a parsing dependency on external variables within the VUI syntax structure. Since these two flags convey information about the encoded video signal rather than the bitstream (contrary to most of the other flags from the general_constraint_info syntax structure), it is suggested to either move these flags to the VUI or at least repeat them within the VUI.

# Proposed VUI syntax

| vui_parameters( ) { | Descriptor |
| --- | --- |
| vui_aspect_ratio_info_present_flag | u(1) |
| if( vui_aspect_ratio_info_present_flag ) { |  |
| vui_aspect_ratio_constant_flag | u(1) |
| vui_aspect_ratio_idc | u(8) |
| if( vui_aspect_ratio_idc = = 255 ) { |  |
| vui_sar_width | u(16) |
| vui_sar_height | u(16) |
| } |  |
| } |  |
| vui_overscan_info_present_flag | u(1) |
| if( vui_overscan_info_present_flag ) |  |
| vui_overscan_appropriate_flag | u(1) |
| vui_colour_description_present_flag | u(1) |
| if( vui_colour_description_present_flag ) { |  |
| vui_colour_primaries | u(8) |
| vui_transfer_characteristics | u(8) |
| vui_matrix_coeffs | u(8) |
| vui_full_range_flag | u(1) |
| } |  |
| general_progressive_source_flag | u(1) |
| general_interlaced_source_flag | u(1) |
| vui_chroma_loc_info_present_flag | u(1) |
| if( vui_chroma_loc_info_present_flag ) { |  |
| if( general_progressive_source_flag && !general_interlaced_source_flag ) |  |
| vui_chroma_sample_loc_type_frame | ue(v) |
| else { |  |
| vui_chroma_sample_loc_type_top_field | ue(v) |
| vui_chroma_sample_loc_type_bottom_field | ue(v) |
| } |  |
| } |  |
| } |  |

The semantics of these two flags should stay the same as the semantics currently specified for the general_constraint_info syntax structure:

general_progressive_source_flag and general_interlaced_source_flag are interpreted as follows:

–	If general_progressive_source_flag is equal to 1 and general_interlaced_source_flag is equal to 0, the source scan type of the pictures should be interpreted as progressive only.

–	Otherwise, if general_progressive_source_flag is equal to 0 and general_interlaced_source_flag is equal to 1, the source scan type of the pictures should be interpreted as interlaced only.

–	Otherwise, if general_progressive_source_flag is equal to 0 and general_interlaced_source_flag is equal to 0, the source scan type of the pictures should be interpreted as unknown or unspecified.

–	Otherwise (general_progressive_source_flag is equal to 1 and general_interlaced_source_flag is equal to 1), the source scan type of each picture is indicated at the picture level using the syntax element source_scan_type in a frame-field information SEI message. It is a requirement of bitstream conformance that when general_progressive_source_flag is equal to 1 and general_interlaced_source_flag is equal to 1, a frame-field information SEI message shall be present in each AU.

# References

- V. Drugeon, JVET-Q0042 “AHG9: On Video Usability Information”, Brussels, 17th JVET meeting, 7–17 Jan. 2020

# Patent rights declaration(s)

Panasonic Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).