| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0092 |
| --- | --- |

| Title: | AHG12: Subpicture size calculation for subpicture extraction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Virginie Drugeon | Tel: Email: | +49-173-6192228 virginie.drugeon@eu.panasonic.com |
| Source: | Panasonic |  |  |

_____________________________

# Abstract

The author of the present document believes that there is a bug in the current subpicture extraction process whereby the size for a subpicture is claimed to be wrongly calculated when the subpicture is located at the bottom and/or right border of a picture that has a size that is not a multiple of the CTU size. It is suggested to solve this bug by calculating the correct size of the subpictures in luma samples based on the position of the subpicture to extract within the picture.

# Problem Statement

When extracting a subpicture from a bitstream, several syntax elements from the SPS and the PPS must be rewritten. In particular, the size of the subpicture within the sub-bitstream has to be rewritten. The current subpicture extraction process in the VVC draft states the following in clause C.7:

Rewrite the value of the pic_width_max_in_luma_samples and pic_height_max_in_luma_samples in all referred to SPS NAL units and the value of pic_width_in_luma_samples and pic_height_in_luma_samples in all referred to PPS NAL units to be equal to subpic_width_minus1[ SubpicIdList[ subPicId ] ] and subpic_height_minus1[ SubpicIdList[ subPicId  ] ].

However, there is an issue for subpictures that are at the bottom and/or right border of the picture when the picture size is not a multiple of the CTU size. In that case, there is automatic Coding Unit splitting at the border. If the subpicture size is rewritten according to the current specification, the size of all subpictures will always be a multiple of the CTU size since the values of subpic_width_minus1 and subpic_height_minus1 are given in terms of CTU. It means that automatic Coding Unit splitting will not be applied, and therefore, that the subpictures located at the bottom and/or right borders of the picture cannot be extracted correctly. Figure 1 shows an example of this issue, whereby the full picture is 1920x1080 pixels, which means that its height is not a multiple of the CTU size, and that there is automatic CU splitting at the bottom of the picture. When extracting subpictures G, H, J or I from Figure 1 using the size rewriting process from above, these subpictures will not be decodable by the decoder, because 72 rows will be missing at the bottom due to automatic CU splitting.

Figure 1: Example showing the issue with subpicture extraction when the height of the picture is not a multiple of the CTU size

# Correct calculation of subpicture size

To solve this issue the subpicture size must be calculated differently based on its position within the picture. If the subpicture is located at the bottom or right boundary of the picture, its size must be calculated based on the full picture size and its top left position, instead of using the width and height information signalled in the SPS. Figure 2 shows the calculation of the heights of subpictures F and L for the same example as in Figure 1, where idxF is the index of subpicture F and idxL the index of subpicture L.

Figure 2: Example of calculation of height of two subpictures

The following modifications to the subpicture sub-bitstream extraction process are proposed:

For the i-th layer with i in the range of 0 to NumLayersInOls[ targetOlsIdx ] − 1, the following applies.

–	subpicId is set equal to the value of subpicIdTarget[ i ].

–	The variable subpicIdx is derived to be such that SubpicIdVal[ subpicIdx ] is equal to subpicId.

–	Rewrite the value of general_level_idc in the profile_tier_level() syntax structure in all referred to SPS NAL units with sps_ptl_dpb_hrd_params_present_flag equal to 1 to be equal to SubpicSetLevelIdc derived in D.3.8 for the set of subpictures consisting of the subpicture with subpicture ID equal to subpicId.

–	Rewrite the values of cpb_size_value_minus1[ tIdTarget ][ j ] and bit_rate_value_minus1[ tIdTarget ][ j ] of the j-th CPB in the ols_hrd_idx[ targetOlsIdx ]-th entry in the list of ols_hrd_parameters() syntax structures in all referred to SPS NAL units to correspond to SubpicCpbSizeVcl[ 0 ][ SubpicIdList[ subPicId ] ], SubpicCpbSizeNal[ 0 ][ SubpicIdList[ subPicId ] ], SubpicBitrateVcl[ 0 ][ SubpicIdList[ subPicId ] ], and SubpicBitrateNal[ 0 ][ SubpicIdList[ subPicId ] ] as derived in D.3.8 for the subpicture with subpicture ID equal to subpicId and j in the range of 0 to hrd_cpb_cnt_minus1.

–	The variables subpicWidthInLumaSamples and subpicHeightInLumaSamples are derived as follows:

if ( ( subpic_top_left_x[ subpicIdx ] + subpic_width_minus1[ subpicIdx ] + 1 ) * CtbSizeY >= pic_width_in_luma_samples )
	subpicWidthInLumaSamples = pic_width_in_luma_samples – 
									subpic_top_left_x[ subpicIdx ] * CtbSizeY
else
	subpicWidthInLumaSamples = (subpic_width_minus1[ subpicIdx ] + 1) * CtbSizeY
if ( ( subpic_top_left_y[ subpicIdx ] + subpic_height_minus1[ subpicIdx ] +1 ) * CtbSizeY >= pic_height_in_luma_samples )
	subpicHeightInLumaSamples = pic_height_in_luma_samples – 
									subpic_top_left_y[ subpicIdx ] * CtbSizeY
else
	subpicHeightInLumaSamples = (subpic_height_minus1[ subpicIdx ] + 1) * CtbSizeY

–	Rewrite the value of the pic_width_max_in_luma_samples and pic_height_max_in_luma_samples in all referred to SPS NAL units and the value of pic_width_in_luma_samples and pic_height_in_luma_samples in all referred to PPS NAL units to be equal to subpic_width_minus1[ SubpicIdList[ subPicId ] ]  subpicWidthInLumaSamples and subpic_height_minus1[ SubpicIdList[ subPicId  ] ] subpicHeightInLumaSamples.

–	Rewrite the value of sps_num_subpics_minus1 in all referenced SPS NAL units and pps_num_subpics_minus1 in all referenced PPS NAL units to 0.

# Conclusion

A bug has been identified in the subpicture extraction process, whereby the size calculation for a subpicture is wrong when the subpicture is located at the bottom and/or right border of a picture that has a size that is not a multiple of the CTU size. It is suggested to solve this bug by calculating the correct size of the subpictures in luma samples based on the position of the subpicture to extract within the picture.

# Patent rights declaration(s)

Panasonic Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).