| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0100-v1 |
| --- | --- |

| Title: | On Decoding Unit Information Signalling |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sachin Deshpande Jonatan Samuelsson Andrew Segall Philip Cowan | Tel: Email: | +1 360 817 8486 sdeshpande@sharplabs.com |
| Source: | Sharp Labs of America, Inc. |  |  |

_____________________________

# Abstract

Modifications are proposed related to decoding unit information signalling.

Following is proposed:

- Proposal 1: A signalling and semantics modification is proposed to decoding unit information SEI message for the syntax element dui_sublayer_delays_present_flag[ bp_max_sublayers_minus1].

- Proposal 2: Since the inference for du_spt_cpb_removal_delay_increment[ i ] when i is less than bp_max_sublayers_minus1 uses the value of du_spt_cpb_removal_delay_increment[ bp_max_sublayers_minus1 ], it is proposed to signal the for loop for syntax elements dui_sublayer_delays_present_flag[ i ]  and du_spt_cpb_removal_delay_increment[ i ] in the reverse order.

# Introduction

VVC WD [1] includes HRD which supports decoding unit based HRD signalling and operation. This is achieved by signalling in picture timing SEI message and decoding unit SEI message and SPS.

Modifications are proposed related to decoding unit information signalling.

Following is proposed:

- Proposal 1: It is asserted that it is unnecessary to signal the syntax element dui_sublayer_delays_present_flag[ bp_max_sublayers_minus1] and its value should instead be inferred. This is because the syntax element du_spt_cpb_removal_delay_increment [ bp_max_sublayers_minus1 ] is used for the inference of du_spt_cpb_removal_delay_increment[ i ], for i in the range of 0 to bp_max_sublayers_minus1−1, when dui_sublayer_delays_present_flag[ i ] is equal to 0 and thus must be signalled. A signalling and semantics modification is proposed to decoding unit information SEI message for this.

- Proposal 2: Since the inference for du_spt_cpb_removal_delay_increment[ i ] when i is less than bp_max_sublayers_minus1 uses the value of du_spt_cpb_removal_delay_increment[ bp_max_sublayers_minus1 ], it is proposed to signal the for loop for syntax elements dui_sublayer_delays_present_flag[ i ]  and du_spt_cpb_removal_delay_increment[ i ] in the reverse order. This will allow knowing the value of du_spt_cpb_removal_delay_increment[ bp_max_sublayers_minus1 ] before the other values du_spt_cpb_removal_delay_increment[ i ] (which may or may not be present) for i less than bp_max_sublayers_minus1, which need to use this value for their inference (if one or more of them are not present based on the value of the dui_sublayer_delays_present_flag[ i ]  ).

# Proposal

## Proposal 1

It is asserted that it is unnecessary to signal the syntax element dui_sublayer_delays_present_flag[ bp_max_sublayers_minus1 ] and its value should instead be inferred. This is because the syntax element du_spt_cpb_removal_delay_increment [ bp_max_sublayers_minus1 ] is used for the inference of du_spt_cpb_removal_delay_increment[ i ], for i in the range of 0 to bp_max_sublayers_minus1−1, when dui_sublayer_delays_present_flag[ i ] is equal to 0 and thus must be signalled.

A signalling and semantics modification is proposed to decoding unit information SEI message for this.

The proposed changes are shown highlighted below compared to VVC WD.

D.5.1. DU information SEI message syntax

| decoding_unit_info( payloadSize ) { | Descriptor |
| --- | --- |
| decoding_unit_idx | ue(v) |
| if( !du_cpb_params_in_pic_timing_sei_flag ) |  |
| for( i = TemporalId; i <= bp_max_sublayers_minus1; i++ ) { |  |
| if( i < bp_max_sublayers_minus1) |  |
| dui_sublayer_delays_present_flag[ i ] | u(1) |
| if( dui_sublayer_delays_present_flag[ i ] ) |  |
| du_spt_cpb_removal_delay_increment[ i ] | u(v) |
| } |  |
| if( !du_dpb_params_in_pic_timing_sei_flag ) |  |
| dpb_output_du_delay_present_flag | u(1) |
| if( dpb_output_du_delay_present_flag ) |  |
| pic_spt_dpb_output_du_delay | u(v) |
| } |  |

dui_sublayer_delays_present_flag[ i ] equal to 1 specifies that du_spt_cpb_removal_delay_increment[ i ] is present for the sublayer with TemporalId equal to i. dui_sublayer_delays_present_flag[ i ] equal to 0 specifies that du_spt_cpb_removal_delay_increment[ i ] is not present for the sublayer with TemporalId equal to i. When du_cpb_params_in_pic_timing_sei_flag is equal to 0, the value of dui_sublayer_delays_present_flag[ bp_max_sublayers_minus1 ] is inferred to be equal to 1. Otherwise Wwhen not present, the value of dui_sublayer_delays_present_flag[ i ] is infered to be equal to 0.

du_spt_cpb_removal_delay_increment[ i ] specifies the duration, in units of clock sub-ticks, between the nominal CPB times of the last DU in decoding order in the current AU and the DU associated with the DU information SEI message when Htid is equal to i. This value is also used to calculate an earliest possible time of arrival of DU data into the CPB for the HSS, as specified in Annex C. The lenght of this syntax element is du_cpb_removal_delay_increment_length_minus1 + 1. When the DU associated with the DU information SEI message is the last DU in the current AU, the value of du_spt_cpb_removal_delay_increment[ i ] shall be equal to 0. When du_spt_cpb_removal_delay_increment[ i ] is not present for any value of i less than bp_max_sublayers_minus1, its value is inferred to be equal to du_spt_cpb_removal_delay_increment[ bp_max_sublayers_minus1 ].

## Proposal 2

Since the inference for du_spt_cpb_removal_delay_increment[ i ] when i is less than bp_max_sublayers_minus1 uses the value of du_spt_cpb_removal_delay_increment[ bp_max_sublayers_minus1 ], it is proposed to signal the for loop for syntax elements dui_sublayer_delays_present_flag[ i ]  and du_spt_cpb_removal_delay_increment[ i ] in the reverse order. This will allow knowing the value of du_spt_cpb_removal_delay_increment[ bp_max_sublayers_minus1 ] before the other values du_spt_cpb_removal_delay_increment[ i ] (which may or may not be present) for i less than bp_max_sublayers_minus1, which need to use this value for their inference (if one or more of them are not present based on the value of the dui_sublayer_delays_present_flag[ i ]  ).

The proposed changes are shown highlighted below compared to VVC WD (JVET-Q2001-vB).

D.5.1. DU information SEI message syntax

| decoding_unit_info( payloadSize ) { | Descriptor |
| --- | --- |
| decoding_unit_idx | ue(v) |
| if( !du_cpb_params_in_pic_timing_sei_flag ) |  |
| for( i = TemporalId; i <= bp_max_sublayers_minus1; i++ ) for( i = bp_max_sublayers_minus1; i >= TemporalId ; i−−) { |  |
| dui_sublayer_delays_present_flag[ i ] | u(1) |
| if( dui_sublayer_delays_present_flag[ i ] ) |  |
| du_spt_cpb_removal_delay_increment[ i ] | u(v) |
| } |  |
| if( !du_dpb_params_in_pic_timing_sei_flag ) |  |
| dpb_output_du_delay_present_flag | u(1) |
| if( dpb_output_du_delay_present_flag ) |  |
| pic_spt_dpb_output_du_delay | u(v) |
| } |  |

# Conclusion

This document proposes modifications to Decoding Unit information signalling in VVC. It is proposed to adopt the proposals in the next version of VVC WD.

# Patent rights declaration(s)

Sharp Labs of America, Inc., may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

[1] JVET-Q2001-vE, Versatile Video Coding (Draft 8), Brussels, January 2020