| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0102-v1 |
| --- | --- |

| Title: | AHG9: On Reference Picture List Override Signalling |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sachin Deshpande Takeshi Chujoh Tomohiro Ikai Jonatan Samuelsson Andrew Segall | Tel: Email: | +1 360 817 8486 sdeshpande@sharplabs.com +81-70-1617-3470 chujoh.takeshi@sharp.co.jp +81-43-299-8526 ikai.tomohiro@sharp.co.jp |
| Source: | Sharp Labs of America, Inc., Sharp Corporation |  |  |

_____________________________

# Abstract

It is proposed to allow signalling of reference picture list override information in picture header. A corresponding modification is proposed to the signalling in slice header and weighted prediction table. It is asserted that since the reference picture list information can be signalled at the picture header, allowing signalling the reference picture list override information at the picture header makes a consistent design.

# Introduction

In VVC WD8 [1] reference picture list information is signalled in ref_pic_lists() structure. This information can be included in picture header or in slice header. However currently the reference picture list override information can only be included in slice header.

It is proposed to allow signalling of reference picture list override information in picture header. A corresponding modification is proposed to the signalling in slice header. It is asserted that since the reference picture list information can be signalled at the picture header, allowing signalling the reference picture list override information at the picture header makes a consistent design. Additionally this makes it unnecessary to signal syntax elements num_l0_weights and num_l1_weights and instead their values can be inferred.

# Proposal

It is proposed to allow signalling of reference picture list override information in picture header. A corresponding modification is proposed to the signalling in slice header and weighted prediction table.

The proposed changes are shown highlighted below compared to VVC WD8 [1].

| picture_header_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| pic_rpl_present_flag | u(1) |
| if( rpl_info_in_ph_flag ) { |  |
| ref_pic_lists() |  |
| pic_num_ref_idx_active_override_flag | u(1) |
| if( pic_num_ref_idx_active_override_flag ) |  |
| for( i = 0; i < 2; i++ ) |  |
| if( num_ref_entries[ i ][ RplsIdx[ i ] ] > 1 ) |  |
| pic_num_ref_idx_active_minus1[ i ] | ue(v) |
| } |  |
| … |  |
| } |  |

Semantics in PPS:

num_ref_idx_default_active_minus1[ i ] plus 1, when i is equal to 0, specifies the inferred value of the variable NumRefIdxActive[ 0 ] for P or B slices with num_ref_idx_active_override_flag equal to 0, and, when i is equal to 1, specifies the inferred value of NumRefIdxActive[ 1 ] for B slices with num_ref_idx_active_override_flag equal to 0. The value of num_ref_idx_default_active_minus1[ i ] shall be in the range of 0 to 14, inclusive.

Semantics in picture header:

pic_num_ref_idx_active_override_flag equal to 1 specifies that the syntax element pic_num_ref_idx_active_minus1[ 0 ] and pic_num_ref_idx_active_minus1[ 1 ] is present or inferred. pic_num_ref_idx_active_override_flag equal to 0 specifies that the syntax elements pic_num_ref_idx_active_minus1[ 0 ] and pic_num_ref_idx_active_minus1[ 1 ] are not present. When not present, the value of pic_num_ref_idx_active_override_flag is inferred to be equal to 0.

pic_num_ref_idx_active_minus1[ i ] is used for the derivation of the variable NumRefIdxActive[ i ]. The value of pic_num_ref_idx_active_minus1[ i ] shall be in the range of 0 to 14, inclusive.

When pic_num_ref_idx_active_override_flag is equal to 1, and pic_num_ref_idx_active_minus1[ i ] is not present, pic_num_ref_idx_active_minus1[ i ] is inferred to be equal to 0.

The variable NumRefIdxActive[ i ] is derived as follows:

for( i = 0; i < 2; i++ ) {
		if( pic_num_ref_idx_active_override_flag )
			NumRefIdxActive[ i ] = pic_num_ref_idx_active_minus1[ i ] + 1	
		else {
			if( num_ref_entries[ i ][ RplsIdx[ i ] ]  >=  num_ref_idx_default_active_minus1[ i ] + 1 )
				NumRefIdxActive[ i ] = num_ref_idx_default_active_minus1[ i ] + 1
			else
				NumRefIdxActive[ i ] = num_ref_entries[ i ][ RplsIdx[ i ] ]
		}
	}
}

The value of NumRefIdxActive[ i ] − 1 specifies the maximum reference index for reference picture list i that may be used to decode the slices of the picture. When the value of NumRefIdxActive[ i ] is equal to 0, no reference index for reference picture list i may be used to decode the slices of the picture.

Additionally, following modification is made to the slice header:

| slice_header( ) { |  |
| --- | --- |
| … |  |
| if( !rpl_info_in_ph_flag &&( ( nal_unit_type != IDR_W_RADL && nal_unit_type != IDR_N_LP ) ¦ ¦ sps_idr_rpl_present_flag ) ) |  |
| ref_pic_lists() |  |
| if( !rpl_info_in_ph_flag ¦ ¦ && ( ( nal_unit_type != IDR_W_RADL && nal_unit_type !=IDR_N_LP ) ¦ ¦ sps_idr_rpl_present_flag ) ) { |  |
| if( ( slice_type != I && num_ref_entries[ 0 ][ RplsIdx[ 0 ] ] > 1 ) ¦ ¦ ( slice_type = = B && num_ref_entries[ 1 ][ RplsIdx[ 1 ] ] > 1 ) ) { |  |
| num_ref_idx_active_override_flag | u(1) |
| if( num_ref_idx_active_override_flag ) |  |
| for( i = 0; i < ( slice_type = = B ? 2: 1 ); i++ ) |  |
| if( num_ref_entries[ i ][ RplsIdx[ i ] ] > 1 ) |  |
| num_ref_idx_active_minus1[ i ] | ue(v) |
| } |  |
| } |  |
| … |  |
| } |  |

Additionally, following modification is made to the weighted prediction table:

| pred_weight_table( ) { | Descriptor |
| --- | --- |
| luma_log2_weight_denom | ue(v) |
| if( ChromaArrayType != 0 ) |  |
| delta_chroma_log2_weight_denom | se(v) |
| if( wp_info_in_ph_flag ) |  |
| num_l0_weights | ue(v) |
| for( i = 0; i < NumWeightsL0NumRefIdxActive[ 0 ]; i++ ) |  |
| luma_weight_l0_flag[ i ] | u(1) |
| if( ChromaArrayType != 0 ) |  |
| for( i = 0; i < NumWeightsL0; i++ ) |  |
| chroma_weight_l0_flag[ i ] | u(1) |
| for( i = 0; i < NumWeightsL0; i++ ) { |  |
| if( luma_weight_l0_flag[ i ] ) { |  |
| delta_luma_weight_l0[ i ] | se(v) |
| luma_offset_l0[ i ] | se(v) |
| } |  |
| if( chroma_weight_l0_flag[ i ] ) |  |
| for( j = 0; j < 2; j++ ) { |  |
| delta_chroma_weight_l0[ i ][ j ] | se(v) |
| delta_chroma_offset_l0[ i ][ j ] | se(v) |
| } |  |
| } |  |
| if( pps_weighted_bipred_flag && wp_info_in_ph_flag ) { |  |
| num_l1_weights | ue(v) |
| for( i = 0; i < NumWeightsL1NumRefIdxActive[ 1 ]; i++ ) |  |
| luma_weight_l1_flag[ i ] | u(1) |
| if( ChromaArrayType != 0 ) |  |
| for( i = 0; i < NumWeightsL1; i++ ) |  |
| chroma_weight_l1_flag[ i ] | u(1) |
| for( i = 0; i < NumWeightsL1; i++ ) { |  |
| if( luma_weight_l1_flag[ i ] ) { |  |
| delta_luma_weight_l1[ i ] | se(v) |
| luma_offset_l1[ i ] | se(v) |
| } |  |
| if( chroma_weight_l1_flag[ i ] ) |  |
| for( j = 0; j < 2; j++ ) { |  |
| delta_chroma_weight_l1[ i ][ j ] | se(v) |
| delta_chroma_offset_l1[ i ][ j ] | se(v) |
| } |  |
| } |  |
| } |  |
| } |  |

num_l0_weights specifies the number of weights signalled for entries in reference picture list 0 when wp_info_in_ph_flag is equal to 1. The value of num_l0_weights shall be in the range of 0 to Min( 15, num_ref_entries[ 0 ][ RplsIdx[ 0 ] ] ), inclusive.

If wp_info_in_ph_flag is equal to 1, the variable NumWeightsL0 is set equal to num_l0_weights. Otherwise (wp_info_in_ph_flag is equal to 0), NumWeightsL0 is set equal to NumRefIdxActive[ 0 ].

num_l1_weights specifies the number of weights signalled for entries in reference picture list 1 when pps_weighted_bipred_flag and wp_info_in_ph_flag are both equal to 1. The value of num_l1_weights shall be in the range of 0 to Min( 15, num_ref_entries[ 1 ][ RplsIdx[ 1 ] ] ), inclusive.

The variable NumWeightsL1 is derived as follows:

if( !pps_weighted_bipred_flag )
	NumWeightsL1 = 0
else if( wp_info_in_ph_flag )		(148)
	NumWeightsL1 = num_l1_weights
else
	NumWeightsL1 = NumRefIdxActive[ 1 ]

# Conclusion

This document proposes modifications related to the reference picture list override signalling in VVC. It is proposed to adopt the proposal in the next version of VVC draft.

# Patent rights declaration(s)

Sharp Labs of America, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Sharp Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

[1] JVET-Q2001, Versatile Video Coding (Draft 8), Brussels, January 2020