| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0104-v1 | Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: Alpbach, AT, 15–24 April 2020 | Document: JVET-Rxxx |
| --- | --- | --- | --- |

| Title: | AHG9: On Raster-Scan Slice Signalling |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sachin Deshpande Jonatan Samuelsson Andrew Segall Philip Cowan | Tel: Email: | +1 360 817 8486 sdeshpande@sharplabs.com |
| Source: | Sharp Labs of America, Inc. |  |  |

_____________________________

# Abstract

Following is proposed:

- Proposal 1: For raster-scan slices, it is proposed not to signal slice_address when picture header is included in slice header. The value of slice_address is inferred in this case.

- Proposal 2: For raster-scan slices, it is proposed not to signal num_tiles_in_slice_minus1 when picture header is included in slice header. As a part of proposal 2, also a missing inference rule is proposed for num_tiles_in_slice_minus1 syntax element. This missing inference rule is also applicable to VVC WD8 independent of the conditional signalling aspect of proposal 2.

# Introduction

VVC WD8 [1] includes two modes of slices, namely the raster-scan slice mode and the rectangular slice mode. In the raster-scan slice mode, a slice contains a sequence of complete tiles in a tile raster scan of a picture. In the rectangular slice mode, a slice contains either a number of complete tiles that collectively form a rectangular region of the picture or a number of consecutive complete CTU rows of one tile that collectively form a rectangular region of the picture.

It is asserted that when picture header is included in slice header, for raster-scan slices, the syntax elements slice_address and num_tiles_in_slice_minus1 do not need to be signalled. Instead their values can be inferred in those cases. Thus following is proposed:

- Proposal 1: For raster-scan slices, it is proposed not to signal slice_address when picture header is included in slice header. The value of slice_address is inferred in this case.

- Proposal 2: For raster-scan slices, it is proposed not to signal num_tiles_in_slice_minus1 when picture header is included in slice header. As a part of proposal 2, also a missing inference rule is proposed for num_tiles_in_slice_minus1 syntax element. This missing inference rule is also applicable to VVC WD8 independent of the conditional signalling aspect of proposal 2.

# Proposal

## Proposal 1

For raster-scan and rectangular slices, it is proposed not to signal slice_address when picture header is included in slice header. The value of slice_address is inferred in this case.

Essentially when picture header is in slice header, slice_address is not signalled for raster-scan slices and rectangular slices and instead it is inferred to be equal to 0.

### Slice header syntax

#### General slice header syntax

| slice_header( ) { | Descriptor |
| --- | --- |
| picture_header_in_slice_header_flag | u(1) |
| if( picture_header_in_slice_header_flag ) |  |
| picture_header_structure( ) |  |
| if( subpic_info_present_flag ) |  |
| slice_subpic_id | u(v) |
| if( ( rect_slice_flag && NumSlicesInSubpic[ CurrSubpicIdx ] > 1 ) ¦ ¦ ( !rect_slice_flag && NumTilesInPic > 1 && !picture_header_in_slice_header_flag) ) |  |
| slice_address | u(v) |
| for( i = 0; i < NumExtraShBits; i++ ) |  |
| sh_extra_bit[ i ] | u(1) |
| if( !rect_slice_flag && NumTilesInPic > 1 ) |  |
| num_tiles_in_slice_minus1 | ue(v) |
| if( ph_inter_slice_allowed_flag ) |  |
| slice_type | ue(v) |
| … |  |
| } | u(1) |

Table 8

slice_address specifies the slice address of the slice. When not present, the value of slice_address is inferred to be equal to 0.

If rect_slice_flag is equal to 0, the following applies:

- The slice address is the raster-scan tile index.

- The length of slice_address is Ceil( Log2 ( NumTilesInPic ) ) bits.

- The value of slice_address shall be in the range of 0 to NumTilesInPic − 1, inclusive.

Otherwise (rect_slice_flag is equal to 1), the following applies:

- The slice address is the subpicture-level slice index of the slice.

- The length of slice_address is Ceil( Log2( NumSlicesInSubpic[ CurrSubpicIdx ] ) ) bits.

- The value of slice_address shall be in the range of 0 to NumSlicesInSubpic[ CurrSubpicIdx ] − 1, inclusive.

It is a requirement of bitstream conformance that the following constraints apply:

- If rect_slice_flag is equal to 0 or subpic_info_present_flag is equal to 0, the value of slice_address shall not be equal to the value of slice_address of any other coded slice NAL unit of the same coded picture.

- Otherwise, the pair of slice_subpic_id and slice_address values shall not be equal to the pair of slice_subpic_id and slice_address values of any other coded slice NAL unit of the same coded picture.

- The shapes of the slices of a picture shall be such that each CTU, when decoded, shall have its entire left boundary and entire top boundary consisting of a picture boundary or consisting of boundaries of previously decoded CTU(s).

## Proposal 2

For raster-scan slices, it is proposed not to signal num_tiles_in_slice_minus1 when picture header is included in slice header. As a part of proposal 2, also a missing inference rule is proposed for num_tiles_in_slice_minus1 syntax element. This missing inference rule is also applicable to VVC WD8 independent of the conditional signalling aspect of proposal 2.

| slice_header( ) { | Descriptor |
| --- | --- |
| picture_header_in_slice_header_flag | u(1) |
| if( picture_header_in_slice_header_flag ) |  |
| picture_header_structure( ) |  |
| if( subpic_info_present_flag ) |  |
| slice_subpic_id | u(v) |
| if( ( rect_slice_flag && NumSlicesInSubpic[ CurrSubpicIdx ] > 1 ) ¦ ¦ ( !rect_slice_flag && NumTilesInPic > 1) ) |  |
| slice_address | u(v) |
| for( i = 0; i < NumExtraShBits; i++ ) |  |
| sh_extra_bit[ i ] | u(1) |
| if( !rect_slice_flag && NumTilesInPic > 1 && !picture_header_in_slice_header_flag) |  |
| num_tiles_in_slice_minus1 | ue(v) |
| if( ph_inter_slice_allowed_flag ) |  |
| slice_type | ue(v) |
| … |  |
| } | u(1) |

It is asserted that the following addition of missing inference rule is applicable to VVC WD8 also independently of the conditional signalling aspect of proposal 2 above.

num_tiles_in_slice_minus1 plus 1, when present, specifies the number of tiles in the slice. The value of num_tiles_in_slice_minus1 shall be in the range of 0 to NumTilesInPic − 1, inclusive. When not present  num_tiles_in_slice_minus1 is inferred to be equal to NumTilesInPic −1.

# Conclusion

This document proposes modifications to slice header signalling in VVC. It is proposed to adopt the proposals in the next version of VVC WD.

# Patent rights declaration(s)

Sharp Labs of America, Inc., may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

[1] JVET-Q2001-vE, Versatile Video Coding (Draft 8), Brussels, January 2020