| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0105-v1 |
| --- | --- |

| Title: | AHG9: On CC-ALF Signalling in SPS |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sachin Deshpande Andrew Segall Jonatan Samuelsson Philip Cowan | Tel: Email: | +1 360 817 8486 sdeshpande@sharplabs.com |
| Source: | Sharp Labs of America, Inc. |  |  |

_____________________________

# Abstract

When sps_alf_enabled_flag is equal to 0 and/or when ChromaArrayType is equal to 0, sps_ccalf_enabled_flag is not signalled.

A missing inference rule is proposed for sps_ccalf_enabled_flag for when it is not present.

# Introduction

In VVC WD8 [1] a syntax element sps_ccalf_enabled_flag is conditionally signalled in SPS to control the presence of CCALF related syntax elements in PH and SH.

When sps_alf_enabled_flag is equal to 0 and/or when ChromaArrayType is equal to 0, sps_ccalf_enabled_flag is not signaled.

A missing inference rule is proposed for sps_ccalf_enabled_flag for when it is not present. It is asserted that the inference rule is necessary for parsing picture header and slice header when sps_alf_enabled_flag is equal to 1 and ChromaArrayType is equal to 0.

# Proposal

A missing inference rule is proposed for sps_ccalf_enabled_flag for when it is not present.

The proposed changes are shown highlighted below compared to VVC WD8 [1].

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| sps_seq_parameter_set_id | u(4) |
| … |  |
| sps_sao_enabled_flag | u(1) |
| sps_alf_enabled_flag | u(1) |
| if( sps_alf_enabled_flag && ChromaArrayType != 0 ) |  |
| sps_ccalf_enabled_flag | u(1) |
| sps_transform_skip_enabled_flag | u(1) |
| … |  |
| } |  |

sps_ccalf_enabled_flag equal to 0 specifies that the cross-component adaptive loop filter is disabled. sps_ccalf_enabled_flag equal to 1 specifies that the cross-component adaptive loop filter may be enabled. When not present sps_ccalf_enabled_flag is inferred to be equal to 0.

# References

[1] JVET-Q2001, Versatile Video Coding (Draft 8), Brussels, January 2020

# Patent rights declaration(s)

Sharp Labs of America, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).