| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0106-v1 |
| --- | --- |

| Title: | On Deblocking Control |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sachin Deshpande Jonatan Samuelsson Andrew Segall Tianyang Zhou Tomohiro Ikai | Tel: Email: | +1 360 817 8486 sdeshpande@sharplabs.com |
| Source: | Sharp Labs of America, Inc., Sharp Corporation |  |  |

_____________________________

# Abstract

Following is proposed related to the deblocking control signaling:

- Proposal 1: It is proposed to move the signalling location of syntax element dbf_info_in_ph_flag to locate it near the other deblocking control parameters signalling.

- Proposal 2: It is proposed to conditionally signal chroma related deblocking filter control parameters only when chroma array type is not equal to 0.

# Introduction

VVC WD [1] includes deblocking control signalling. This includes signalling in PPS, and optionally in picture header or slice header.

Following is proposed related to the deblocking control signaling:

- Proposal 1: It is proposed to move the signalling location of syntax element dbf_info_in_ph_flag to locate it near the other deblocking control parameters signalling. It is asserted that this can also avoid a conditional signalling (if) check (when deblocking_filter_control_present_flag is equal to 0).

- Proposal 2: It is proposed to conditionally signal chroma related deblocking filter control parameters only when chroma array type is not equal to 0. When not signalled the value for those syntax elements is inferred.

# Proposals

## Proposal 1

It is proposed to move the signalling location of syntax element dbf_info_in_ph_flag to locate it together with the other deblocking control parameters signalling. It is asserted that this can also avoid a conditional signalling (if) check (when deblocking_filter_control_present_flag is equal to 0) .

The proposed changes are shown highlighted below compared to VVC WD.

#### Picture parameter set RBSP syntax

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| pps_pic_parameter_set_id | ue(v) |
| … | u(4) |
| deblocking_filter_control_present_flag | u(1) |
| if( deblocking_filter_control_present_flag ) { |  |
| deblocking_filter_override_enabled_flag | u(1) |
| pps_deblocking_filter_disabled_flag | u(1) |
| if( deblocking_filter_override_enabled_flag ) |  |
| dbf_info_in_ph_flag | u(1) |
| if( !pps_deblocking_filter_disabled_flag ) { |  |
| pps_beta_offset_div2 | se(v) |
| pps_tc_offset_div2 | se(v) |
| pps_cb_beta_offset_div2 | se(v) |
| pps_cb_tc_offset_div2 | se(v) |
| pps_cr_beta_offset_div2 | se(v) |
| pps_cr_tc_offset_div2 | se(v) |
| } |  |
| } |  |
| rpl_info_in_ph_flag | u(1) |
| if( deblocking_filter_override_enabled_flag ) |  |
| dbf_info_in_ph_flag | u(1) |
| … |  |
| } |  |

## Proposal 2

It is proposed to conditionally signal chroma related deblocking filter control parameters only when chroma array type is not equal to 0. When not signalled the value for those syntax elements is inferred.

The proposed changes are shown highlighted below compared to VVC WD.

For the proposed modification in PPS two options are listed.

Option 1: Uses one flag in PPS to control all chroma related parameters

Option 2: Uses a separate flag to control only chroma related deblocking filter parameters

Option 1:

Uses one flag in PPS to control all chroma related parameters

#### 7.3.2.4 Picture parameter set RBSP syntax

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| pps_pic_parameter_set_id | ue(v) |
| pps_seq_parameter_set_id | u(4) |
| pps_chroma_params_present_flag | u(1) |
| … | se(v) |
| cu_qp_delta_enabled_flag | u(1) |
| pps_chroma_tool_offsets_present_flag | u(1) |
| if( pps_chroma_tool_offsets_present_flag pps_chroma_params_present_flag ) { |  |
| pps_cb_qp_offset | se(v) |
| pps_cr_qp_offset | se(v) |
| pps_joint_cbcr_qp_offset_present_flag | u(1) |
| if( pps_joint_cbcr_qp_offset_present_flag ) |  |
| pps_joint_cbcr_qp_offset_value | se(v) |
| pps_slice_chroma_qp_offsets_present_flag | u(1) |
| pps_cu_chroma_qp_offset_list_enabled_flag | u(1) |
| } |  |
| if( pps_cu_chroma_qp_offset_list_enabled_flag ) { |  |
| chroma_qp_offset_list_len_minus1 | ue(v) |
| for( i = 0; i <= chroma_qp_offset_list_len_minus1; i++ ) { |  |
| cb_qp_offset_list[ i ] | se(v) |
| cr_qp_offset_list[ i ] | se(v) |
| if( pps_joint_cbcr_qp_offset_present_flag ) |  |
| joint_cbcr_qp_offset_list[ i ] | se(v) |
| } |  |
| } |  |
| pps_weighted_pred_flag | u(1) |
| pps_weighted_bipred_flag | u(1) |
| deblocking_filter_control_present_flag | u(1) |
| if( deblocking_filter_control_present_flag ) { |  |
| deblocking_filter_override_enabled_flag | u(1) |
| pps_deblocking_filter_disabled_flag | u(1) |
| if( !pps_deblocking_filter_disabled_flag ) { |  |
| pps_beta_offset_div2 | se(v) |
| pps_tc_offset_div2 | se(v) |
| if( pps_chroma_params_present_flag ) { |  |
| pps_cb_beta_offset_div2 | se(v) |
| pps_cb_tc_offset_div2 | se(v) |
| pps_cr_beta_offset_div2 | se(v) |
| pps_cr_tc_offset_div2 | se(v) |
| } |  |
| } |  |
| } |  |
| rpl_info_in_ph_flag | u(1) |
| if( deblocking_filter_override_enabled_flag ) |  |
| dbf_info_in_ph_flag | u(1) |
| sao_info_in_ph_flag | u(1) |
| … |  |

pps_chroma_params_present_flag equal to 1 specifies that chroma related PPS syntax elements (including pps_cb_qp_offset, pps_cr_qp_offset, pps_joint_cbcr_qp_offset_present_flag, pps_joint_cbcr_qp_offset_value , pps_slice_chroma_qp_offsets_present_flag , pps_cu_chroma_qp_offset_list_enabled_flag, pps_cb_beta_offset_div2, pps_cb_tc_offset_div2, pps_cr_beta_offset_div2, pps_cr_tc_offset_div2) are present in the PPS RBSP syntax structure. pps_chroma_params_present_flag equal to 0 specifies that chroma related PPS syntax elements (including pps_cb_qp_offset, pps_cr_qp_offset, pps_jointcbcr_qp_offset_present_flag, pps_joint_cbcr_qp_offset_value , pps_slice_chroma_qp_offsets_present_flag , pps_cu_chroma_qp_offset_list_enabled_flag, pps_cb_beta_offset_div2, pps_cb_tc_offset_div2, pps_cr_beta_offset_div2, pps_cr_tc_offset_div2) are not present in the PPS RBSP syntax structure. When ChromaArrayType is equal to 0, the value of pps_chroma_params_present_flag shall be equal to 0.

pps_chroma_tool_offsets_present_flag equal to 1 specifies that chroma tool offsets related syntax elements are present in the PPS RBSP syntax structure. pps_chroma_tool_offsets_present_flag equal to 0 specifies that chroma tool offsets related syntax elements are not present in in the PPS RBSP syntax structure. When ChromaArrayType is equal to 0, the value of pps_chroma_tool_offsets_present_flag shall be equal to 0.

Option 2:

Alternatively, a separate flag (pps_chroma_dbf_params_present_flag ) to control the presence of chroma deblocking filter parameters may be included in PPS. Only when this flag is equal to 1, syntax elements pps_cb_beta_offset_div2, pps_cb_tc_offset_div2, pps_cr_beta_offset_div2, pps_cr_tc_offset_div2 are included and not otherwise.

#### Picture parameter set RBSP syntax

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| pps_pic_parameter_set_id | ue(v) |
| … | u(4) |
| deblocking_filter_control_present_flag | u(1) |
| if( deblocking_filter_control_present_flag ) { |  |
| deblocking_filter_override_enabled_flag | u(1) |
| if( !deblocking_filter_override_enabled_flag ) { |  |
| pps_deblocking_filter_disabled_flag | u(1) |
| if( !pps_deblocking_filter_disabled_flag ) { |  |
| pps_beta_offset_div2 | se(v) |
| pps_tc_offset_div2 | se(v) |
| pps_chroma_dbf_params_present_flag | u(1) |
| if(pps_chroma_dbf_params_present_flag ) { |  |
| pps_cb_beta_offset_div2 | se(v) |
| pps_cb_tc_offset_div2 | se(v) |
| pps_cr_beta_offset_div2 | se(v) |
| pps_cr_tc_offset_div2 | se(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| rpl_info_in_ph_flag | u(1) |
| … |  |
| } |  |

pps_chroma_dbf_params_present_flag equal to 1 specifies that syntax elements pps_cb_beta_offset_div2, pps_cb_tc_offset_div2, pps_cr_beta_offset_div2, pps_cr_tc_offset_div2 are present in the PPS RBSP syntax structure. pps_chroma_dbf_params_present_flag equal to 0 specifies that syntax elements pps_cb_beta_offset_div2, pps_cb_tc_offset_div2, pps_cr_beta_offset_div2, pps_cr_tc_offset_div2 are not present in the PPS RBSP syntax structure. When ChromaArrayType is equal to 0, the value of pps_chroma_dbf_params_present_flag shall be equal to 0.

Common for Option 1 and Option 2:

The following modifications in picture header and slice header apply to both option 1 and option 2.

#### 7.3.2.7 Picture header structure syntax

| picture_header_structure( ) { | Descriptor |
| --- | --- |
| gdr_or_irap_pic_flag | u(1) |
| … |  |
| if( deblocking_filter_override_enabled_flag && dbf_info_in_ph_flag ) { |  |
| ph_deblocking_filter_override_flag | u(1) |
| if( ph_deblocking_filter_override_flag ) { |  |
| ph_deblocking_filter_disabled_flag | u(1) |
| if( !ph_deblocking_filter_disabled_flag ) { |  |
| ph_beta_offset_div2 | se(v) |
| ph_tc_offset_div2 | se(v) |
| if( ChromaArrayType != 0 ) { |  |
| ph_cb_beta_offset_div2 | se(v) |
| ph_cb_tc_offset_div2 | se(v) |
| ph_cr_beta_offset_div2 | se(v) |
| ph_cr_tc_offset_div2 | se(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| … |  |
| } |  |

ph_beta_offset_div2 and ph_tc_offset_div2 specify the deblocking parameter offsets for β and tC (divided by 2) that are applied to the luma component for the slices associated with the PH. The values of ph_beta_offset_div2 and ph_tc_offset_div2 shall both be in the range of −12 to 12, inclusive. When not present, the values of ph_beta_offset_div2 and ph_tc_offset_div2 are inferred to be equal to pps_beta_offset_div2 and pps_tc_offset_div2, respectively.

ph_cb_beta_offset_div2 and ph_cb_tc_offset_div2 specify the deblocking parameter offsets for β and tC (divided by 2) that are applied to the Cb component for the slices associated with the PH. The values of ph_cb_beta_offset_div2 and ph_cb_tc_offset_div2 shall both be in the range of −12 to 12, inclusive. When not present, the values of ph_cb_beta_offset_div2 and ph_cb_tc_offset_div2 are inferred to be equal to pps_cb_beta_offset_div2 and pps_cb_tc_offset_div2, respectively.

ph_cr_beta_offset_div2 and ph_cr_tc_offset_div2 specify the deblocking parameter offsets for β and tC (divided by 2) that are applied to the Cr component for the slices associated with the PH. The values of ph_cr_beta_offset_div2 and ph_cr_tc_offset_div2 shall both be in the range of −12 to 12, inclusive. When not present, the values of ph_cr_beta_offset_div2 and ph_cr_tc_offset_div2 are inferred to be equal to pps_cr_beta_offset_div2 and pps_cr_tc_offset_div2, respectively.

### 7.3.7 Slice header syntax

#### 7.3.7.1 General slice header syntax

| slice_header( ) { | Descriptor |
| --- | --- |
| picture_header_in_slice_header_flag | u(1) |
| … |  |
| if( deblocking_filter_override_enabled_flag && !dbf_info_in_ph_flag ) |  |
| slice_deblocking_filter_override_flag | u(1) |
| if( slice_deblocking_filter_override_flag ) { |  |
| slice_deblocking_filter_disabled_flag | u(1) |
| if( !slice_deblocking_filter_disabled_flag ) { |  |
| slice_beta_offset_div2 | se(v) |
| slice_tc_offset_div2 | se(v) |
| if( ChromaArrayType != 0 ) { |  |
| slice_cb_beta_offset_div2 | se(v) |
| slice_cb_tc_offset_div2 | se(v) |
| slice_cr_beta_offset_div2 | se(v) |
| slice_cr_tc_offset_div2 | se(v) |
| } |  |
| } |  |
| } |  |
| … |  |
| } |  |

slice_beta_offset_div2 and slice_tc_offset_div2 specify the deblocking parameter offsets for β and tC (divided by 2) that are applied to the luma component for the current slice. The values of slice_beta_offset_div2 and slice_tc_offset_div2 shall both be in the range of −12 to 12, inclusive. When not present, the values of slice_beta_offset_div2 and slice_tc_offset_div2 are inferred to be equal to ph_beta_offset_div2 and ph_tc_offset_div2, respectively.

slice_cb_beta_offset_div2 and slice_cb_tc_offset_div2 specify the deblocking parameter offsets for β and tC (divided by 2) that are applied to the Cb component for the current slice. The values of slice_cb_beta_offset_div2 and slice_cb_tc_offset_div2 shall both be in the range of −12 to 12, inclusive. When not present, the values of slice_cb_beta_offset_div2 and slice_cb_tc_offset_div2 are inferred to be equal to ph_cb_beta_offset_div2 and ph_cb_tc_offset_div2, respectively.

slice_cb_beta_offset_div2 and slice_cb_tc_offset_div2 specify the deblocking parameter offsets for β and tC (divided by 2) that are applied to the Cr component for the current slice. The values of slice_cr_beta_offset_div2 and slice_cr_tc_offset_div2 shall both be in the range of −12 to 12, inclusive. When not present, the values of slice_cr_beta_offset_div2 and slice_cr_tc_offset_div2 are inferred to be equal to ph_cr_beta_offset_div2 and ph_cr_tc_offset_div2, respectively.

# Conclusion

This document proposes modifications related to the deblocking control signalling in VVC. It is proposed to adopt the proposals in the next version of VVC draft.

# Patent rights declaration(s)

Sharp Labs of America, Inc., may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Sharp Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

[1] JVET-Q2001-vE, Versatile Video Coding (Draft 8), Brussels, January 2020