| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0107-v1 |
| --- | --- |

| Title: | AHG8/ AHG9: On Temporal Sublayers Information |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sachin Deshpande Jonatan Samuelsson Andrew Segall Philip Cowan | Tel: Email: | +1 360 817 8486 sdeshpande@sharplabs.com |
| Source: | Sharp Labs of America, Inc. |  |  |

_____________________________

# Abstract

This contribution contains modifications related to the temporal sublayers information signalling in VPS and sub-bitstream extraction. Following is proposed:

- Proposal 1: It is proposed to signal the syntax elements max_tid_ref_present_flag[ i ], max_tid_il_ref_pics_plus1[ i ] only when ols_mode_idc is not equal to 1 and each_layer_is_an_ols_flag is not equal to 1.

- Proposal 2: It is proposed that the if condition for signalling ptl_max_temporal_id[ i ], dpb_max_temporal_id[ i ], and hrd_max_tid[ i ] only use the flag vps_all_layers_same_num_sublayers_flag instead of using the flag vps_all_layers_same_num_sublayers_flag and vps_max_sublayers_minus1 syntax element. Also modified inference rules for ptl_max_temporal_id[ i ], dpb_max_temporal_id[ i ], and hrd_max_tid[ i ], when not present are proposed.

- Proposal 3: Modifications are proposed to the semantics of max_tid_il_ref_pics_plus1[ i ] and to the sub-bitstream extraction process to account for GDR pictures.

# Introduction

In VVC WD8 [1] Video Parameter Set (VPS) includes signalling of the syntax elements max_tid_ref_present_flag[ I ], max_tid_il_ref_pics_plus1[ I ].

Following is proposed:

- Proposal 1: It is proposed to signal the syntax elements max_tid_ref_present_flag[ I ], max_tid_il_ref_pics_plus1[ I ] only when ols_mode_idc is not equal to 1 and each_layer_is_an_ols_flag is not equal to 1.

- Proposal 2: It is proposed that the if condition for signalling ptl_max_temporal_id[ I ], dpb_max_temporal_id[ I ], and hrd_max_tid[ I ] only use the flag vps_all_layers_same_num_sublayers_flag instead of using the flag vps_all_layers_same_num_sublayers_flag and vps_max_sublayers_minus1 syntax element. Also modified inference rules for ptl_max_temporal_id[ I ], dpb_max_temporal_id[ I ], and hrd_max_tid[ I ], when not present are proposed.

- Proposal 3: Modifications are proposed to the semantics of max_tid_il_ref_pics_plus1[ I ] and to the sub-bitstream extraction process to account for GDR pictures.

# Proposal

## Proposal 1

It is asserted that in VVC WD8, the syntax elements max_tid_il_ref_pics_plus1[ i ] are used only for the derivation of NumSubLayersInLayerInOLS[ i ][ j ], specifying the number of sublayers in the j-th layer in the i-th OLS. And that NumSubLayersInLayerInOLS[ targetOlsIdx ][ j ], is used for the sub-bitstream extraction for a target OLS index, targetOlsIdx.

ols_mode_idc equal to 1 specifies that the total number of OLSs specified by the VPS is equal to vps_max_layers_minus1 + 1, the i-th OLS includes the layers with layer indices from 0 to i, inclusive, and for each OLS all layers in the OLS are output. each_layer_is_an_ols_flag equal to 1 specifies that each OLS contains only one layer and each layer itself in a CVS referring to the VPS is an OLS with the single included layer being the only output layer.

It is asserted that when ols_mode_idc is equal to 1 or when each_layer_is_an_ols_flag is equal to 1, the number of sublayers in the j-th layer in the i-th OLS , i.e. NumSubLayersInLayerInOLS[ i ][ j ],  can be derived without needing the syntax elements max_tid_il_ref_pics_plus1[ i ].

Thus it is proposed to not signal the syntax elements max_tid_ref_present_flag[ i ], max_tid_il_ref_pics_plus1[ i ] when either of the following is true:

- ols_mode_idc is equal to 1

- each_layer_is_an_ols_flag is equal to 1.

The proposed changes are shown highlighted below compared to VVC WD. The syntax elements moved are shown highlighted.

#### Video parameter set RBSP syntax

| video_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| vps_video_parameter_set_id | u(4) |
| vps_max_layers_minus1 | u(6) |
| vps_max_sublayers_minus1 | u(3) |
| if( vps_max_layers_minus1 > 0 && vps_max_sublayers_minus1 > 0 ) |  |
| vps_all_layers_same_num_sublayers_flag | u(1) |
| if( vps_max_layers_minus1 > 0 ) |  |
| vps_all_independent_layers_flag | u(1) |
| for( i = 0; i <= vps_max_layers_minus1; i++ ) { |  |
| vps_layer_id[ i ] | u(6) |
| if( i > 0 && !vps_all_independent_layers_flag ) { |  |
| vps_independent_layer_flag[ i ] | u(1) |
| if( !vps_independent_layer_flag[ i ] ) { |  |
| for( j = 0; j < i; j++ ) |  |
| vps_direct_ref_layer_flag[ i ][ j ] | u(1) |
| max_tid_ref_present_flag[ i ] | u(1) |
| if( max_tid_ref_present_flag[ i ] ) |  |
| max_tid_il_ref_pics_plus1[ i ] | u(3) |
| } |  |
| } |  |
| } |  |
| if( vps_max_layers_minus1 > 0 ) { |  |
| if( vps_all_independent_layers_flag ) |  |
| each_layer_is_an_ols_flag | u(1) |
| if( !each_layer_is_an_ols_flag ) { |  |
| if( !vps_all_independent_layers_flag ) |  |
| ols_mode_idc | u(2) |
| if( ols_mode_idc = = 2 ) { |  |
| num_output_layer_sets_minus1 | u(8) |
| for( i = 1; i <= num_output_layer_sets_minus1; i ++) |  |
| for( j = 0; j <= vps_max_layers_minus1; j++ ) |  |
| ols_output_layer_flag[ i ][ j ] | u(1) |
| } |  |
| } |  |
| } |  |
| for( i = 1; i <= vps_max_layers_minus1; i++ ) { |  |
| if( ols_mode_idc != 1 && !each_layer_is_an_ols_flag && !vps_independent_layer_flag[ i ] ) { |  |
| max_tid_ref_present_flag[ i ] | u(1) |
| if( max_tid_ref_present_flag[ i ] ) |  |
| max_tid_il_ref_pics_plus1[ i ] | u(3) |
| } |  |
| } |  |

## Proposal 2

Following is proposed as asserted simplifications:

- Condition for signalling ptl_max_temporal_id[ i ], dpb_max_temporal_id[ i ], and hrd_max_tid[ i ], is simplified to only use the flag vps_all_layers_same_num_sublayers_flag instead of using the flag vps_all_layers_same_num_sublayers_flag and vps_max_sublayers_minus1 syntax element.

- Simplified inference rules for ptl_max_temporal_id[ i ], dpb_max_temporal_id[ i ], and hrd_max_tid[ i ], when not present are proposed.

This is because when not present, the value of vps_all_layers_same_num_sublayers_flag is inferred to be equal to 1.

The proposed changes are shown highlighted below compared to VVC WD.

#### Video parameter set RBSP syntax

| video_parameter_set_rbsp( ) { | Descriptor |  |
| --- | --- | --- |
| vps_video_parameter_set_id | u(4) |  |
| vps_max_layers_minus1 | u(6) |  |
| vps_max_sublayers_minus1 | u(3) |  |
| if( vps_max_layers_minus1 > 0 && vps_max_sublayers_minus1 > 0 ) |  |  |
| vps_all_layers_same_num_sublayers_flag | u(1) |  |
| if( vps_max_layers_minus1 > 0 ) |  |  |
| vps_all_independent_layers_flag | u(1) |  |
| for( i = 0; i <= vps_max_layers_minus1; i++ ) { |  |  |
| vps_layer_id[ i ] | u(6) |  |
| if( i > 0 && !vps_all_independent_layers_flag ) { |  |  |
| vps_independent_layer_flag[ i ] | u(1) |  |
| if( !vps_independent_layer_flag[ i ] ) { |  |  |
| for( j = 0; j < i; j++ ) |  |  |
| vps_direct_ref_layer_flag[ i ][ j ] | u(1) |  |
| max_tid_ref_present_flag[ i ] | u(1) |  |
| if( max_tid_ref_present_flag[ i ] ) |  |  |
| max_tid_il_ref_pics_plus1[ i ] | u(3) |  |
| } |  |  |
| } |  |  |
| } |  |  |
| if( vps_max_layers_minus1 > 0 ) { |  |  |
| if( vps_all_independent_layers_flag ) |  |  |
| each_layer_is_an_ols_flag | u(1) |  |
| if( !each_layer_is_an_ols_flag ) { |  |  |
| if( !vps_all_independent_layers_flag ) |  |  |
| ols_mode_idc | u(2) |  |
| if( ols_mode_idc = = 2 ) { |  |  |
| num_output_layer_sets_minus1 | u(8) |  |
| for( i = 1; i <= num_output_layer_sets_minus1; i ++) |  |  |
| for( j = 0; j <= vps_max_layers_minus1; j++ ) |  |  |
| ols_output_layer_flag[ i ][ j ] | u(1) |  |
| } |  |  |
| } |  |  |
| } |  |  |
| vps_num_ptls_minus1 | u(8) |
| for( i = 0; i <= vps_num_ptls_minus1; i++ ) { |  |  |
| if( i > 0 ) |  |  |
| pt_present_flag[ i ] | u(1) |  |
| if( vps_max_sublayers_minus1 > 0 && !vps_all_layers_same_num_sublayers_flag) |  |  |
| ptl_max_temporal_id[ i ] | u(3) |  |
| } |  |  |
| while( !byte_aligned( ) ) |  |  |
| vps_ptl_alignment_zero_bit /* equal to 0 */ | f(1) |  |
| for( i = 0; i <= vps_num_ptls_minus1; i++ ) |  |  |
| profile_tier_level( pt_present_flag[ i ], ptl_max_temporal_id[ i ] ) |  |
| for( i = 0; i < TotalNumOlss; i++ ) |  |  |
| if( vps_num_ptls_minus1 > 0 ) |  |  |
| ols_ptl_idx[ i ] | u(8) |  |
| if( !vps_all_independent_layers_flag ) |  |  |
| vps_num_dpb_params | ue(v) |
| if( vps_num_dpb_params > 0 && vps_max_sublayers_minus1 > 0 ) |  |  |
| vps_sublayer_dpb_params_present_flag | u(1) |  |
| for( i = 0; i < vps_num_dpb_params; i++ ) { |  |  |
| if( vps_max_sublayers_minus1 > 0 && !vps_all_layers_same_num_sublayers_flag) |  |  |
| dpb_max_temporal_id[ i ] | u(3) |  |
| dpb_parameters( dpb_max_temporal_id[ i ], vps_sublayer_dpb_params_present_flag ) |  |
| } |  |
| for( i = 0; i < TotalNumOlss; i++ ) { |  |  |
| if( NumLayersInOls[ i ] > 1 ) { |  |  |
| ols_dpb_pic_width[ i ] | ue(v) |  |
| ols_dpb_pic_height[ i ] | ue(v) |  |
| if( vps_num_dpb_params > 1 ) |  |  |
| ols_dpb_params_idx[ i ] | ue(v) |  |
| } |  |  |
| } |  |  |
| if( !each_layer_is_an_ols_flag ) |  |  |
| vps_general_hrd_params_present_flag | u(1) |  |
| if( vps_general_hrd_params_present_flag ) { |  |  |
| general_hrd_parameters( ) |  |  |
| if( vps_max_sublayers_minus1 > 0 ) |  |  |
| vps_sublayer_cpb_params_present_flag | u(1) |  |
| num_ols_hrd_params_minus1 | ue(v) |  |
| for( i = 0; i <= num_ols_hrd_params_minus1; i++ ) { |  |  |
| if( vps_max_sublayers_minus1 > 0 && !vps_all_layers_same_num_sublayers_flag) |  |  |
| hrd_max_tid[ i ] | u(3) |  |
| firstSubLayer = vps_sublayer_cpb_params_present_flag ? 0 : hrd_max_tid[ i ] |  |  |
| ols_hrd_parameters( firstSubLayer, hrd_max_tid[ i ] ) |  |  |
| } |  |  |
| if( num_ols_hrd_params_minus1 + 1 != TotalNumOlss && num_ols_hrd_params_minus1 > 0 ) |  |  |
| for( i = 1; i < TotalNumOlss; i++ ) |  |  |
| if( NumLayersInOls[ i ] > 1 ) |  |  |
| ols_hrd_idx[ i ] | ue(v) |  |
| } |  |  |
| vps_extension_flag | u(1) |  |
| if( vps_extension_flag ) |  |  |
| while( more_rbsp_data( ) ) |  |  |
| vps_extension_data_flag | u(1) |  |
| rbsp_trailing_bits( ) |  |  |
| } |  |  |

…

ptl_max_temporal_id[ i ] specifies the TemporalId of the highest sublayer representation for which the level information is present in the i-th profile_tier_level( ) syntax structure in the VPS. The value of ptl_max_temporal_id[ i ] shall be in the range of 0 to vps_max_sublayers_minus1, inclusive. When vps_max_sublayers_minus1 is equal to 0, the value of ptl_max_temporal_id[ i ] is inferred to be equal to 0. When vps_max_sublayers_minus1 is greater than 0 and vps_all_layers_same_num_sublayers_flag is equal to 1, the value of ptl_max_temporal_id[ i ] is inferred to be equal to vps_max_sublayers_minus1. When not present ptl_max_temporal_id[ i ] is inferred to be equal to vps_max_sublayers_minus1.

dpb_max_temporal_id[ i ] specifies the TemporalId of the highest sublayer representation for which the DPB parameters may be present in the i-th dpb_parameters( ) syntax strutcure in the VPS. The value of dpb_max_temporal_id[ i ] shall be in the range of 0 to vps_max_sublayers_minus1, inclusive. When vps_max_sublayers_minus1 is equal to 0, the value of dpb_max_temporal_id[ i ] is inferred to be equal to 0. When vps_max_sublayers_minus1 is greater than 0 and vps_all_layers_same_num_sublayers_flag is equal to 1, the value of dpb_max_temporal_id[ i ] is inferred to be equal to vps_max_sublayers_minus1. When not present dpb_max_temporal_id[ i ] is inferred to be equal to vps_max_sublayers_minus1.

hrd_max_tid[ i ] specifies the TemporalId of the highest sublayer representation for which the HRD parameters are contained in the i-th ols_hrd_parameters( ) syntax structure. The value of hrd_max_tid[ i ] shall be in the range of 0 to vps_max_sublayers_minus1, inclusive. When vps_max_sublayers_minus1 is equal to 0, the value of hrd_max_tid[ i ] is inferred to be equal to 0. When vps_max_sublayers_minus1 is greater than 0 and vps_all_layers_same_num_sublayers_flag is equal to 1, the value of hrd_max_tid[ i ] is inferred to be equal to vps_max_sublayers_minus1. When not present hrd_max_tid[ i ] is inferred to be equal to vps_max_sublayers_minus1.

## Proposal 3

In VVC WD8 GDR pictures can start a CLVS. It is asserted that it is possible to have bitstreams which do not have any IRAP pictures and only have GDR pictures or GDR pictures with recovery_poc_cnt equal to 0.

Modification is proposed to max_tid_il_ref_pics_plus1 semantics and to sub-bitstream extraction process to allow GDR pictures to be retained correctly in this case.

max_tid_il_ref_pics_plus1[ i ] equal to 0 specifies that inter-layer prediction is not used by non-IRAP pictures and non-GDR pictures with recovery_poc_cnt equal to 0 of the i-th layer. max_tid_il_ref_pics_plus1[ i ] greater than 0 specifies that, for decoding pictures of the i-th layer, no picture with TemporalId greater than max_tid_il_ref_pics_plus1[ i ] − 1 is used as ILRP. When not present, the value of max_tid_il_ref_pics_plus1[ i ] is inferred to be equal to 7.

Also it is asserted that in general (irrespective of this proposal) the first setence above is not clear and the semantics above are better expressed as shown below:

max_tid_il_ref_pics_plus1[ i ] equal to 0 specifies that inter-layer prediction is not used by non-IRAP pictures and non-GDR pictures with recovery_poc_cnt equal to 0 are not used as ILRP for decoding pictures  of the i-th layer. max_tid_il_ref_pics_plus1[ i ] greater than 0 specifies that, for decoding pictures of the i-th layer, no picture with TemporalId greater than max_tid_il_ref_pics_plus1[ i ] − 1 is used as ILRP. When not present, the value of max_tid_il_ref_pics_plus1[ i ] is inferred to be equal to 7.

C.6 Sub-bitstream extraction process

Inputs to this process are a bitstream inBitstream, a target OLS index targetOlsIdx, and a target highest TemporalId value tIdTarget.

Output of this process is a sub-bitstream outBitstream.

It is a requirement of bitstream conformance for the input bitstream that any output sub-bitstream that satisfies all of the following conditions shall be a conforming bitstream:

–	The output sub-bitstream is the output of the process specified in this clause with the bitstream, targetOlsIdx equal to an index to the list of OLSs specified by the VPS, and tIdTarget equal to any value in the range of 0 to 6, inclusive, as inputs.

–	The output sub-bitstream contains at least one VCL NAL unit with nuh_layer_id equal to each of the nuh_layer_id values in LayerIdInOls[ targetOlsIdx ].

–	The output sub-bitstream contains at least one VCL NAL unit with TemporalId equal to tIdTarget.

NOTE – A conforming bitstream contains one or more coded slice NAL units with TemporalId equal to 0, but does not have to contain coded slice NAL units with nuh_layer_id equal to 0.

The output sub-bitstream OutBitstream is derived as follows:

–	The bitstream outBitstream is set to be identical to the bitstream inBitstream.

–	Remove from outBitstream all NAL units with TemporalId greater than tIdTarget.

–	Remove from outBitstream all NAL units with nal_unit_type not equal to any of VPS_NUT, DCI_NUT, and EOB_NUT and with nuh_layer_id not included in the list LayerIdInOls[ targetOlsIdx ].

–	Remove from outBitstream all NAL units for which all of the following conditions are true:

–	nal_unit_type is not equal to IDR_W_RADL, IDR_N_LP, or CRA_NUT or GDR_NUT with associated recovery_poc_cnt equal to 0.

–	nuh_layer_id is equal to LayerIdInOls[ targetOlsIdx ][ j ] for a value of j in the range of 0 to NumLayersInOls[ targetOlsIdx ] − 1 inclusive.

–	TemporalId is greater than or equal to NumSubLayersInLayerInOLS[ targetOlsIdx ][ j ].

–	Remove from outBitstream all SEI NAL units that contain a scalable nesting SEI message that has nesting_ols_flag equal to 1 and there is no value of i in the range of 0 to nesting_num_olss_minus1, inclusive, such that NestingOlsIdx[ i ] is equal to targetOlsIdx.

–	When LayerIdInOls[ targetOlsIdx ] does not include all values of nuh_layer_id in all NAL units in the bitstream, the following applies:

–	Remove from outBitstream all SEI NAL units that contain a non-scalable-nested SEI message with payloadType equal to 0 (buffering period) or 130 (decoding unit information).

–	When general_same_pic_timing_in_all_ols_flag is equal to 0, remove from outBitstream all SEI NAL units that contain a non-scalable-nested SEI message with payloadType equal to 1 (picture timing).

–	When outBitstream contains SEI NAL units that contain a scalable nesting SEI message with nesting_ols_flag equal to 1 and are applicable to outBitstream (NestingOlsIdx[ i ] is equal to targetOlsIdx), the following applies:

–	If same_pic_timing_within_ols_flag is equal to 0, extract appropriate non-scalable-nested SEI message with payloadType equal to 0 (buffering period), 1 (picture timing), or 130 (decoding unit information) from the scalable nesting SEI message and include those SEI messages in outBitstream.

–	Otherwise (same_pic_timing_within_ols_flag is equal to 1), extract appropriate non-scalable-nested SEI message with payloadType equal to 0 (buffering period) or 130 (decoding unit information) from the scalable nesting SEI message and include those SEI messages in outBitstream.

# Conclusion

This document proposes modifications to temporal sublayers signalling related signalling in VPS and sub-bitstream extraction. It is proposed to adopt the proposals in the next version of VVC draft.

# Patent rights declaration(s)

Sharp Labs of America, Inc., may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

[1] JVET-Q2001-vE, Versatile Video Coding (Draft 8), Brussels, January 2020