| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0108-v1 |
| --- | --- |

| Title: | AHG9: On Decoding Capability Information and PTL Signalling |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sachin Deshpande Jonatan Samuelsson Andrew Segall Philip Cowan | Tel: Email: | +1 360 817 8486 sdeshpande@sharplabs.com |
| Source: | Sharp Labs of America, Inc. |  |  |

_____________________________

# Abstract

Following is proposed related to DCI and PTL signalling:

- Proposal 1: It is proposed that dci_max_sublayers_minus1 syntax element be removed and instead those bits and the reserved zero bit be used for the syntax element dci_num_ptls_minus1.

- Proposal 2: It is proposed to rearrange the syntax elements in profile-tier-level signalling structure such that general_level_idc syntax element which is unconditionally signalled is first in the structure and the other conditional signalling which is all based on profileTierPresentFlag is together,  thus requiring only a single if check.

- Proposal 3: It is proposed to conditionally signal sps_ptl_dpb_hrd_params_present_flag only when sps_video_parameter_set_id is not equal to 0.

# Introduction

VVC WD [1] includes a Decoding Capability Information (DCI) NAL Unit which includes signalling of profile-tier-level information.

Following is proposed related to DCI and PTL signalling:

- Proposal 1: It is proposed that dci_max_sublayers_minus1 syntax element be removed and instead those bits and the reserved zero bit be used for the syntax element dci_num_ptls_minus1.  Also a semantic constraint is proposed for the syntax element dci_num_ptls_minus1.

- Proposal 2: It is proposed to rearrange the syntax elements in profile-tier-level signalling structure such that general_level_idc syntax element which is unconditionally signalled is first in the structure and the other conditional signalling which is all based on profileTierPresentFlag is together,  thus requiring only a single if check.

- Proposal 3: It is proposed to conditionally signal sps_ptl_dpb_hrd_params_present_flag only when sps_video_parameter_set_id is not equal to 0.

# Proposals

## Proposal 1

It is proposed that dci_max_sublayers_minus1 syntax element be removed and instead those bits and the reserved zero bit should be used for the syntax element dci_num_ptls_minus1.

The motivation for this change is the following:

- It is asserted that previously the syntax element dci_max_sublayers_minus1 was used as a second input argument to profile_tier_level syntax structure present in decoding_capability_information_rbsp( ). However since the profile_tier_level structure in DCI currently does not specify information about sublayers, thus this syntax element is not used and not useful.

- VPS allows signalling  vps_num_ptls_minus1 +1  number of profile-tier-level structures, where vps_num_ptls_minus1 is u(8) coded. It is proposed to align the signalling of dci_num_ptls_minus1 with this, even though typically fewer PTL structures will be signalled in DCI than in VPS.

The proposed changes are shown highlighted compared with VVC WD.

#### Decoding capability information RBSP syntax

| decoding_capability_information_rbsp( ) { | Descriptor |
| --- | --- |
| dci_max_sublayers_minus1 | u(3) |
| dci_reserved_zero_bit | u(1) |
| dci_num_ptls_minus1 | u(4)u(8) |
| for( i = 0; i <= dci_num_ptls_minus1; i++ ) |  |
| profile_tier_level( 1, 0 ) |  |
| dci_extension_flag | u(1) |
| if( dci_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| dci_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

dci_num_ptls_minus1 plus 1 specifies the number of profile_tier_level( ) syntax structures in the DCI NAL unit. The value of dci_num_ptls_minus1 shall be in the range of 0 to vps_num_ptls_minus1, inclusive.

It is a requirement of bitstream conformance that each OLS in a CVS in the bitstream shall conforms to at least one of the profile_tier_level( ) syntax structures in the DCI NAL unit.

NOTE 2 – The DCI NAL unit may include PTL information, possibly carried in multiple profile_tier_level( ) syntax structures, that applies collectively to multiple OLSs, and does not need to include PTL information for each of the OLSs individually.

Alternatively, it is proposed to remove the syntax element dci_max_sublayers_minus1 and use some those bits as reserved zero bits and allocate some bits to dci_num_ptls_minus1 (for example 3 reserved bits and 5 bits for dci_num_ptls_minus1).

## Proposal 2

It is proposed to rearrange the syntax elements in profile-tier-level signalling structure such that general_level_idc syntax element which is unconditionally signalled is first syntax element in the structure. Since profileTierPresentFlag can be equal to 0 or 1, this movement of syntax allows the unconditionally signaled syntax element general_level_idc to be at a fixed location.

Additionally with this movement of the syntax, all the syntax which is conditionally signaled based on profileTierPresentFlag is together thus requiring only a single if check.

The proposed changes are shown highlighted compared with VVC WD.

### Profile, tier, and level syntax

#### General profile, tier, and level syntax

| profile_tier_level( profileTierPresentFlag, maxNumSubLayersMinus1 ) { | Descriptor |
| --- | --- |
| general_level_idc | u(8) |
| if( profileTierPresentFlag ) { |  |
| general_profile_idc | u(7) |
| general_tier_flag | u(1) |
| general_constraint_info( ) |  |
| } |  |
| general_level_idc | u(8) |
| if( profileTierPresentFlag ) { |  |
| num_sub_profiles | u(8) |
| for( i = 0; i < num_sub_profiles; i++ ) |  |
| general_sub_profile_idc[ i ] | u(32) |
| } |  |
| for( i = 0; i < maxNumSubLayersMinus1; i++ ) |  |
| sublayer_level_present_flag[ i ] | u(1) |
| while( !byte_aligned( ) ) |  |
| ptl_alignment_zero_bit | f(1) |
| for( i = 0; i < maxNumSubLayersMinus1; i++ ) |  |
| if( sublayer_level_present_flag[ i ] ) |  |
| sublayer_level_idc[ i ] | u(8) |
| } |  |

## Proposal 3

When sps_video_parameter_set_id is equal to 0, the SPS does not refer to a VPS and no VPS is referred to when decoding each CLVS referring to the SPS. Currently in VVC WD, a flag sps_ptl_dpb_hrd_params_present_flag  is signalled and it controls the presence of the PTL, DPB, HRD parameter structures in SPS.  When sps_video_parameter_set_id is equal to 0, the PTL, DPB, HRD parameters must be signalled in SPS. Since these structures are important it is argued that the syntax should make sure that these structures are definitely included in SPS in this case.

It is proposed to conditionally signal sps_ptl_dpb_hrd_params_present_flag only when sps_video_parameter_set_id is not equal to 0. When not signalled the value of sps_ptl_dpb_hrd_params_present_flag is inferred equal to 1. This syntactically ensures that PTL, DPB and HRD parameters are included in SPS when there is no VPS.

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| sps_seq_parameter_set_id | u(4) |
| sps_video_parameter_set_id | u(4) |
| sps_max_sublayers_minus1 | u(3) |
| sps_reserved_zero_4bits | u(4) |
| if( sps_video_parameter_set_id != 0 ) |  |
| sps_ptl_dpb_hrd_params_present_flag | u(1) |
| if( sps_ptl_dpb_hrd_params_present_flag ) |  |
| profile_tier_level( 1, sps_max_sublayers_minus1 ) |  |
| … |  |

sps_ptl_dpb_hrd_params_present_flag equal to 1 specifies that a profile_tier_level( ) syntax structure and a dpb_parameters( ) syntax structure are present in the SPS, and a general_hrd_parameters( ) syntax structure and an ols_hrd_parameters( ) syntax structure may also be present in the SPS. sps_ptl_dpb_hrd_params_present_flag equal to 0 specifies that none of these four syntax structures is present in the SPS. The value of sps_ptl_dpb_hrd_params_present_flag shall be equal to vps_independent_layer_flag[ GeneralLayerIdx[ nuh_layer_id ] ]. When not present sps_ptl_dpb_hrd_params_present_flag is inferred to be equal to 1.

# Conclusion

This document proposes modifications related to the DCI and PTL signalling in VVC. It is proposed to adopt the proposals in the next version of VVC draft.

# Patent rights declaration(s)

Sharp Labs of America, Inc., may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

[1] JVET-Q2001-vE, Versatile Video Coding (Draft 8), Brussels, January 2020