| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0109-v1 |
| --- | --- |

| Title: | AHG9/AHG12: On tile, slice, and related loop filter control flags |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Lulin Chen, Chih-Wei Hsu, Chia-Ming Tsai, Olena Chubach, Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {lulin.chen, cw.hsu, chia-ming.tsai, olena.chubach, yuwen.huang, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

- Abstract

JVET-Q0120 proposed to remove loop_filter_across_slices_enabled_flag as alleged when similar control for the subpicture boundaries is in place, the flag usage is diminished. This contribution describes the usage of loop_filter_across_slices_enabled_flag to consolidate its usefulness.

Introduction

In the VVC Draft 8 [1], Fig. 4, Fig.5, and Fig. 6 are informative illustrations (see above from left to right) for CTU, tile, and slice. For control of the loop filtering operations in the decoding process, the two control flags, loop_filter_across_slices_enabled_flag and loop_filter_across_tiles_enabled_flag, are specified in the PPS as highlighted below.

| no_pic_partition_flag | u(1) |
| --- | --- |
| if( !no_pic_partition_flag ) { |  |
| pps_log2_ctu_size_minus5 | u(2) |
| … |  |
| loop_filter_across_tiles_enabled_flag | u(1) |
| loop_filter_across_slices_enabled_flag | u(1) |
| } |  |

slice: An integer number of complete tiles or an integer number of consecutive complete CTU rows within a tile of a picture that are exclusively contained in a single NAL unit.

subpicture: An rectangular region of one or more slices within a picture.

tile: A rectangular region of CTUs within a particular tile column and a particular tile row in a picture.

Discussions

From the definitions and illustrations above, there are generally three cases for tile and slice in applications:

- Tiles in raster scan slices (as shown in the left figure in Section 1),

- Tiles in rectangular slices (as shown in the middle figure in Section 1)

- Rectangular slices in tiles (as shown in the right part of the picture in the right figure in Section 1)

After a picture is partitioned as one of the cases, the tile and slice boundaries impose constraints on coding tools in the decoding process. Here are the list and comparison of those related parts for convenience.

Derivation process for neighbouring block availability (Section 6.4.4 of VVC Draft 8)

The neighbouring block availability availableN is derived as follows:

–	If one or more of the following conditions are true, availableN is set equal to FALSE:

–	xNbY is less than 0.

–	yNbY is less than 0.

–	xNbY is greater than or equal to pic_width_in_luma_samples.

–	yNbY is greater than or equal to pic_height_in_luma_samples.

–	IsAvailable[ cIdx ][ xNbY ][ yNbY ] is equal to FALSE.

–	The neighbouring block is contained in a different slice than the current block.

–	The neighbouring block is contained in a different tile than the current block.

–	sps_entropy_coding_sync_enabled_flag is equal to 1 and ( xNbY  >>  CtbLog2SizeY ) is greater than or equal to (xCurr  >>  CtbLog2SizeY ) + 1.

–	Otherwise, availableN is set equal to TRUE.

When all of the following conditions are true, availableN is set equal to FALSE:

–	checkPredModeY is equal to TRUE.

–	availableN is set equal to TRUE.

–	CuPredMode[ 0][ xNbY ][ yNbY ] is not equal to CuPredMode[ 0][ xCurr ][ yCurr ].

Processes that need to check the availability of neighbouring blocks

- Intra prediction mode:

  - Intra MPM mode list

  - Mark the availability of neighbouring reference samples

  - CCLM luma downsampling

  - Check the availability of neighbouring chroma samples in CCLM

- Inter prediction mode:

  - Spatial merge candidate list

  - Spatial MVP candidate list

  - Subblock merging candidate list in SbTMVP

  - Affine control point MVP candidate list

  - Reconstructed affine control point MVP candidate list

  - Blending weight in CIIP

  - IBC spatial BV candidates

- Predicted luma quantization parameter derivation

- Deriving chroma scale from neighboring luma samples in LMCS

Derivation process for quantization parameters (Section 8.7.1 of VVC Draft 8)

- The variable qPY_PREV is derived as follows:

–	If one or more of the following conditions are true, qPY_PREV is set equal to SliceQpY:

–	The current quantization group is the first quantization group in a slice.

–	The current quantization group is the first quantization group in a tile.

–	The current quantization group is the first quantization group in a CTB row of a tile and sps_entropy_coding_sync_enabled_flag is equal to 1.

–	Otherwise, qPY_PREV is set equal to the luma quantization parameter QpY of the last luma coding unit in the previous quantization group in decoding order.

CABAC initialization process (Section 9.3.1 of VVC Draft 8)

The initialization process as specified in clause 9.3.2 is invoked when starting the parsing of the CTU syntax specified in clause 7.3.10.2 and one or more of the following conditions are true:

- The CTU is the first CTU in a slice.

- The CTU is the first CTU in a tile.

- The value of sps_entropy_coding_sync_enabled_flag is equal to 1 and the CTU is the first CTU in a CTU row of a tile.

Comparisons of loop filter control flags for subpicture, slice, and tile boundaries

DF: deblocking filtering; SAO: sample adaptive offset; ALF: adaptive loop filtering

|  | loop_filter_across_subpic_enabled_flag[i] | loop_filter_across_tiles_enabled_flag | loop_filter_across_slices_enabled_flag |
| --- | --- | --- | --- |
| DF | Disable DF on both sides of a to-be-processed block edge if any side of the to-be-processed block edge has a corresponding loop_filter_across_subpic_enabled_flag[i] equal to 0 | Disable DF on both sides of a to-be-processed horizontal/vertical block edge if the two sides of the to-be-processed horizontal/vertical block edge belong to different tiles and loop_filter_across_tiles_enabled_flag of the right/bottom side tile is equal to 0 | Disable DF on both sides of a to-be-processed horizontal/vertical block edge if the two sides of the to-be-processed horizontal/vertical block edge belong to different slices and loop_filter_across_tiles_enabled_flag of the right/bottom side slice is equal to 0 |
| SAO | Disable SAO of a to-be-processed sample if SAO of the to-be-processed sample would need any sample from a different subpicture | Disable SAO of a to-be-processed sample if SAO of the to-be-processed sample would need any sample from a different tile | Disable SAO of a to-be-processed sample if SAO of the to-be-processed sample would need any sample from a different slice |
| ALF | Apply sample padding for ALF of a to-be-processed sample if ALF of the to-be-processed sample would need any sample from a different subpicture, where the sample padding avoids using samples from different subpictures | Apply sample padding for ALF of a to-be-processed sample if ALF of the to-be-processed sample would need any sample from a different tile, where the sample padding avoids using samples from different tiles | Apply sample padding for ALF of a to-be-processed sample if ALF of the to-be-processed sample would need any sample from a different slice, where the sample padding avoids using samples from different slices |

From the above, it can be seen that in the decoding processes, the tile boundary and the slice boundary are treated similarly. However, in VVC Draft 8, there are some differences between the usage of tile and slice as follows.

- Subpicture layout is directly based on slice instead of tile.

- Subpicture boundaries coincide with slice boundaries (and tile boundaries in turns).

- Subpicture boundaries related processing overrides the processing of tile and slice boundaries.

- Slice size defines the data size in a single NAL unit.

- Parallel processing may be based on tile partitions or slice layout.

The following discussions assume tile or slice based parallel processing is not used and a tile partition and a slice layout are given.

For Case 1 and Case 2, the tile partition inside the slice does not contribute much in coding efficiency instead of placing the impositions of:

- CTUs in a tile are in raster scan order

- Need of tile boundaries checking (actually, affecting use of coding tools and consequently reducing the coding efficiency)

- Possible tile-based parallel processing (actually, slice-based parallel processing may be possible as well and may replace tile-based parallel processing)

For a better coding efficiency, the use cases can set loop_filter_across_tiles_enabled_flag to 1, while loop_filter_across_slices_enabled_flag can be set separtely as needed.

For Case 3, the impositions are:

- CTUs in a slice are in raster scan order

- Need of slice boundaries checking (the same as that of tile, but only a slice can be placed in a single NAL unit)

- slice-based parallel processing may be possible

The tile partition outside the slices does not affect essential slice processing. Case 3 is actually motivated by the use cases with subpictures. In the use cases subpicture boundaries coincide with the tile boundaries and the subpicture boundary related processing overrides the tile boundary related processing.

Tile Boundaries

Slice Boundaries

Further for Case 3, see the illustrated example above, the flexibility of different slice partitions cannot be replaced by a finer tile partition granularity because a tile partition is always aligned to the picture width. Therefore the control of two types of boundary related processing are both needed with the two flags of loop_filter_across_tiles_enabled_flag and loop_filter_across_slices_enabled_flag as in VVC Draft 8.

To this end, we can conclude from the above cases that the flexibility of different slice partitions cannot be replaced by a finer tile partition and both flags, loop_filter_across_tiles_enabled_flag and loop_filter_across_slices_enabled_flag, are needed as in the current VVC draft. And for further discussions, based on the above list and comparison, it is alleged that control of the slice boundary related processing makes more sense than that of the tile boundary related processing.

References

- “Versatile Video Coding (Draft 8)”, JVET-Q2001-vE, January 2020, Brussels, Belgium.

Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).