| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0111 |
| --- | --- |

| Title: | AHG9: Vertical slice boundaries |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jonatan Samuelsson Sachin Deshpande Andrew Segall | Email: | samuelssonj@sharplabs.com |
| Source: | Sharp Labs of America, Inc. |  |  |

_____________________________

# Abstract

This contribution provides an analysis of using subpictures for server-side composition. It is asserted that it would be beneficial to be able to merge bitstreams from several different sources into a combined bitstream composed of multiple different subpictures. The contribution details some of the parameters and settings that needs to be aligned between different sources in order to support this use case and includes a proposal for one modification asserted to improve feasibility and reduce implementational burden; to allow a tile to include multiple slices either vertically or horizontally (which is different from the current VVC draft where only horizontally structured slices are allowed within a tile).

# Proposal

The complete proposed text changes based on a clean version of JVET-Q2001-vE are attached to this contribution.

In summary, the following is proposed:

- A new syntax element, vertical_slice_boundaries_flag, for indicating if a tile is split vertically or horizontally.

- Modification to CTU scan derivation to include two cases depending on the value of the proposed flag.

- Update to syntax and decoding process since a vertical slice boundary is no longer required to be aligned with a vertical tile boundary.

- Replace the level limit on number of vertical tile boundaries with a limit on the sum of vertical tile boundaries and vertical slice boundaries.

It is proposed to allow a tile to be split into slices either horizontally (as in the current VVC draft) or vertically (not possible in the current VVC draft) by introducing a flag, vertical_slice_boundaries_flag, signalled for a tile that contain more than one slice, indicating if the slices are horizontal slices (spanning the entire tile width) or vertical slices (spanning the entire tile height).

Since the syntax element that was previously used to indicate the heights of different slices is now  proposed to represent either the height or the width (depending on the value of vertical_slice_boundaries_flag) it is proposed to rename it from exp_slice_height_in_ctus_minus1 to something “direction neutral” such as exp_slice_span_in_ctus_minus1.

It is proposed to update equation 30 in section 6.5.1 (that derives scanning order and CTB to slice mapping) so that it would include two different versions of how CTBs are added to slices depending on the value of vertical_slice_boundaries_flag:

Additionally there are some places in the syntax and decoding process that are proposed to be updated since with the proposed changes the first CTU row of a slice is no longer necessarily aligned with the first CTU row of a tile.

## How to ensure that there are not too many vertical boundaries in the picture

With the proposed flag it is possible to introduce vertical prediction-breaking boundaries at positions that are not aligned with vertical tile boundaries. In the current VVC draft, there is a limitation (expressed in relation to the bitstream level) on the number of vertical tile boundaries. It would desirable to avoid that more vertical boundaries could be introduced with the proposed change compared to what is allowed in the current VVC draft, as this could be difficult for decoders to support. Therefore, it is proposed to extend the current limit on vertical tile boundaries to also include vertical slice boundaries. It is proposed that the total number of CTU columns that include a vertical slice or tile boundary is limited to the same amount as the limit on vertical tile boundaries in the current VVC draft.

# Motivation

One of the unique features of VVC is the concept of subpictures, which, among other things, makes it possible to encode spatial regions of the pictures of a video sequence independent of each other with a spatial layout that is consistent throughout a Coded Layer Video Sequence (CLVS). Since subpictures can be signalled to be independent of each other, it is possible to perform encoding of different spatial regions of the final video by encoders that do not have access to, or knowledge about the content of the other spatial regions.

Recent development in internet connectivity in terms of speed and reliability has enabled applications that rely on servers (central or at the edge) to perform resource heavy computations and rendering in the cloud and then transmit the resulting rendition as a compressed video stream to the user. One cluster of such applications is cloud gaming, for example Google Stadia [1] and Nvidia GeForce Now [2]. Other examples such as Network-assisted VR stitching and Interactive media services can be found in the Use cases and requirements document (N17502) of MPEG’s Network Based Media Processing work [3].

Figure 1 is an image borrowed from the CD text of NBMP (N18236) [4] that illustrates how several different source encodings (N videos) is composed into a single stream (1 video) transmitted to the user.

Figure 1. Workflow for E-Sports use case from N18236.

## Tiling

An analysis of alignment between source encoders is included with this contribution as Appendix A. In that analysis, all aspects and features of the VVC draft have been determined to provide sufficient support for subpicture-based server-side composition, with the exception of tiling.

The primary reason for this is that all other aspects can be determined, aligned, and configured without depending on the layout of the composition. However, in order to produce valid bitstreams that fulfill the requirements of tiles, slices and subpictures, some source encodes might need to contain internal tile boundaries that depend on the position of subpicture boundaries outside the subpicture in question.

The example composition from the E-sport use case in NBMP [4] is used here to illustrate the problem, but it should be noted that same problem will occur for various different layouts.

Figure 2. Example composition in E-Sport use case.

In the example in Figure 2, there are five different source encoders producing five different videos with resolutions configured such that they can be combined into a single bitstream.

But since all vertical subpicture boundaries in the current VVC draft needs to be aligned with tile boundaries, the desired composition is not allowed unless the different source encoders are configured to include internal tile boundaries that are aligned with the vertical subpicture boundaries outside of its own subpicture. The only way to enable this composition is through splitting all three of the wide videos into two tiles with a tile boundary in the middle.

Figure 3. Required tile structure in the E-Sport use case example.

This will obviously have coding efficiency impact, but it also requires the source encoders to have support for producing compressed video with vertical tile boundaries at any arbitrary CTU boundary (as determined by the layout of the composition). Furthermore, for use cases that additionally involves repositioning of different subpictures spatially, then the source encoders might need to support reconfiguring the tile layout when such repositioning occurs (which might not be at an IRAP picture). One way to avoid running is to this problem would be to constrain the use case to only allow layouts in which all vertical subpicture boundaries are fully aligned.

The authors of this contribution believes that it is unrealistic to expect such flexible tile boundary support at any arbitrary CTU boundary from source encoders, and that it would be too restrictive to constrain the use case to only allow layouts where all vertical subpicture boundaries were fully aligned.

Another example is shown below in which the main video portion of a video conferencing composition would have to be coded with four vertical tiles in order for the thumbnail videos to be contained in their own subpictures with the current VVC WD signalling:

Figure 4. Required tile structure in a video conferencing composition example.

It is asserted that it would be desirable to allow vertical subpicture boundaries that do not require a tile boundary to be introduced across the entire picture at that position.

## Other use cases for vertical subpicture boundaries

It should be noted that subpicture-based server-side composition is not the only use case that would benefit from allowing vertical subpicture boundaries that do not introduce prediction-breaking boundaries across the entire picture.

The following ERP layout is presented in JVET-Q0212 [5] and a similar layout is presented in JVET-Q0403 [6] (but in that layout the top and bottom view are put at the top of the composed video):

Figure 5. Example ERP layout as presented in JVET-Q0212.

Encoding layout shown in Figure 5 with three subpictures will result in a tile boundary that needs to cut through the center view at the position of the boundary between the top view and the bottom view as shown in Figure 6.

Figure 6. Required tile structure in the ERP layout example.

It is asserted that it is generally undesirable to be forced to insert a tile boundary cutting through the center view. It would be desirable to allow splitting the lower portion of the video into two subpictures without affecting how the center view is coded.

# How the proposed change addresses the example use cases

The Figures 7-9 illustrates the difference in the picture partitioning between the current VVC draft and the proposed text. In all these examples, there is a 1-to-1 mapping between slices and subpictures (each slice is a subpicture and each subpicture is a slice), but it should be noted that both the current VVC draft and the proposed text allows for subpictures to be split into multiple slices.

Figure 7. Example difference between current VVC draft (left) and proposed text (right).

Figure 8. Example difference between current VVC draft (left) and proposed text (right).

Figure 9. Example difference between current VVC draft (left) and proposed text (right).

# Summary

In order to provide adequate support for subpicture-based server-side composition it is proposed to introduce a flag to indicate if a tile is split into slices horizontally (as in the current VVC draft) or vertically (not possible in the current VVC draft). It is asserted that allowing a tile to be split into slices vertically enables subpicture-based server-side composition to be performed without requiring internal tiling of independent source encoders based on the composition layout. It is also asserted that the total number of prediction-breaking boundaries in the pictures can be (and is suggested to be) limited in a way that is fully aligned with the limitation on number of vertical tile boundaries in the current VVC draft.

# References

[1] https://stadia.google.com/

[2] https://www.nvidia.com/en-us/geforce-now/

[3] N17502 NBMP Use cases and requirements (v4)

[4] M49160 “Text of ISO/IEC CD 23090-8 Network-based Media Processing”

[5] JVET-Q0212, “On sub-picture wrap around signaling” Y. He, A. Hamza (InterDigital), B. Choi, S. Wenger (Tencent), January 2020

[6] JVET-Q0403, “On subpicture specific MV wraparound” R. Skupin, Y. Sanchez, K. Suehring, T. Schierl (HHI), January 2020

# Patent rights declaration(s)

Sharp Labs of America, Inc., may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Appendix A: Analysis of source encoder alignment

In use-cases where videos from different source are merged together into a combined video using subpictures for spatial composition, it is necessary to have a high level of alignment between the different source encoders. For efficient composition, it is desirable to not require any transcoding or rewriting below the parameter set level. This means that NAL units containing picture header, slice header and slice data can be included untouched in the merged bitstream.

In order to accomplish this, there are several parameters and settings that needs to be controlled and/or aligned between various source encoders.

The following table provides an analysis of to what extent various features of the VVC draft can be aligned between various source encoders and comments on how such alignment is achieved.

| Feature / aspect | Alignment feasible | Comment |
| --- | --- | --- |
| POC (coding order) | Yes | Encoders must use a predefined coding order pattern – in most low-delay applications, coding order is the same as output order. |
| Reference picture lists | Yes | Encoders must be configurable to use the same (fixed) reference configuration (disable dynamic reference picture selection). |
| NAL unit types | Yes | Encoders must generally be configurable to use the same NAL unit type. mixed_nalu_types_in_pic_flag can be used for specific cases where different types are desired from different encoders. |
| Bit depth and chroma format | Yes | Encoders must use the same internal bit depth and chroma format. |
| Picture resolution | Yes | Encoders must be able to provide encoded video at resolutions that can be composed into a merged video. Generally, resampling (and sometimes cropping), must be supported to create (sub)pictures consisting of integer number CTUs. |
| Reference picture resampling | Yes | Reference picture resampling needs to be disabled in the source encoders since the resolution of the different subpictures must remain constant throughout the CLVS. |
| Tile and slice structure | No | Tile structure needs be consistent across the picture and all vertical subpicture boundaries need to constitute tile boundaries. For layouts where all vertical subpicture boundaries are aligned, this can be achieved by not allowing any internal vertical tiling. For layouts where not all vertical subpicture boundaries are aligned, this can be difficult to support and vertical prediction-breaking boundaries needs to be introduced with a risk of reduced subjective quality and coding performance. See detailed discussion and a potential solution below. |
| Picture level coding tools | Yes | Encoders must have support for a common subset of coding tools for all tools that cannot be different in different slices. |
| Slice level coding tools | Yes | Coding tools that can be different in different slices can be used differently by different source encoders but must be possible to control such that the composed video obeys the constraints on these tools. . |
| Parameter set ids | Yes | Encoders need to use the same PPS ID. For APS ID, different encoders must be configured to use different APS ids unless the content of the APS is aligned between different encoders. |
| Subpic ID | Yes | Encoders must be configured to include Subpic ID and must use different Subpic IDs. |
| Slice address | Yes | Slice address mapping is within each subpicture. No alignment needed. |
| Quantization parameters | Yes | Different slices can use different QP and different methods for locally adaptive QP, but the same granularity of CU level qp delta coding must be used (since it is signalled in PH). |