| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0112 |
| --- | --- |

| Title: | AHG9: On picture header |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jonatan Samuelsson Sachin Deshpande Andrew Segall | Email: | samuelssonj@sharplabs.com |
| Source: | Sharp Labs of America, Inc. |  |  |

_____________________________

# Abstract

This contribution includes a proposal for a modification in the picture header. It is asserted that the proposed modification improves clarity, removes redundancy and reduces the risk of incorrect (or confusing) values being signaled for IRAP pictures. In summary, it is proposed that the flag ph_inter_slice_allowed_flag is not signalled for IRAP pictures. Instead, the value of ph_inter_slice_allowed_flag is inferred to be equal to 0 for IRAP pictures.

# Introduction

The current VVC draft includes two flags at the beginning of the picture header that can be used to determine if a picture is an IRAP picture. When gdr_or_irap_pic_flag is equal to 1 and gdr_pic_flag is equal to 0, then the picture is an IRAP picture.

The current VVC draft also includes the flag ph_inter_slice_allowed_flag that indicates if inter slices may be present in the picture. However, when the picture is an IRAP picture, all slices must be intra slices. It is asserted that it is redundant and confusing to signal ph_inter_slice_allowed_flag for IRAP pictures.

# Proposal

It is proposed that the flag ph_inter_slice_allowed_flag is inferred to be equal to 0 for IRAP pictures instead of being signalled. As a minor detail of the proposal it is suggested to clarify that gdr_or_irap_pic_flag has to be set equal to 1 when the picture is a GDR picture (this is aligned with the semantics of gdr_pic_flag which needs to be present and equal to 1 in order for recovery_poc_cnt to be signalled).

| picture_header_structure( ) { | Descriptor |
| --- | --- |
| gdr_or_irap_pic_flag | u(1) |
| if( gdr_or_irap_pic_flag ) |  |
| gdr_pic_flag | u(1) |
| if( !gdr_or_irap_pic_flag ¦ ¦ gdr_pic_flag ) |  |
| ph_inter_slice_allowed_flag | u(1) |
| if( ph_inter_slice_allowed_flag ) |  |
| ph_intra_slice_allowed_flag | u(1) |
| non_reference_picture_flag | u(1) |
| ph_pic_parameter_set_id | ue(v) |
| ph_pic_order_cnt_lsb | u(v) |
| if( gdr_or_irap_pic_flag ) |  |

#### Picture header structure semantics

The PH syntax structure contains information that is common for all slices of the coded picture associated with the PH syntax structure.

gdr_or_irap_pic_flag equal to 1 specifies that the current picture is a GDR or IRAP picture. gdr_or_irap_pic_flag equal to 0 specifies that the current picture is not a GDR picture and may or may not be a GDR or an IRAP picture.

gdr_pic_flag equal to 1 specifies the picture associated with the PH is a GDR picture. gdr_pic_flag equal to 0 specifies that the picture associated with the PH is not a GDR picture. When not present, the value of gdr_pic_flag is inferred to be equal to 0. When gdr_enabled_flag is equal to 0, the value of gdr_pic_flag shall be equal to 0.

ph_inter_slice_allowed_flag equal to 0 specifies that all coded slices of the picture have slice_type equal to 2. ph_inter_slice_allowed_flag equal to 1 specifies that there may or may not be one or more coded slices in the picture that have slice_type equal to 0 or 1. When not present, the value of ph_inter_slice_allowed_flag is inferred to be equal to 0.

ph_intra_slice_allowed_flag equal to 0 specifies that all coded slices of the picture have slice_type equal to 0 or 1. ph_intra_slice_allowed_flag equal to 1 specifies that there may or may not be one or more coded slices in the picture that have slice_type equal to 2.When not present, the value of ph_intra_slice_allowed_flag is inferred to be equal to 1.

# Summary

It is proposed that the flag ph_inter_slice_allowed_flag is inferred to be equal to 0 for IRAP pictures instead of being signalled.

# Patent rights declaration(s)

Sharp Labs of America, Inc., may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).