| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0116 |
| --- | --- |

| Title: | AHG11/AHG14: On sign data hiding of transform skip block |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Mohammed Golam Sarwer Yan Ye, Jiancong Luo 525 Almanor Ave, Sunnyvale, CA | Tel: Email: | + 1 408-858-6780 m.sarwer@alibaba-inc.com |
| Source: | Alibaba Group |  |  |

_____________________________

# Abstract

It is asserted that, in VTM-8.0, if slice_ts_residual_coding_disabled_flag == 1, the encoder RDOQ process does not select proper residual coding method for transform skip (TS) and BDPCM blocks. In this contribution, the encoder RDOQ process is modified so that it can select correct residual coding method during RDOQ process.

It is further reported that, in VTM-8.0 software, an encoder-decoder mismatch is also observed if both slice_ts_residual_coding_disabled_flag == 1 and pic_sign_data_hiding_enabled_flag == 1. This contribution also proposes two methods to resolve the encoder-decoder mismatch issue. In the first method, sign data hiding (SDH) is disabled for BDPCM blocks only, whereas in the second method, SDH is disabled for both TS and BDPCM blocks. The encoder RDOQ bug fix (mentioned above) are included in both of the methods.

# Problem Statement

In VVC draft 8, both the transform-skip (TS) and BDPCM blocks are allowed to select either regular residual coding or TS residual coding method. If slice_ts_residual_coding_disabled_flag is equal to 0, blocks coded in TS and BDPCM modes of that slice select TS residual coding. If slice_ts_residual_coding_disabled_flag is equal to 1, the TS and BDPCM coded blocks of that slice select regular residual coding.

In current design, following issues are observed when slice_ts_residual_coding_disabled_flag == 1.

- Encoder RDOQ bug: If the slice_ts_residual_coding_disabled_flag ==1, both of the TS and BDPCM blocks performed RDOQ process using TS residual coding, which is not correct. The proper residual coding method on which RDOQ is performed should be regular residual coding, since the residuals are coded using regular residual coding (given value of slice_ts_residual_coding_disabled_flag is equal to 1).

- Encoder-decoder mismatch when sign data hiding is enabled: In VVC draft 8, both the TS and BDPCM blocks are allowed to select sign data hiding (SDH) if both pic_sign_data_hiding_enabled_flag ==1, and   slice_ts_residual_coding_disabled_flag == 1.  However, current VTM software produces encoder-decoder mismatch under these conditions.

# Proposed methods

## Method 1: Encoder RDOQ bug-fix

In the method 1, the encoder RDOQ algorithm of VTM reference software is modified. In the proposed method, if the slice_ts_residual_coding_disabled_flag ==1, both of the TS and BDPCM blocks performed RDOQ process using regular residual coding method.

The simulation results of the proposed encoder RDOQ bug-fix is shown in Table 1.  In both anchor and test, slice_ts_residual_coding_disabled_flag is set to 1 for all slices. All other configurations are kept the same as CTC (for instance, dependent quantization is enabled). Since the proposed method uses correct residual coding method in RDOQ process, bit estimations during RDOQ process become more accurate and coding gain is observed especially in class F and TGM sequences.

Table 1: performance improvement of proposed encoder bug-fix over VTM-8.0

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | 0.04% | 0.06% | 102% | 100% |
| Class A2 | 0.01% | -0.08% | 0.00% | 101% | 100% |
| Class B | 0.00% | -0.01% | 0.02% | 101% | 100% |
| Class C | -0.01% | -0.02% | 0.06% | 101% | 99% |
| Class E | 0.00% | -0.02% | -0.05% | 101% | 100% |
| Overall | 0.00% | -0.02% | 0.02% | 101% | 100% |
| Class D | -0.01% | 0.07% | 0.17% | 100% | 100% |
| Class F | -0.17% | -0.08% | -0.02% | 100% | 100% |
| Class SCC | -0.36% | -0.45% | -0.42% | 101% | 99% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | 0.06% | -0.03% | 100% | 100% |
| Class A2 | -0.01% | 0.03% | -0.08% | 100% | 100% |
| Class B | 0.01% | -0.04% | -0.04% | 100% | 100% |
| Class C | 0.00% | -0.02% | 0.05% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.00% | -0.02% | 100% | 100% |
| Class D | 0.00% | -0.04% | 0.00% | 100% | 101% |
| Class F | -0.12% | -0.17% | -0.03% | 100% | 100% |
| Class SCC | -0.28% | -0.24% | -0.29% | 100% | 99% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.01% | -0.07% | 0.05% | 100% | 100% |
| Class C | 0.05% | -0.10% | -0.20% | 100% | 100% |
| Class E | -0.15% | -0.13% | 0.36% | 100% | 100% |
| Overall | -0.02% | -0.10% | 0.04% | 100% | 100% |
| Class D | -0.19% | 0.49% | 0.65% | 100% | 101% |
| Class F | -0.18% | 0.29% | 0.12% | 100% | 99% |
| Class SCC | -0.39% | -0.14% | -0.16% | 100% | 100% |

## Method 2: Interaction between sign data hiding and TS mode

As mentioned in section 1, the current VTM software introduces encoder-decoder mismatch if both slice_ts_residual_coding_disabled_flag == 1 and pic_sign_data_hiding_enabled_flag ==1. By fixing the encoder RDOQ bug described in method 1 (section 2.1) can partially fix this mismatch issue. The bug fix in method 1 (described in section 2.1) can guarantee SDH’s necessary conditions for TS without BDPCM blocks, therefore, mismatch due to TS without BDPCM blocks can be resolved. However, mismatch still exist for TS with BDPCM block since there is not any encoder algorithm to perform SDH with BDPCM.

In order to completely resolve the issue of encoder-decoder mismatch in case of SDH, the following two methods are proposed on the top of encoder RDOQ bug fix described in method 1.

### Method 2.1: disable SDH for BDPCM blocks

Given that there is not any efficient encoder algorithm to guarantee the necessary SDH’s condition for BDPCM block, in this method, we proposes to disable SDH for BDPCM blocks ( i.e. BdpcmFlag ==1).

The changes as compared to VVC draft 8 is shown in Table 2.

Table 2: residual coding syntax of the proposed method 2.1

| residual_coding( x0, y0, log2TbWidth, log2TbHeight, cIdx ) { | Descriptor |  |
| --- | --- | --- |
| ……………….. |  |  |
| if( ph_dep_quant_enabled_flag ¦ ¦ !pic_sign_data_hiding_enabled_flag ¦ ¦ BdpcmFlag[ x0 ][ y0 ][ cIdx ]) |  |
| signHidden = 0 |  |
| else |  |
| signHidden = ( lastSigScanPosSb − firstSigScanPosSb > 3 ? 1 : 0 ) |  |
| … |  |
| } |  |  |

The redundant condition checking of ph_dep_quant_enabled_flag  is removed from Table 2 because according to current VVC specification, there is not any valid case where both of the flags pic_sign_data_hiding_enabled_flag and ph_dep_quant_enabled_flag are 1.

### Method 2.2: disable SDH for both TS and BDPCM blocks

In this method, if the transform_skip_flag == 1, the SDH is disabled regardless the value of BdpcmFlag. The changes as compared to VVC draft 8 is shown in Table 3. The key difference between method 2.1 and method 2.2 is that SDH of TS without BDPCM blocks is enabled in method 2.1 but disabled in method 2.2.

Table 3: residual coding syntax of the proposed method 2.2

| residual_coding( x0, y0, log2TbWidth, log2TbHeight, cIdx ) { | Descriptor |  |
| --- | --- | --- |
| ……………….. |  |  |
| if( ph_dep_quant_enabled_flag ¦ ¦ !pic_sign_data_hiding_enabled_flag ¦ ¦ transform_skip_flag [ x0 ][ y0 ][ cIdx ]) |  |
| signHidden = 0 |  |
| else |  |
| signHidden = ( lastSigScanPosSb − firstSigScanPosSb > 3 ? 1 : 0 ) |  |
| … |  |
| } |  |  |

# Simulation results

The proposed methods 2.1 and 2.2 are implemented on top of encoder RDOQ bug fix (described in method 1). Following two tests are conducted:

- Test 1: method 2.1.

- Test 2: method 2.2.

Following are the test configurations:

- Dependent quantization is disabled

- Sign data hiding is enabled

- slice_ts_residual_coding_disabled_flag is set to 1 for all slices.

- All other settings are same as CTC

The simulation results are tabulated in Table 4. Since under the above mentioned conditions, VTM-8.0 produce encoder-decoder mismatch, there is not any valid results of VTM-8.0. Therefore, test 1 is used as anchor.

Table 4:  performance of test 2 over test 1

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over Test 1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.03% | 0.03% | 0.05% | 100% | 100% |
| Class A2 | 0.00% | -0.02% | 0.02% | 100% | 100% |
| Class B | 0.01% | 0.01% | -0.07% | 99% | 100% |
| Class C | 0.03% | 0.01% | 0.04% | 100% | 100% |
| Class E | -0.01% | 0.04% | 0.11% | 99% | 100% |
| Overall | 0.01% | 0.01% | 0.02% | 99% | 100% |
| Class D | 0.03% | 0.05% | 0.15% | 99% | 101% |
| Class F | -0.13% | 0.16% | 0.07% | 100% | 100% |
| Class SCC | -0.38% | 0.43% | 0.31% | 99% | 100% |
|  | Random access Main10 |  |  |  |  |
|  | Over Test 1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | 0.03% | 0.02% | 100% | 100% |
| Class A2 | 0.02% | 0.11% | 0.10% | 100% | 100% |
| Class B | 0.02% | -0.03% | -0.04% | 100% | 100% |
| Class C | 0.02% | -0.06% | 0.16% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.02% | 0.00% | 0.05% | 100% | 100% |
| Class D | 0.09% | 0.08% | 0.09% | 100% | 99% |
| Class F | -0.06% | 0.26% | 0.26% | 100% | 100% |
| Class SCC | -0.40% | 0.38% | 0.40% | 100% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over Test 1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.01% | 0.14% | 0.62% | 100% | 100% |
| Class C | 0.04% | 0.14% | 0.10% | 100% | 100% |
| Class E | -0.03% | -0.65% | -0.52% | 100% | 100% |
| Overall | 0.01% | -0.06% | 0.16% | 100% | 100% |
| Class D | 0.08% | 0.05% | -0.41% | 100% | 100% |
| Class F | -0.11% | 0.62% | 0.16% | 100% | 99% |
| Class SCC | -0.28% | 0.10% | 0.14% | 100% | 100% |

# Conclusion

In this contribution, the encoder RDOQ process is modified so that encoder can select right residual coding method during RDOQ process. This contribution also fixes the encoder/decoder mismatch issue when slice_ts_residual_coding_disabled_flag == 1 and pic_sign_data_hiding_enabled_flag ==1. In the method 2.1, SDH is disabled only for BDPCM block but enabled for TS with non BDPCM blocks. In the method 2.2, SDH is disabled both TS with and without BDPCM blocks. The method 2.2 can achieve -0.38%(AI), -0.40%(RA), and -0.28% (LB) luma BD-rate as compared to method 2.1 for TGM sequences.

# Patent rights declaration(s)

Alibaba Group may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).