| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0117-v2 |
| --- | --- |

| Title: | AHG9/AHG12: On signaling of subpicture and slice in PPS |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Byeongdoo Choi Stephan Wenger Shan Liu | Email: | bdchoi@tencent.com |
| Source: | Tencent |  |  |

_____________________________

# Abstract

This contribution proposes the following clean-ups in PPS:

- Proposal-1: Signaling pps_num_subpics_minus1 in PPS as mandatory, to avoid parsing dependency on SPS.

- Proposal-2: Replacing num_slices_in_pic_minus1 with num_slices_in_pic_minus_num_sub_pics, for efficient signaling.

# Proposal-1: Signaling pps_num_subpics_minus1 in PPS as mandatory

In the latest version of VVC draft (JVET-Q2001-vE), pps_num_subpics_minus1 is optionally signaled when subpic_id_mapping_in_pps_flag is equal to 1. The value of pps_num_subpics_minus1 in PPS is equal to the value of sps_num_subpics_minus1 in SPS. When the number of subpictures is used for parsing the syntax elements in PPS, the pps_num_subpics_minus1 needs to be signaled in PPS to avoid parsing dependency, even though the sps_num_subpics_minus1 is signaled in SPS.

The problem is that the value of sps_num_subpics_minus1 is used for derivation of the value of num_slices_in_pic_minus1, when single_slice_per_subpic_flag is equal to 1. The value of num_slices_in_pic_minus1 is used for parsing the subsequent syntax elements in PPS, including tile_idx_delta_present_flag, slice_width_in_tiles_minus1[ i ], slice_height_in_tiles_minus1[ i ] and so on. Thus, the value of sps_num_subpics_minus1 is needed to parse the slice partitioning information in PPS. It causes a parsing dependency from SPS to PPS.

Therefore, it is proposed to relocate the pps_num_subpics_minus1 out of the conditional signaling bracket, to signal the syntax element as mandatory in PPS.

Proposed Text of Proposal-1:

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| pps_num_subpics_minus1 | ue(v) |
| subpic_id_mapping_in_pps_flag | u(1) |
| if( subpic_id_mapping_in_pps_flag ) { |  |
| pps_num_subpics_minus1 | ue(v) |
| pps_subpic_id_len_minus1 | ue(v) |
| for( i = 0; i <= pps_num_subpic_minus1; i++ ) |  |
| pps_subpic_id[ i ] | u(v) |
| } |  |
| no_pic_partition_flag | u(1) |
| if( !no_pic_partition_flag ) { |  |
| pps_log2_ctu_size_minus5 | u(2) |
| num_exp_tile_columns_minus1 | ue(v) |
| num_exp_tile_rows_minus1 | ue(v) |
| for( i = 0; i <= num_exp_tile_columns_minus1; i++ ) |  |
| tile_column_width_minus1[ i ] | ue(v) |
| for( i = 0; i <= num_exp_tile_rows_minus1; i++ ) |  |
| tile_row_height_minus1[ i ] | ue(v) |
| if( NumTilesInPic > 1 ) |  |
| rect_slice_flag | u(1) |
| if( rect_slice_flag ) |  |
| single_slice_per_subpic_flag | u(1) |
| if( rect_slice_flag && !single_slice_per_subpic_flag ) { |  |
| num_slices_in_pic_minus1 | ue(v) |
| if( num_slices_in_pic_minus1 > 0 ) |  |
| tile_idx_delta_present_flag | u(1) |
| for( i = 0; i < num_slices_in_pic_minus1; i++ ) { |  |
| if( NumTileColumns > 1 ) |  |
| slice_width_in_tiles_minus1[ i ] | ue(v) |
| if( NumTileRows > 1 && ( tile_idx_delta_present_flag ¦ ¦ SliceTopLeftTileIdx[ i ] % NumTileColumns = = 0 ) ) |  |
| slice_height_in_tiles_minus1[ i ] | ue(v) |
| if( slice_width_in_tiles_minus1[ i ] = = 0 && slice_height_in_tiles_minus1[ i ] = = 0 && RowHeight[ SliceTopLeftTileIdx[ i ] / NumTileColumns ] > 1 ) { |  |
| num_exp_slices_in_tile[ i ] | ue(v) |
| for( j = 0; j < num_exp_slices_in_tile[ i ]; j++ ) |  |
| exp_slice_height_in_ctus_minus1[ i ][ j ] | ue(v) |
| i += NumSlicesInTile[ i ] − 1 |  |
| } |  |
| if( tile_idx_delta_present_flag && i < num_slices_in_pic_minus1 ) |  |
| tile_idx_delta[ i ] | se(v) |
| } |  |
| } |  |
| loop_filter_across_tiles_enabled_flag | u(1) |
| loop_filter_across_slices_enabled_flag | u(1) |
| } |  |
| … |  |
| } |  |

num_slices_in_pic_minus1 plus 1 specifies the number of rectangular slices in each picture referring to the PPS. The value of num_slices_in_pic_minus1 shall be in the range of 0 to MaxSlicesPerPicture − 1, inclusive, where MaxSlicesPerPicture is specified in Annex A. When no_pic_partition_flag is equal to 1, the value of num_slices_in_pic_minus1 is inferred to be equal to 0. When single_slice_per_subpic_flag is equal to 1, the value of num_slices_in_pic_minus1 is inferred to be equal to sps_num_subpics_minus1 pps_num_subpics_minus1.

# Proposal-2: Replacing num_slices_in_pic_minus1 with num_slices_in_pic_minus_num_sub_pics

In the latest version of VVC draft (JVET-Q2001-vE), num_slices_in_pic_minus1 is signaled with ue(v) coding in PPS. The issue is that the number of slices should be greater than or equal to the number of the subpictures, always. Therefore, for efficient signaling, it is proposed to replace num_slices_in_pic_minus1 with num_slices_in_pic_minus_num_subpics or num_slices_in_pic_minus_num_subpics_minus1.

Proposed Text (option-1) of Proposal-2:

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| pps_num_subpics_minus1 | ue(v) |
| subpic_id_mapping_in_pps_flag | u(1) |
| if( subpic_id_mapping_in_pps_flag ) { |  |
| pps_num_subpics_minus1 | ue(v) |
| pps_subpic_id_len_minus1 | ue(v) |
| for( i = 0; i <= pps_num_subpic_minus1; i++ ) |  |
| pps_subpic_id[ i ] | u(v) |
| } |  |
| no_pic_partition_flag | u(1) |
| if( !no_pic_partition_flag ) { |  |
| pps_log2_ctu_size_minus5 | u(2) |
| num_exp_tile_columns_minus1 | ue(v) |
| num_exp_tile_rows_minus1 | ue(v) |
| for( i = 0; i <= num_exp_tile_columns_minus1; i++ ) |  |
| tile_column_width_minus1[ i ] | ue(v) |
| for( i = 0; i <= num_exp_tile_rows_minus1; i++ ) |  |
| tile_row_height_minus1[ i ] | ue(v) |
| if( NumTilesInPic > 1 ) |  |
| rect_slice_flag | u(1) |
| if( rect_slice_flag ) |  |
| single_slice_per_subpic_flag | u(1) |
| if( rect_slice_flag && !single_slice_per_subpic_flag ) { |  |
| num_slices_in_pic_minus1_num_subpics | ue(v) |
| if( NumSlicesInPic > 1 ) |  |
| tile_idx_delta_present_flag | u(1) |
| for( i = 0; i < NumSlicesInPic-1; i++ ) { |  |
| if( NumTileColumns > 1 ) |  |
| slice_width_in_tiles_minus1[ i ] | ue(v) |
| if( NumTileRows > 1 && ( tile_idx_delta_present_flag ¦ ¦ SliceTopLeftTileIdx[ i ] % NumTileColumns = = 0 ) ) |  |
| slice_height_in_tiles_minus1[ i ] | ue(v) |
| if( slice_width_in_tiles_minus1[ i ] = = 0 && slice_height_in_tiles_minus1[ i ] = = 0 && RowHeight[ SliceTopLeftTileIdx[ i ] / NumTileColumns ] > 1 ) { |  |
| num_exp_slices_in_tile[ i ] | ue(v) |
| for( j = 0; j < num_exp_slices_in_tile[ i ]; j++ ) |  |
| exp_slice_height_in_ctus_minus1[ i ][ j ] | ue(v) |
| i += NumSlicesInTile[ i ] − 1 |  |
| } |  |
| if( tile_idx_delta_present_flag && i < NumSlicesInPic - 1 ) |  |
| tile_idx_delta[ i ] | se(v) |
| } |  |
| } |  |
| loop_filter_across_tiles_enabled_flag | u(1) |
| loop_filter_across_slices_enabled_flag | u(1) |
| } |  |
| … |  |
| } |  |

num_slices_in_pic_minus_num_subpics plus pps_num_subpics_minus1 + 1 specifies the number of rectangular slices in each picture referring to the PPS. The value of num_slices_in_pic_minus_num_subpics shall be in the range of 0 to MaxSlicesPerPicture − pps_num_subpics_minus1 - 1, inclusive, where MaxSlicesPerPicture is specified in Annex A. When single_slice_per_subpic_flag is equal to 1, the value of num_slices_in_pic_minus_num_sub_pics is inferred to be equal to 0.

The value of the variable NumSlicesInPic that indicates the number of slices in the picture is derived as follows:

if ( no_pic_partition_flag == 1 )
	NumSlicesInPic = 1
else
	NumSlicesInPic = pps_num_subpics_minus1 + num_slices_in_pic_minus_num_subpics + 1

Proposed Text (option-2) of Proposal-2:

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| pps_num_subpics_minus1 | ue(v) |
| subpic_id_mapping_in_pps_flag | u(1) |
| if( subpic_id_mapping_in_pps_flag ) { |  |
| pps_num_subpics_minus1 | ue(v) |
| pps_subpic_id_len_minus1 | ue(v) |
| for( i = 0; i <= pps_num_subpic_minus1; i++ ) |  |
| pps_subpic_id[ i ] | u(v) |
| } |  |
| no_pic_partition_flag | u(1) |
| if( !no_pic_partition_flag ) { |  |
| pps_log2_ctu_size_minus5 | u(2) |
| num_exp_tile_columns_minus1 | ue(v) |
| num_exp_tile_rows_minus1 | ue(v) |
| for( i = 0; i <= num_exp_tile_columns_minus1; i++ ) |  |
| tile_column_width_minus1[ i ] | ue(v) |
| for( i = 0; i <= num_exp_tile_rows_minus1; i++ ) |  |
| tile_row_height_minus1[ i ] | ue(v) |
| if( NumTilesInPic > 1 ) |  |
| rect_slice_flag | u(1) |
| if( rect_slice_flag ) |  |
| single_slice_per_subpic_flag | u(1) |
| if( rect_slice_flag && !single_slice_per_subpic_flag ) { |  |
| num_slices_in_pic_minus1_num_subpics_minus1 | ue(v) |
| if(NumSlicesInPic > 1 ) |  |
| tile_idx_delta_present_flag | u(1) |
| for( i = 0; i < NumSlicesInPic - 1; i++ ) { |  |
| if( NumTileColumns > 1 ) |  |
| slice_width_in_tiles_minus1[ i ] | ue(v) |
| if( NumTileRows > 1 && ( tile_idx_delta_present_flag ¦ ¦ SliceTopLeftTileIdx[ i ] % NumTileColumns = = 0 ) ) |  |
| slice_height_in_tiles_minus1[ i ] | ue(v) |
| if( slice_width_in_tiles_minus1[ i ] = = 0 && slice_height_in_tiles_minus1[ i ] = = 0 && RowHeight[ SliceTopLeftTileIdx[ i ] / NumTileColumns ] > 1 ) { |  |
| num_exp_slices_in_tile[ i ] | ue(v) |
| for( j = 0; j < num_exp_slices_in_tile[ i ]; j++ ) |  |
| exp_slice_height_in_ctus_minus1[ i ][ j ] | ue(v) |
| i += NumSlicesInTile[ i ] − 1 |  |
| } |  |
| if( tile_idx_delta_present_flag && i < NumSlicesInPic - 1) |  |
| tile_idx_delta[ i ] | se(v) |
| } |  |
| } |  |
| loop_filter_across_tiles_enabled_flag | u(1) |
| loop_filter_across_slices_enabled_flag | u(1) |
| } |  |
| … |  |
| } |  |

pps_num_subpics_minus1 shall be equal to sps_num_subpics_minus1.

num_slices_in_pic_minus_num_subpics_minus1 plus pps_num_subpics_minus1 + 2 specifies the number of rectangular slices in each picture referring to the PPS. The value of num_slices_in_pic_minus num_subpics_minus1 shall be in the range of 0 to MaxSlicesPerPicture − pps_num_subpics_minus1 - 2, inclusive, where MaxSlicesPerPicture is specified in Annex A. When single_slice_per_subpic_flag is equal to 1, the value of num_slices_in_pic_minus_num_sub_pics_minus1 is inferred to be equal to 0.

The value of the variable NumSlicesInPic that indicates the number of slices in the picture is derived as follows:

if ( no_pic_partition_flag == 1 )
	NumSlicesInPic = 1
else if ( single_slice_per_subpic_flag == 1 )

NumSlicesInPic = pps_num_subpics_minus1 + 1
else
	NumSlicesInPic = pps_num_subpics_minus1 +  num_slices_in_pic_minus_num_subpics_minus1 + 2

# Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).