| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0119 |
| --- | --- |

| Title: | AHG8/AHG9: On derivation of sublayer number in output layer set |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Byeongdoo Choi Stephan Wenger Shan Liu | Email: | bdchoi@tencent.com |
| Source: | Tencent |  |  |

_____________________________

# Abstract

This contribution proposes a few clean-ups and bug-fixes on the derivation process of the variables NumSubLayersInLayerInOLS[] and layerIncludedInOlsFlag[][] for output layer set:

- Changing the default value of max_tid_il_ref_pics_plus1[] from 7 to vps_max_sublayers_minus1 + 1, to avoid the wrong derivation case of the value of the variable NumSubLayersInLayerInOLS.

- Skipping the derivation process of the NumSubLayersInLayerInOLS[] and layerIncludedInOlsFlag[][] values, when vps_all_independent_layers_flag is equal to 1.

- Bug-fixing in the interation loop in eq. (4).

# Introduction

In the latest version of VVC draft (JVET-Q2001-vE), the values of the variables NumSubLayersInLayerInOLS[] and layerIncludedInOlsFlag[][] are derived from the syntax element max_tid_il_ref_pics_plus1 and the variables RefLayerIdx[][] and NumRefLayers[ ] that are derived from vps_direct_ref_layer_flag[][], in equation (40). This contribution proposes a few clean-ups and bug-fixes on the derivation process, including:

- Change the default value of max_tid_il_ref_pics_plus1[] from 7 to vps_max_sublayers_minus1 + 1, to avoid the wrong derivation case of the value of the variable NumSubLayersInLayerInOLS.

- Skip the derivation process of the NumSubLayersInLayerInOLS[] and layerIncludedInOlsFlag[][] values, when vps_all_independent_layers_flag is equal to 1.

- Bug fix in the interation loop in eq. (4).

# Proposed Text

max_tid_il_ref_pics_plus1[ i ] equal to 0 specifies that inter-layer prediction is not used by non-IRAP pictures of the i-th layer. max_tid_il_ref_pics_plus1[ i ] greater than 0 specifies that, for decoding pictures of the i-th layer, no picture with TemporalId greater than max_tid_il_ref_pics_plus1[ i ] − 1 is used as ILRP. When not present, the value of max_tid_il_ref_pics_plus1[ i ] is inferred to be equal to 7 vps_max_sublayers_minus1 + 1.

The variable NumOutputLayersInOls[ i ], specifying the number of output layers in the i-th OLS, the variable NumSubLayersInLayerInOLS[ i ][ j ], specifying the number of sublayers in the j-th layer in the i-th OLS, the variable OutputLayerIdInOls[ i ][ j ], specifying the nuh_layer_id value of the j-th output layer in the i-th OLS, and the variable LayerUsedAsOutputLayerFlag[ k ], specifying whether the k-th layer is used as an output layer in at least one OLS, are derived as follows:

NumOutputLayersInOls[ 0 ] = 1
OutputLayerIdInOls[ 0 ][ 0 ] = vps_layer_id[ 0 ]
NumSubLayersInLayerInOLS[ 0 ][ 0 ] = vps_max_sub_layers_minus1 + 1
LayerUsedAsOutputLayerFlag[ 0 ] = 1
for( i = 1, i  <=  vps_max_layers_minus1; i++ ) {
	if( each_layer_is_an_ols_flag  | |  ols_mode_idc < 2 )
		LayerUsedAsOutputLayerFlag[ i ] = 1
	else /*( !each_layer_is_an_ols_flag  &&  ols_mode_idc  = =  2 ) */
		LayerUsedAsOutputLayerFlag[ i ] = 0
}
for( i = 1; i < TotalNumOlss; i++ )
	if( each_layer_is_an_ols_flag  | |  ols_mode_idc  = =  0 ) {
		NumOutputLayersInOls[ i ] = 1
		OutputLayerIdInOls[ i ][ 0 ] = vps_layer_id[ i ]
		for( j = 0; j < i  &&  ( ols_mode_idc  = =  0 ); j++ )
			NumSubLayersInLayerInOLS[ i ][ j ] = max_tid_il_ref_pics_plus1[ i ]
		NumSubLayersInLayerInOLS[ i ][ i ] = vps_max_sub_layers_minus1 + 1
	} else if( ols_mode_idc  = =  1 ) {
		NumOutputLayersInOls[ i ] = i + 1
		for( j = 0; j < NumOutputLayersInOls[ i ]; j++ ) {
			OutputLayerIdInOls[ i ][ j ] = vps_layer_id[ j ]
			NumSubLayersInLayerInOLS[ i ][ j ] = vps_max_sub_layers_minus1 + 1
		}
	} else if( ols_mode_idc  = =  2 ) {
		for( j = 0; j  <=  vps_max_layers_minus1; j++ ) {
			layerIncludedInOlsFlag[ i ][ j ] = 0
			NumSubLayersInLayerInOLS[ i ][ j ] = 0
		}
		for( k = 0, j = 0; k  <=  vps_max_layers_minus1; k++ )	(40)
			if( ols_output_layer_flag[ i ][ k ] ) {
				layerIncludedInOlsFlag[ i ][ k ] = 1
				LayerUsedAsOutputLayerFlag[ k ] = 1
				OutputLayerIdx[ i ][ j ] = k
				OutputLayerIdInOls[ i ][ j++ ] = vps_layer_id[ k ]
				NumSubLayersInLayerInOLS[ i ][ j++ ] = vps_max_sub_layers_minus1 + 1
			}

if( !vps_all_independent_layers_flag ) {
			NumOutputLayersInOls[ i ] = j
			for( j = 0; j < NumOutputLayersInOls[ i ]; j++ ) {
				idx = OutputLayerIdx[ i ][ j ]
				if( !vps_independent_layer_flag[ idx ] )
					for( k = 0; k < NumRefLayers[ idx ]; k++ ) {
						layerIncludedInOlsFlag[ i ][ RefLayerIdx[ idx ][ k ] ] = 1
						if( NumSubLayersInLayerInOLS[ i ][ RefLayerIdx[ idx ][ k ] ] <
							       max_tid_il_ref_pics_plus1[ OutputLayerIdInOls[ i ][ j ] ] )
						    NumSubLayersInLayerInOLS[ i ][ RefLayerIdx[ idx ][ k ] ] = 
							       max_tid_il_ref_pics_plus1[ OutputLayerIdInOls[ i ][ j ] ]
					}
				}
			}

}
	}

# Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).