| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0120 |
| --- | --- |

| Title: | AHG9/AHG12: On mixed NAL unit types |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Byeongdoo Choi Stephan Wenger Shan Liu | Email: | bdchoi@tencent.com |
| Source: | Tencent |  |  |

_____________________________

# Abstract

This contribution proposes a simple way to deal with the CRA pictures and the associated leading pictures with mixed NAL unit types. To avoid a complicated subpicture-wise reference picture handling, we propose the derivation rule of HandleCraAsCvsStartFlag and NoOutputBeforeRecoveryFlag values, as follows:

- When mixed_nalu_types_in_pic_flag is equal to 1 and at least a VCL NAL unit of a picture has nal_unit_type equal to CRA_NUT,  HandleCraAsCvsStartFlag and NoOutputBeforeRecoveryFlag for the picture are both set equal to 0.

# Problem Statement

In the latest VVC spec. draft, when the value of mixed_nalu_types_in_pic_flag in PPS is equal to 1, the picture shall have at least two subpictures and VCL NAL units of the picture shall have two different nal_unit_type values. When the VCL NAL units of a subpicture have a NAL unit type equal to one among STSA_NUT, RADL_NUT, RASL_NUT, IDR_W_RADL, IDR_N_LP, and CRA_NUT, the other NAL unit type shall be equal to TRAIL_NUT, RADL_NUT, or RASL_NUT.

A problem is that it is not clear how to deal with the (leading) pictures of which NAL unit types are RASL_NUT + TRAIL_NUT or RASL_NUT + RADL_NUT. The leading pictures are associated with a picture of which a subpicture has nal_unit_type equal to CRA_NUT and the other subpicture has nal_unit_type equal to TRAIL_NUT, RADL_NUT, or RASL_NUT. Thus, a set of subpictures of the leading pictures may be associated with a set of subpictures of the picture with CRA_NUT. When the values of HandleCraAsCvsStartFlag and NoOutputBeforeRecoveryFlag are equal to 1, do we need to generate unavaiable reference subpictures of the leading picture with RASL NAL units? It is complicated.

# Proposal

In order to address the above issue, straightforwardly, we proposes the following derivation rule of HandleCraAsCvsStartFlag and NoOutputBeforeRecoveryFlag values.

- When mixed_nalu_types_in_pic_flag is equal to 1 and at least a VCL NAL unit of a picture has nal_unit_type equal to CRA_NUT,  HandleCraAsCvsStartFlag and NoOutputBeforeRecoveryFlag for the picture are both set equal to 0.

It assumes that a subpicture-wise random access is not allowed when two or more subpictures are merged into a coded bitstream. When a set of subpictures are extracted, it may have different random access period than the merged bitstream.

Proposed Text

For each IRAP picture in the bitstream, the following applies:

–	If the picture is the first picture of a layer in the bitstream in decoding order, an IDR picture, or the first picture of a layer that follows an EOS NAL unit of the layer in decoding order, the variable NoOutputBeforeRecoveryFlag for the picture is set equal to 1.

–	Otherwise, when the picture is a CRA picture, the following applies:

–	If some external means not specified in this Specification is available to set the variable HandleCraAsCvsStartFlag for the picture to a value , HandleCraAsCvsStartFlag for the picture is set equal to the value provided by the external means and NoOutputBeforeRecoveryFlag is set equal to HandleCraAsCvsStartFlag.

–	Otherwise, HandleCraAsCvsStartFlag and NoOutputBeforeRecoveryFlag are both set equal to 0 for the picture.

For each GDR picture in the bitstream, the following applies:

–	If the picture is the first picture of a layer in the bitstream in decoding order or the first picture of a layer that follows an EOS NAL unit of the layer in decoding order, the variable NoOutputBeforeRecoveryFlag for the picture is set equal to 1.

–	Otherwise, if some external means not specified in this Specification is available to set the variable HandleGdrAsCvsStartFlag for the picture to a value , HandleGdrAsCvsStartFlag is set equal to the value provided by the external means and NoOutputBeforeRecoveryFlag is set equal to HandleGdrAsCvsStartFlag for the picture.

–	Otherwise, HandleGdrAsCvsStartFlag and NoOutputBeforeRecoveryFlag for the picture are both set equal to 0.

When mixed_nalu_types_in_pic_flag is equal to 1 and at least a VCL NAL unit of a picture has nal_unit_type equal to CRA_NUT,  HandleCraAsCvsStartFlag and NoOutputBeforeRecoveryFlag for the picture are both set equal to 0.

# Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).