| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0126 |
| --- | --- |

| Title: | AHG9/AHG12: On signaling of subpicture ID |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Byeongdoo Choi Stephan Wenger Shan Liu | Email: | bdchoi@tencent.com |
| Source: | Tencent |  |  |

_____________________________

# Abstract

This contribution proposes a constraint that the value of subpic_treated_as_pic_flag[] shall be equal to 1, when the value of SubpicIdVal[] of the subpicture is changed from the previous picture. It is intended to guarantee that only independently coded subpictures can be relocated by subpicture ID remapping in PPS.

# Problem Statement

In the latest version of VVC spec. draft, subpicture ID may be explicitly signaled in SPS or PPS. When signaled in PPS, the mapping information subpicture IDs to subpictures can be updated, in picture-level. Accordingly, the subpictures (or subpictures’ contents) may be relocated in the coded picture. This feature was agreed to enable subpictures of interest to be contained in the coded picture, through the extraction or merge process, where the subpictures of interest may be changed over time, like viewport-dependent processing in VR360. Fig. 1 illustrates the case.

An issue is that only independently coded subpictures may be able to be relocated within the coded video sequence. In this contribution, to make sure that only subpictures that are treated as pictures can be relocated or reordered, a constraint is proposed as in the following section.

Fig. 1 Example of subpicture ID remapping

# Proposal

When the current picture is not the first picture in the CLVS, if the value of SubpicIdVal[] of the ith subpicture is changed from the previous picture, the value of subpic_treated_as_pic_flag[ i ] of the ith subpicture shall be equal to 1.

Proposed Text

pps_subpic_id[ i ] specifies the subpicture ID of the i-th subpicture. The length of the pps_subpic_id[ i ] syntax element is pps_subpic_id_len_minus1 + 1 bits.

The variable SubpicIdVal[ i ], for each value of i in the range of 0 to sps_num_subpics_minus1, inclusive, is derived as follows:

for( i = 0; i  <=  sps_num_subpics_minus1; i++ )
	if( subpic_id_mapping_explicitly_signalled_flag )
		SubpicIdVal[ i ] = subpic_id_mapping_in_pps_flag ? pps_subpic_id[ i ] : sps_subpic_id[ i ]	(80)
	else
		SubpicIdVal[ i ] = i

It is a requirement of bitstream conformance that all the following constraints apply:

–	For any two differenty values of i and j in the range of 0 to sps_num_subpics_minus1, inclusive, SubpicIdVal[ i ] shall not be equal to SubpicIdVal[ j ].

–	When the current picture is not the first picture of the CLVS, for each value of i in the range of 0 to sps_num_subpics_minus1, inclusive, if the value of SubpicIdVal[ i ] is not equal to the value of SubpicIdVal[ i ] of the previous picture in decoding order in the same layer, the nal_unit_type for all coded slice NAL units of the subpicture in the current picture with subpicture index i shall be equal to a particular value in the range of IDR_W_RADL to CRA_NUT, inclusive.

–	When the current picture is not the first picture of the CLVS, for each value of i in the range of 0 to sps_num_subpics_minus1, inclusive, if the value of SubpicIdVal[ i ] is not equal to the value of SubpicIdVal[ i ] of the previous picture in decoding order in the same layer, subpic_treated_as_pic_flag[ i ] shall be equal to 1.

# Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).