| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0131 |
| --- | --- |

| Title: | AHG2: On Chroma QT split in 4:2:2 format coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Han Huang, Wei-Jung Chien, Vadim Seregin, Marta Karczewicz (Qualcomm) Tzu-Der Chuang, Chia-Ming Tsai, Shih-Ta Hsiang, Chih-Wei Hsu, Ching-Yeh Chen, Yu-Wen Huang, Shaw-Min Lei (MediaTek) | Email: | {hanhuang, wchien, vseregin, martak}@qti.qualcomm.com {peter.chuang, chia-ming.tsai, shih-ta.hsiang, cw.hsu, chingyeh.chen, yuwen.huang, shawmin.lei} @mediatek.com |
| Source: | Qualcomm Inc. and MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution reports several issues that are caused by the adoption of JVET-Q0471 in JVET Q meeting. The issues include inconsistent meaning of minimum quad tree size in chroma tree among different color formats, inconsistent handling of bottom-right picture boundary among different color formats, mismatch between VTM-8.0 and VVC draft 8, and implementation bugs in the VTM-8.0. It’s also noted that the gain reported in JVET-Q0471 can be achieved by encoder configuration. This contribution proposes to revert JVET-Q0471.

# Problem Statement

In the contribution JVET-Q0471 that was adopted in JVET Q meeting, it’s stated that “there is a bug in chroma QT split checking, where the luma coding block size is checked against minimum chroma QT size, the equation used is (cbSize <= MinQtSizeC)”. Some performance drop for 4:2:2 coding was reported in non-CTC configurations.  Therefore, it’s further proposed to change the condition for chroma quad split to allow one more level of quad split specifically for 4:2:2 coding. The allowed quad split process was changed as following and is adopted in the current VVC draft 8.

The variable allowSplitQt is derived as follows:

- If one or more of the following conditions are true, allowSplitQt is set equal to FALSE:

- treeType is equal to SINGLE_TREE or DUAL_TREE_LUMA and cbSize is less than or equal to MinQtSizeY

- treeType is equal to DUAL_TREE_CHROMA and cbSize is less than or equal to ( MinQtSizeC * SubHeightC / SubWidthC )

- mttDepth is not equal to 0

- treeType is equal to DUAL_TREE_CHROMA and ( cbSize / SubWidthC ) is less than or equal to 4

- treeType is equal to DUAL_TREE_CHROMA and modeType is equal to MODE_TYPE_INTRA

- Otherwise, allowSplitQt is set equal to TRUE.

Wherein the subsampling ratio of chroma samples (SubHeightC / SubWidthC) is used to determine the scaling of allowed minimum quad split size in chroma tree.

The following issues are identified:

- Inconsistent meaning of minimum quad tree size in chroma tree among different color formats.   The actual minimum quad tree size in chroma tree in 4:2:2 color format is half of that in 4:2:0 and 4:4:4. The encoder configure of minimum quad tree size in chroma tree doesn’t reflect the actual size when coding in 4:2:2 format. For example, when the user want to stop the quad split when the CU width is equal to 16 chroma samples, the minimum quad tree size in chroma tree is set equal to 16 in 4:2:0 and 4:4:4 format. However, the minimum quad tree size in chroma tree should be set equal to 32 in encoder configure in 4:2:0 format because of the allowing one more level of quad split for 4:2:2 format.

- Cannot disable the quad split for chroma tree in 4:2:2 format when CTU size is equal to 64x64. In VVC draft 8, when coding in 4:2:0 and 4:4:4 format with CTU size equal to 64, one can disable quad split for chroma tree by setting the MinQtSizeC equal to CTU size, e.g. 64. In 4:2:2 format, due to allowing one more level of quad split, the MinQtSizeC shall be set to 128 to disable the quad split. However, in VVC draft 8, the maximum value of MinQtSizeC cannot be larger than the CTU size, which means the quad split cannot be disabled. (The MinQtSizeC is derived by (1<< MinQtLog2SizeC), and the MinQtLog2SizeC is derived by  MinCbLog2SizeY + ph_log2_diff_min_qt_min_cb_intra_slice_chroma. The value of ph_log2_diff_min_qt_min_cb_intra_slice_chroma is constrained in the range of 0 to CtbLog2SizeY − MinCbLog2SizeY, which means the maximun value of MinQtLog2SizeC is equal to CtbLog2SizeY.)

- Inconsistent handling of bottom-right picture boundary among different color formats. Considering the case when x0 + cbWidth is greater than pic_width_in_luma_samples and y0 + cbHeight is greater than pic_height_in_luma_samples and cbWidth is equal to MinQtSizeC in the chroma tree. In the 4:2:2 chroma format, binary split is allowed since cbWidth is not greater than minQtSize (refer to the highlighted part in Table 1), quad split is also allowed since cbSize (cbWidth) is greater than MinQtSizeC * SubHeightC / SubWidthC (which is MinQtSizeC/2).  But in the 4:2:0 format and 4:4:4 format, SubHeightC / SubWidthC is equal to 1, the condition of cbSize is equal to or less than MinQtSize is true, therefore quad tree split is not allowed. Also note that allowing both quad tree and binary tree split at the bottom-right picture boundary causes redundant split.

- Mismatch between VTM-8.0 and VVC draft 8. In VTM-8.0, when x0 + cbWidth is greater than pic_width_in_luma_samples and y0 + cbHeight is greater than pic_height_in_luma_samples and cbWidth is equal to MinQtSizeC in the chroma tree, quad split is derived as implicit split. But in the VVC draft 8, both horizontal binary split and quad split are allowed.

Table 1 Quoted specifications of "Allowed binary split process"

The variable allowBtSplit is derived as follows: - … - Otherwise, if all of the following conditions are true, allowBtSplit is set equal to FALSE - x0 + cbWidth is greater than pic_width_in_luma_samples - y0 + cbHeight is greater than pic_height_in_luma_samples - cbWidth is greater than minQtSize

It’s also noted that the VTM-7.0 “bug” reported in JVET-Q0471 may due to different interpretation of the configuration at encoder. The minimum quad tree size in chroma tree was interpreted as in chroma samples, thus the bug in the VTM-7.0. However, the encoder configuration of minimum quad tree size should be interpreted as in luma samples in VTM-7.0 (therefore, the bug is invalid). The performance improve of the proposed method in JVET-Q0471 compares to the “bug fix” is achieved by allowing one more level of quad split in a hard-coded way, which can be achieved equivalently by changing the encoder configuration. As a side evident, the reference/test in the bug fix result is identical to the test/reference in the proposed method’s result in JVET-Q0471.

# Solutions

To solve the above-mentioned issues, several solutions are studied.

In one solution, the VVC specifications can be aligned with VTM-8.0 to solve the mismatch and inconsistent handling of bottom-right picture boundary among different color formats. The text change of the corresponding condition in allowed binary split process is as follows:

- if all of the following conditions are true, allowBtSplit is set equal to FALSE

- x0 + cbWidth is greater than pic_width_in_luma_samples

- y0 + cbHeight is greater than pic_height_in_luma_samples

- cbWidth is greater than minQtSize and treeType is not equal to DUAL_TREE_CHROMA

- if all of the following conditions are true, allowBtSplit is set equal to FALSE

- x0 + cbWidth is greater than pic_width_in_luma_samples

- y0 + cbHeight is greater than pic_height_in_luma_samples

- cbWidth is greater than (minQtSize* SubHeightC / SubWidthC)  and treeType is equal to DUAL_TREE_CHROMA

Then all the relative input for cbSize in the specifications should be changed to cbHeight to avoid inconsistent meaning of minimum quad tree size in chroma tree among different color formats.

In another solution, it is proposed to revert JVET-Q0471. The allowed quad split process is simply changed as following. All four issues mentioned in Section-1 can be resolved.

The variable allowSplitQt is derived as follows:

- If one or more of the following conditions are true, allowSplitQt is set equal to FALSE:

- treeType is equal to SINGLE_TREE or DUAL_TREE_LUMA and cbSize is less than or equal to MinQtSizeY

- treeType is equal to DUAL_TREE_CHROMA and cbSize is less than or equal to ( MinQtSizeC * SubHeightC / SubWidthC )

- mttDepth is not equal to 0

- treeType is equal to DUAL_TREE_CHROMA and ( cbSize / SubWidthC ) is less than or equal to 4

- treeType is equal to DUAL_TREE_CHROMA and modeType is equal to MODE_TYPE_INTRA

- Otherwise, allowSplitQt is set equal to TRUE.

# Test results

In the VTM-8.0, the minimum QT size for luma component is mistakenly reduced by half in 4:2:2 coding and the minimum and maximum split depth derivation at encoder side is not properly modified according to the adoption of JVET-Q0471. A ticket (#991) was submitted to the VTM bug tracker to address this issue. Compares to VTM-8.0 under 4:2:2 common test conditions, the bug fix shows 0.00%, 0.00% 0.03% bd-rates differences in all intra test conditions, -0.01%, -0.07%, 0.02% in random access test conditions. In the following test, the VTM-8.0 with the bug fix is used as code base.  The reference is generated by setting MaxTTChromaISlice equals to 64 and MinQTChromaISlice equals to 32. The test is generated by turning JVET-Q0471 off and setting MaxTTChromaISlice equals to 64 and MinQTChromaISlice equals to 16. The bd-rate differences are 0.00%, 0.00%, 0.00% as shown in Table 3.

Table 2 Test results of bug fix

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | VTM8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Traffic | -0.01% | -0.02% | 0.05% | 99% | 95% |
| Kimono | 0.00% | 0.06% | -0.04% | 100% | 86% |
| EBUHorse | 0.01% | -0.01% | 0.07% | 98% | 103% |
| EBUGraphics | -0.01% | 0.03% | -0.06% | 96% | 94% |
| EBUWaterRocksClose | 0.00% | -0.02% | 0.04% | 97% | 96% |
| EBUKidsSoccer | 0.01% | -0.10% | 0.08% | 97% | 95% |
| Seeking | 0.02% | 0.03% | 0.05% | 97% | 87% |
| Overall | 0.00% | 0.00% | 0.03% | 98% | 95% |
|  | Random access Main10 |  |  |  |  |
|  | VTM8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Traffic | -0.07% | -0.13% | 0.39% | 96% | 92% |
| Kimono | 0.01% | -0.15% | -0.11% | 96% | 95% |
| EBUHorse | 0.00% | 0.05% | -0.02% | 96% | 90% |
| EBUGraphics | 0.00% | 0.02% | 0.10% | 94% | 95% |
| EBUWaterRocksClose | 0.00% | 0.03% | 0.08% | 92% | 91% |
| EBUKidsSoccer | -0.01% | -0.34% | -0.28% | 95% | 87% |
| Seeking | 0.03% | 0.00% | -0.01% | 95% | 93% |
| Overall | -0.01% | -0.07% | 0.02% | 96% | 94% |
|  | Low delay B Main10 |  |  |  |  |
|  | VTM8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Traffic |  |  |  |  |  |
| Kimono |  |  |  |  |  |
| EBUHorse |  |  |  |  |  |
| EBUGraphics |  |  |  |  |  |
| EBUWaterRocksClose |  |  |  |  |  |
| EBUKidsSoccer |  |  |  |  |  |
| Seeking |  |  |  |  |  |
| Overall |  |  |  |  |  |

Table 3 Test results of Q0471 off versus bug fix

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Q0471 fix |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Traffic | 0.00% | 0.00% | 0.00% | 100% | 104% |
| Kimono | 0.00% | 0.00% | 0.00% | 101% | 105% |
| EBUHorse | 0.00% | 0.00% | 0.00% | 102% | 105% |
| EBUGraphics | 0.00% | 0.00% | 0.00% | 103% | 110% |
| EBUWaterRocksClose | 0.00% | 0.00% | 0.00% | 103% | 106% |
| EBUKidsSoccer | 0.00% | 0.00% | 0.00% | 104% | 107% |
| Seeking | 0.00% | 0.00% | 0.00% | 103% | 116% |
| Overall | 0.00% | 0.00% | 0.00% | 101% | 105% |
|  | Random access Main10 |  |  |  |  |
|  | Q0471 fix |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Traffic | 0.00% | 0.00% | 0.00% | 107% | 110% |
| Kimono | 0.00% | 0.00% | 0.00% | 105% | 110% |
| EBUHorse | 0.00% | 0.00% | 0.00% | 104% | 112% |
| EBUGraphics | 0.00% | 0.00% | 0.00% | 104% | 110% |
| EBUWaterRocksClose | 0.00% | 0.00% | 0.00% | 106% | 110% |
| EBUKidsSoccer | 0.00% | 0.00% | 0.00% | 105% | 110% |
| Seeking | 0.00% | 0.00% | 0.00% | 104% | 109% |
| Overall | 0.00% | 0.00% | 0.00% | 104% | 107% |
|  | Low delay B Main10 |  |  |  |  |
|  | Q0471 fix |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Traffic |  |  |  |  |  |
| Kimono |  |  |  |  |  |
| EBUHorse |  |  |  |  |  |
| EBUGraphics |  |  |  |  |  |
| EBUWaterRocksClose |  |  |  |  |  |
| EBUKidsSoccer |  |  |  |  |  |
| Seeking |  |  |  |  |  |
| Overall |  |  |  |  |  |

# Conclusion

In this contribution, several issues that are caused by the adoption of JVET-Q0471 in JVET Q meeting are reported. Two solutions are provided to address the issues. It’s proposed to adopt solution 2 in the next version of VVC draft.

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).