| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0132 |
| --- | --- |

| Title: | AHG9: On signalling of chroma related APS |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Shunsuke Iwamura Shimpei Nemoto Atsuro Ichigaya | Tel: Email: | +81-3-5494-3353 iwamura.s-gc@nhk.or.jp |
| Source: | NHK |  |  |

_____________________________

- Abstract

This contribution proposes cleanup of chroma related adaptation parameter set (APS) signalling. In the current specification, in order to avoid redundant signalling, it is allowed not to send chroma related scaling list by signalling chroma_scaling_list_enabled_flag as proposed in JVET-Q0505. This could be beneficial when the input video is monochrome. However, there remains redundancy in APS when signalling chroma related adaptive loop filter (ALF) parameters. In this contribution, it is proposed to send chroma_aps_enabled_flag in APS syntax instead of signalling chroma_scaling_list_enabled_flag, and the proposed flag commonly controls the signalling of scaling list, ALF and CCALF parameters associated with chroma component.

Introduction

In the last JVET meeting, chroma_scaling_list_enabled_flag is newly introduced in APS, which allows not to signal scaling list associated with chroma component as proposed in JVET-Q0505. For monochrome video signals, redundant signalling for chroma scaling list can be avoid by this flag. However, there still exists redundancy in APS when signalling ALF and CCALF parameters for chroma component such as alf_chroma_filter_signal_flag, alf_cc_cb_filter_signal_flag, and alf_cc_cr_filter_signal_flag when video format is monochrome.

Proposed method and spec change

To avoid the redundant signalling, this contribution proposes to move chroma_scaling_list_enabled_flag from scaling data syntax to APS RBSP syntax and rename as chroma_aps_enabled_flag. The proposed flag commonly control chroma related APS parameters to avoid signalling chroma related APS parameters.

- 7.3.2.5 Adaptation parameter set RBSP syntax

| adaptation_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| adaptation_parameter_set_id | u(5) |
| aps_params_type | u(3) |
| if ( aps_params_type != LMCS_APS ) |  |
| chroma_aps_present_flag | u(1) |
| if( aps_params_type = = ALF_APS ) |  |
| alf_data( chroma_aps_present_flag ) |  |
| else if( aps_params_type = = LMCS_APS ) |  |
| lmcs_data( ) |  |
| else if( aps_params_type = = SCALING_APS ) |  |
| scaling_list_data( chroma_aps_present_flag ) |  |
| … |  |

- 7.3.2.19 Adaptive loop filter data syntax

| alf_data( chroma_aps_present_flag ) { | Descriptor |
| --- | --- |
| alf_luma_filter_signal_flag | u(1) |
| if ( chroma_aps_present_flag ) { |  |
| alf_chroma_filter_signal_flag | u(1) |
| alf_cc_cb_filter_signal_flag | u(1) |
| alf_cc_cr_filter_signal_flag | u(1) |
| } |  |
| … |  |

- 7.3.2.21 Scaling list data syntax

| scaling_list_data( chroma_aps_present_flag ) { | Descriptor |
| --- | --- |
| scaling_matrix_for_lfnst_disabled_flag | u(1) |
| scaling_list_chroma_present_flag | u(1) |
| for( id = 0; id < 28; id ++ ) |  |
| matrixSize = (id < 2 ) ? 2 : ( ( id < 8 ) ? 4 : 8 ) |  |
| if( scaling_list_chroma_present_flag chroma_aps_present_flag ¦ ¦ ( id % 3 = = 2 ) ¦ ¦ ( id = = 27 ) ) { |  |
| … |  |

- 7.4.3.5 Adaptation parameter set semantics

chroma_aps_present_flag equal to 1 specifies that APS parameters associated with chroma component are present in the APS. chroma_aps_present_flag equal to 0 specifies that APS parameters associated with chroma component are not present in the APS.

- 7.4.3.21 Scaling list data semantics

scaling_list_chroma_present_flag equal to 1 specifies that chroma scaling lists are present in scaling_list_data( ). scaling_list_chroma_present_flag equal to 0 specifies that chroma scaling lists are not present in scaling_list_data( ). It is a requirement of bitstream conformance that scaling_list_chroma_present_flag shall be equal to 0 when ChromaArrayType is equal to 0, and shall be equal to 1 when ChromaArrayType is not equal to 0.

Patent rights declaration(s)

NHK may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).