| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0134-v1 |
| --- | --- |

| Title: | AHG2: Mismatch related to deblocking of subblock motion edges. |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Brian Heng Minhua Zhou Wade Wan | Email: | firstname.lastname@broadcom.com |
| Source: | Broadcom Inc. |  |  |

_____________________________

# Abstract

This contribution asserts that there is a mismatch between the VVC draft text and VTM sotware related to deblocking of coding subblock boundaries. Within a subblock motion CU, the length of the deblocking filter used depends on the distance the nearest transform edge. However, when this neighboring transform edge aligns with a virtual boundary, the behavior of the text and software differ.

Specifically, the VTM software treats the neighboring transform edge as a transform edge, regardless of whether it aligns with a virtual boundary or not. While the VVC draft text ignores the neighboring transform edge altogether if it aligns with a virtual boundary. This difference will cause the text and software to use different filter lengths for subblock motion edges, and therefore they will produce different results.

This contribution proposes to modify the text to match the software behavior to resolve this mismatch. Proposed text changes are provided.

# Introduction

As defined in Section 8.8.3.4 of the VVC draft text, the deblocking filter uses a filter length of 2 when the current edge is 8 pixels away from a neighboring transform edge. However, when this neighboring transform edge aligns with a virtual boundary, the behavior of the text and software differ. Specific details are provided in the following sections.

# Current Draft Text

## Marking of transform edges

For deblocking filtering purposes, the locations of transform edges are marked in Section 8.8.3.3 of the VVC draft text. For example, the following text is used for recording vertical transform block edges:

- The value of edgeFlags[ x ][ y ] is derived as follows:

    - If VirtualBoundariesPresentFlag equal to 1 and ( xCb + x ) is equal to VirtualBoundariesPosX[ n ] for any n = 0..NumVerVirtualBoundaries − 1, edgeFlags[ x ][ y ] is set equal to 0.

    - Otherwise, if x is equal to 0, edgeFlags[ x ][ y ] is set equal to filterEdgeFlag.

    - Otherwise, if the location ( xCb + x , yCb + y ) is at a transform block edge, edgeFlags[ x ][ y ] is set equal to 1.

As highlighted above, note that the transform edge is ignored (edgeFlag set equal to 0) if it aligns with a virtual boundary. This disables filtering for the virtual boundary (as intended), but it also “erases” the transform edge so it will no longer be considered a transform edge at all in future sections.

## Maximum filter length for subblock boundaries

In the VVC draft text, Section 8.8.3.4, the deblocking filter length for an edge within a subblock motion CU (affine / SBTMVP) is determined based on the distance from the current edge to the nearest transform edge. Specifically, if the current edge is 8-pixels away from a neighboring transform edge, the filter length is reduced to 2.

For example, for a vertical edge this is specified in Section 8.8.3.4 with the following text:

- Otherwise, if one or more of the following conditions are true:

- xEdge is equal to 1,

- xEdge is equal to ( nCbW / 8 ) − 1,

- edgeTbFlags[ x − sbW ][ y ] is equal to 1,

- edgeTbFlags[ x + sbW ][ y ] is equal to 1,

the following applies:

maxFilterLengthPs[ x ][ y ] = 2		(1255)

maxFilterLengthQs[ x ][ y ] = 2		(1256)

The last two conditions above cover the general “8-pixels away from a transform edge” cases:

- edgeTbFlags[ x − sbW ][ y ] is equal to 1,

- edgeTbFlags[ x + sbW ][ y ] is equal to 1,

And, the first two conditions cover the edges 8-pixels away from the CU edge:

- xEdge is equal to 1,

- xEdge is equal to ( nCbW / 8 ) − 1,

Like virtual boundaries, disabled slice/tile/subpicture boundaries also “erase” transform edges in Section 8.8.3.3. But unlike virtual boundaries, the two conditions above ensure that the CU edge is always treated as a transform edge regardless of whether the transform edge has been erased or not (slice/tile/subpicture edges are always CU aligned, but virtual boundaries are not). Therefore, the current behavior for virtual boundaries is an inconsistent in this respect.

# Mismatch between Text and Software

As described above, transform edges that align with virtual boundaries are not marked as transform edges in Section 8.8.3.3. Then, in Section 8.8.3.4, those “erased” edges are not considered transform edges for the purposes of subblock filter length derivation.

In contrast, the software currently treats transform edges as transform edges (for subblock filter length derivation), regardless of whether they align with a virtual boundary or not.

# Proposed Solution

This contribution asserts that the virtual boundary behavior should be consistent with the behavior defined for slice/tile/subpic boundaries for the purposes of deriving subblock filter lengths. Specifically, virtual boundaries should not have an impact on calculations regarding the distance to the nearest transform edge for subblock boundary filter length derivation.

To this end, the current contribution proposes implementing virtual boundary processing for the deblocking filter in a later section, specifically Section 8.8.3.5 (Derivation process of boundary filtering strength).

In particular, the behavior to disable filtering across virtual boundaries could be specified with the following highlighted text:

For xDi with i = 0..xN and yDj with j = 0..yN, the following applies:

- If edgeFlags[ xDi ][ yDj ] is equal to 0, the variable bS[ xDi ][ yDj ] is set equal to 0.

- Otherwise, if edgeType is equal to EDGE_VER, VirtualBoundariesPresentFlag equal to 1, and ( xCb + xDi ) is equal to VirtualBoundariesPosX[ n ] for any n = 0..NumVerVirtualBoundaries - 1, the variable bS[ xDi ][ yDj ] is set equal to 0.

- Otherwise, if edgeType is equal to EDGE_HOR, VirtualBoundariesPresentFlag equal to 1, and ( yCb + yDj ) is equal to VirtualBoundariesPosY[ n ] for any n = 0..NumHorVirtualBoundaries - 1, the variable bS[ xDi ][ yDj ] is set equal to 0.

- Otherwise…

By moving virtual boundary processing to this point, all the existing text for virtual boundary deblocking filtering could then be removed. Detailed text changes are provided in the attached document.

# Patent rights declaration(s)

Broadcom Inc. does not have current or pending patent rights relating to the technology described in this contribution.