| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0135 |
| --- | --- |

| Title: | AHG12: On subpicture layout signalling |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Mitsuru Katsumata Mitsuhiro Hirabayashi Teruhiko Suzuki 2-10-1 Osaki Shinagawa-ku, Tokyo, 141-8610, Japan | Tel: Email: | +81-50-3750-7393 Mitsuru.Katsumata@sony.com Mitsuhiro.Hirabayashi@sony.com Teruhiko.S@sony.com |
| Source: | Sony Corporation |  |  |

_____________________________

- Abstract

This contribution proposes that in addition to the current subpicture layout in the unit of CtbSizeY, subpicture layout can be signalled in integer multiples units of CtbSizeY for saving bits.

Option1:  signal subpic_unit_num_ctus_minus1 syntax element that is the number of CtbSizeY in the subpicture layout.  All syntax elements of subpicture layout use the same in units.

Option2: signal subpic_unit_num_ctus_x_minus1 and subpic_unit_num_ctus_y_minus1 syntax elements that are the number of CtbSizeY in the subpicture layout.  The first syntax element indicate the units for the subpic_ctu_top_left_x[ i ] and subpic_width_minus1[ i ], and the second for the subpic_ctu_top_left_y[ i ] and subpic_heihgt_minus1[ i ].

Option3: combine the Option1 with Option2 and two options can be selected.

Introduction

VVC specification draft in JVET-Q2001-vE includes support for subpicture layout signalling.  The signalling specifies from four syntax elements, below.

subpic_ctu_top_left_x[ i ] specifies horizontal position of top left CTU of i-th subpicture in unit of CtbSizeY. The length of the syntax element is Ceil( Log2( ( pic_width_max_in_luma_samples + CtbSizeY  − 1 )  >>  CtbLog2SizeY ) ) bits.

subpic_ctu_top_left_y[ i ] specifies vertical position of top left CTU of i-th subpicture in unit of CtbSizeY. The length of the syntax element is Ceil( Log2( ( pic_height_max_in_luma_samples + CtbSizeY − 1 )  >>  CtbLog2SizeY ) ) bits.

subpic_width_minus1[ i ] plus 1 specifies the width of the i-th subpicture in units of CtbSizeY. The length of the syntax element is Ceil( Log2( ( pic_width_max_in_luma_samples + CtbSizeY − 1 )  >>  CtbLog2SizeY ) ) bits.

subpic_height_minus1[ i ] plus 1 specifies the height of the i-th subpicture in units of CtbSizeY. The length of the syntax element is Ceil( Log2( ( pic_height_max_in_luma_samples + CtbSizeY − 1 )  >>  CtbLog2SizeY ) ) bits.

These syntax elements are all in unit of CtbSizeY because the subpicture is split into the slices.

There are use cases that picture consists of subpictures with one or two resolutions.  For example, the subpicture layout that each subpicture represents a cubemap face for a 360 cubemap projection has only one resolution.  In other use case, the subpicture layout in Figure 7 of the VVC specification (Fig. 1) has two resolutions, and a large subpicture is twice a small subpicture.  It is assumed that such subpicture layouts are often used.

Fig. 1 Figure 7 in VVC specification

A picture that is partitioned into 18 tiles, 24 slices and 24 subpictures (informative)

In these use cases, subpictures layout can be expressed in multiples units of CtbSizeY.  When using multiples units of CtbSizeY, it can save the bit length of the four syntax elements of the subpicture layout.

Proposal

This contribution proposes that in addition to the current subpicture layout in the unit of CtbSizeY, subpicture layout can be signalled in integer multiples units of CtbSizeY for saving bits.

Option1:  signal subpic_unit_num_ctus_minus1 syntax element that is the number of CtbSizeY in the subpicture layout.  All syntax elements of subpicture layout use the same in units.

Option2: signal subpic_unit_num_ctus_x_minus1 and subpic_unit_num_ctus_y_minus1 syntax elements that are the number of CtbSizeY in the subpicture layout.  The first syntax element indicate the units for the subpic_ctu_top_left_x[ i ] and subpic_width_minus1[ i ], and the second for the subpic_ctu_top_left_y[ i ] and subpic_heihgt_minus1[ i ].

Option3: combine the Option1 with Option2 and two options can be selected.

When subpicture layout use in unit of one CtbSizeY, the same as the current VVC draft, each option increase only 1 bit than current VVC signalling.

The proposed syntax and semantics are marked in yellow with removed text in red strike through on top of the current VVC draft (JVET-Q2001-vE)

Option1:

It is proposed to signal subpic_unit_num_ctus_minus1 syntax element that is the number of CtbSizeY in the subpicture layout.  All syntax elements of subpicture layout use the same in units.

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … | u(4) |
| sps_log2_ctu_size_minus5 | u(2) |
| subpic_info_present_flag | u(1) |
| if( subpic_info_present_flag ) { |  |
| sps_num_subpics_minus1 | ue(v) |
| sps_independent_subpics_flag | u(1) |
| subpic_unit_num_ctus_present_flag | u(1) |
| if(subpic_unit_num_ctus_present_flag) |  |
| subpic_unit_num_ctus_minus1 | ue(v) |
| for( i = 0; sps_num_subpics_minus1 > 0 && i <= sps_num_subpics_minus1; i++ ) { |  |
| if( i > 0 && pic_width_max_in_luma_samples > CtbSizeY ) |  |
| subpic_ctu_top_left_x[ i ] | u(v) |
| if( i > 0 && pic_height_max_in_luma_samples > CtbSizeY ) { |  |
| subpic_ctu_top_left_y[ i ] | u(v) |
| if( i < sps_num_subpics_minus1 && pic_width_max_in_luma_samples > CtbSizeY ) |  |
| subpic_width_minus1[ i ] | u(v) |
| if( i < sps_num_subpics_minus1 && pic_height_max_in_luma_samples > CtbSizeY ) |  |
| subpic_height_minus1[ i ] | u(v) |
| if( !sps_independent_subpics_flag) { |  |
| subpic_treated_as_pic_flag[ i ] | u(1) |
| loop_filter_across_subpic_enabled_flag[ i ] | u(1) |
| } |  |
| } |  |
| … |  |

subpic_unit_num_ctus_present_flag equal to 1 specifies that the subpic_unit_using_multti_ctu_minus1 syntax element is present. subpic_unit_num_ctus_present_flag equal to 0 specifies that the subpic_unit_using_multti_ctu_minus1 syntax element is not present. When not present, the value of subpic_unit_num_ctus_present_flag is inferred to be equal to 0.

subpic_unit_num_ctus_minus1 is the value to determine a unit of the subpic_ctu_top_left_x[ i ], subpic_ctu_top_left_y[ i ], subpic_width_minus1[ i ], and subpic_height_minus1[ i ].  The unit of these syntax elements is (subpic_unit_num_ctus_minus1 +1) * CtbSizeY.  When not present, the value of subpic_unit_num_ctus_minus1 is inferred to be equal to 0.

The variables SubpicUnitCtus are derived as follows:

SubpicUnitCtus = ( subpic_unit_num_ctus_minus1 +1 ) * CtbSizeY	(xx)

subpic_ctu_top_left_x[ i ] specifies horizontal position of top left CTU of i-th subpicture in unit of SubpicUnitCtus CtbSizeY. The length of the syntax element is Ceil( Log2( ( pic_width_max_in_luma_samples + CtbSizeY  − 1 )  >>  CtbLog2SizeY ) ) Ceil( Log2( (pic_width_max_in_luma_samples + SubpicUnitCtus − 1)  >>  Log2(SubpicUnitCtus ) ) ) bits. When not present, the value of subpic_ctu_top_left_x[ i ] is inferred to be equal to 0.

subpic_ctu_top_left_y[ i ] specifies vertical position of top left CTU of i-th subpicture in unit of SubpicUnitCtus CtbSizeY. The length of the syntax element is Ceil( Log2( ( pic_height_max_in_luma_samples + CtbSizeY − 1 )  >>  CtbLog2SizeY ) ) Ceil( Log2( (pic_width_height_in_luma_samples + SubpicUnitCtus − 1)  >>  Log2(SubpicUnitCtus ) ) ) its. When not present, the value of subpic_ctu_top_left_y[ i ] is inferred to be equal to 0.

subpic_width_minus1[ i ] plus 1 specifies the width of the i-th subpicture in units of SubpicUnitCtus CtbSizeY. The length of the syntax element is Ceil( Log2( ( pic_width_max_in_luma_samples + CtbSizeY − 1 )  >>  CtbLog2SizeY ) ) Ceil( Log2( (pic_width_max_in_luma_samples + SubpicUnitCtus − 1)  >>  Log2(SubpicUnitCtus ) ) ) bits. When not present, the value of subpic_width_minus1[ i ] is inferred to be equal to ( ( pic_width_max_in_luma_samples + CtbSizeY− 1 )  >>  CtbLog2SizeY ) − subpic_ctu_top_left_x[ i ] − 1 ( (pic_width_max_in_luma_samples + SubpicUnitCtus − 1)  >>  Log2(SubpicUnitCtus ) ) - subpic_ctu_top_left_x[ i ] - 1.

subpic_height_minus1[ i ] plus 1 specifies the height of the i-th subpicture in units of SubpicUnitCtus CtbSizeY. The length of the syntax element is Ceil( Log2( ( pic_height_max_in_luma_samples + CtbSizeY − 1 )  >>  CtbLog2SizeY ) ) Ceil( Log2( (pic_width_height_in_luma_samples + SubpicUnitCtus − 1)  >>  Log2(SubpicUnitCtus ) ) ) bits. When not present, the value of subpic_height_minus1[ i ] is inferred to be equal to ( ( pic_height_max_in_luma_samples + CtbSizeY− 1 )  >>  CtbLog2SizeY ) − subpic_ctu_top_left_y[ i ] − 1 ( (pic_height_max_in_luma_samples + SubpicUnitCtus − 1)  >>  Log2(SubpicUnitCtus ) ) - subpic_ctu_top_left_y[ i ] - 1.

6.5.1

for( i = 0; i  <=  sps_num_subpics_minus1; i++ ) {
	leftX = subpic_ctu_top_left_x[ i ] * (subpic_unit_num_ctus_minus1+1)
	rightX = leftX + subpic_width_minus1[ i ] * (subpic_unit_num_ctus_minus1 +1)
	SubpicWidthInTiles[ i ] = ctbToTileColIdx[ rightX ] + 1 − ctbToTileColIdx[ leftX ]	(29)
	topY = subpic_ctu_top_left_y[ i ] * (subpic_unit_num_ctus_minus1 +1)
	bottomY = topY + subpic_height_minus1[ i ] * (subpic_unit_num_ctus_minus1 +1)
	SubpicHeightInTiles[ i ] = ctbToTileRowIdx[ bottomY ] + 1 − ctbToTileRowIdx[ topY ]
	if( SubpicHeightInTiles[ i ]  = =  1  &&
			subpic_height_minus1[ i ] + 1 < 	RowHeight[ ctbToTileRowIdx[ topY ] ] )
		subpicHeightLessThanOneTileFlag[ i ] = 1
else
		subpicHeightLessThanOneTileFlag[ i ] = 0
}

if( single_slice_per_subpic_flag ) {
	for( i = 0; i  <=   sps_num_subpics_minus1; i++ ) {
		NumCtusInSlice[ i ] = 0 
		if( subpicHeightLessThanOneTileFlag[ i ] ) /* The slice consists of a number of CTU rows in a tile. */
			AddCtbsToSlice( i, subpic_ctu_top_left_x[ i ] * (subpic_unit_num_ctus_minus1 +1),
				(subpic_ctu_top_left_x[ i ] + subpic_width_minus1[ i ] + 1) * (subpic_unit_num_ctus_minus1 +1), subpic_ctu_top_left_y[ i ] * (subpic_unit_num_ctus_minus1 +1),
				(subpic_ctu_top_left_y[ i ] + subpic_height_minus1[ i ] + 1) * (subpic_unit_num_ctus_minus1 +1))
		else { /* The slice consists of a number of complete tiles covering a rectangular region. */
			tileX = CtbToTileColBd[ subpic_ctu_top_left_x[ i ] * (subpic_unit_num_ctus_minus1 +1)]
			tileY = CtbToTileRowBd[ subpic_ctu_top_left_y[ i ] * (subpic_unit_num_ctus_minus1 +1)]
			for( j = 0; j < SubpicHeightInTiles[ i ]; j++ )
				for( k = 0; k < SubpicWidthInTiles[ i ]; k++ )
					AddCtbsToSlice( i, tileColBd[ tileX + k ], tileColBd[ tileX + k + 1 ], tileRowBd[ tileY + j ],
							tileRowBd[ tileY + j + 1 ] )
		}
	}
} else {
…

for( i = 0; i  <=  sps_num_subpics_minus1; i++ ) {
	NumSlicesInSubpic[ i ] = 0
	for( j = 0; j  <=  num_slices_in_pic_minus1; j++ ) {
		posX = CtbAddrInSlice[ j ][ 0 ] % PicWidthInCtbsY
		posY = CtbAddrInSlice[ j ][ 0 ] / PicWidthInCtbsY
		if( ( posX  >=  subpic_ctu_top_left_x[ i ] * (subpic_unit_num_ctus_minus1 +1))  &&	(32)
				( posX < (subpic_ctu_top_left_x[ i ] + subpic_width_minus1[ i ] + 1) * (subpic_unit_num_ctus_minus1 +1))  &&
				( posY  >=  subpic_ctu_top_left_y[ i ] * (subpic_unit_num_ctus_minus1 +1))  &&
				( posY < (subpic_ctu_top_left_y[ i ] + subpic_height_minus1[ i ] + 1 ) * (subpic_unit_num_ctus_minus1 +1) ) )
			NumSlicesInSubpic[ i ]++
}

7.4.3.14

The variable subpicSizeInMinCbsY for the subpicture with subpicture index subpicIdxA is derived to be equal to ( ( subpic_width_minus1[ subpicIdxA ] + 1) *SubpicUnitCtus CtbSizeY / MinCbSizeY * ( subpic_height_minus1[ subpicIdxA ] + 1) * SubpicUnitCtus CtbSizeY / MinCbSizeY ).

7.4.8.1

The variables SubpicLeftBoundaryPos, SubpicTopBoundaryPos, SubpicRightBoundaryPos, and SubpicBotBoundaryPos are derived as follows:

if( subpic_treated_as_pic_flag[ CurrSubpicIdx ] ) {
	SubpicLeftBoundaryPos = subpic_ctu_top_left_x[ CurrSubpicIdx ] * SubpicUnitCtus CtbSizeY
	SubpicRightBoundaryPos = Min( pic_width_max_in_luma_samples − 1,
			( subpic_ctu_top_left_x[ CurrSubpicIdx ] +
			subpic_width_minus1[ CurrSubpicIdx ] + 1 ) * SubpicUnitCtus CtbSizeY − 1 )
	SubpicTopBoundaryPos = subpic_ctu_top_left_y[ CurrSubpicIdx ] * SubpicUnitCtus CtbSizeY	(117)
	SubpicBotBoundaryPos = Min( pic_height_max_in_luma_samples − 1,
			( subpic_ctu_top_left_y[ CurrSubpicIdx ] +
			subpic_height_minus1[ CurrSubpicIdx ] + 1 ) * SubpicUnitCtus CtbSizeY − 1 )
}

D.7.2

The variable SubpicSizeY[ j ] is set equal to ( subpic_width_minus1[ j ] + 1 ) *  SubpicUnitCtus CtbSizeY * ( subpic_height_minus1[ j ] + 1 ) *  SubpicUnitCtus CtbSizeY .

Option2:

It is proposed to signal subpic_unit_num_ctus_x_minus1 and subpic_unit_num_ctus_y_minus1 syntax elements that are the number of CtbSizeY in the subpicture layout.  The first syntax element indicate the units for the subpic_ctu_top_left_x[ i ] and subpic_width_minus1[ i ], and the second for the subpic_ctu_top_left_y[ i ] and subpic_heihgt_minus1[ i ].

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … | u(4) |
| sps_log2_ctu_size_minus5 | u(2) |
| subpic_info_present_flag | u(1) |
| if( subpic_info_present_flag ) { |  |
| sps_num_subpics_minus1 | ue(v) |
| sps_independent_subpics_flag | u(1) |
| subpic_unit_num_ctus_present_flag | u(1) |
| if(subpic_unit_num_ctus_present_flag){ |  |
| subpic_unit_num_ctus_x_minus1 | ue(v) |
| subpic_unit_num_ctus_y_minus1 | ue(v) |
| } |  |
| for( i = 0; sps_num_subpics_minus1 > 0 && i <= sps_num_subpics_minus1; i++ ) { |  |
| if( i > 0 && pic_width_max_in_luma_samples > CtbSizeY ) |  |
| subpic_ctu_top_left_x[ i ] | u(v) |
| if( i > 0 && pic_height_max_in_luma_samples > CtbSizeY ) { |  |
| subpic_ctu_top_left_y[ i ] | u(v) |
| if( i < sps_num_subpics_minus1 && pic_width_max_in_luma_samples > CtbSizeY ) |  |
| subpic_width_minus1[ i ] | u(v) |
| if( i < sps_num_subpics_minus1 && pic_height_max_in_luma_samples > CtbSizeY ) |  |
| subpic_height_minus1[ i ] | u(v) |
| if( !sps_independent_subpics_flag) { |  |
| subpic_treated_as_pic_flag[ i ] | u(1) |
| loop_filter_across_subpic_enabled_flag[ i ] | u(1) |
| } |  |
| } |  |
| … |  |

subpic_unit_num_ctus_present_flag equal to 1 specifies that the subpic_unit_num_ctus_x_minus1 and subpic_unit_num_ctus_y_minus1 syntax element are present. subpic_unit_num_ctus_present_flag equal to 0 specifies that the subpic_unit_num_ctus_x_minus1 and subpic_unit_num_ctus_y_minus1 syntax element are not present. When not present, the value of subpic_unit_num_ctus_present_flag is inferred to be equal to 0.

subpic_unit_num_ctus_x_minus1 is the value to determine a unit of the subpic_ctu_top_left_x[ i ] and subpic_width_minus1[ i ].  The unit of these syntax elements is (subpic_unit_num_ctus_x_minus1 +1) * CtbSizeY.  When not present, the value of subpic_unit_num_ctus_x_minus1 is inferred to be equal to 0.

The variables SubpicUnitCtusX are derived as follows:

SubpicUnitCtusX = ( subpic_unit_num_ctus_x_minus1 +1 ) * CtbSizeY	(xx)

subpic_unit_num_ctus_y_minus1 is the value to determine a unit of the subpic_ctu_top_left_y[ i ] and subpic_height_minus1[ i ].  The unit of these syntax elements is (subpic_unit_num_ctus_y_minus1 +1) * CtbSizeY.  When not present, the value of subpic_unit_num_ctus_y_minus1 is inferred to be equal to 0.

The variables SubpicUnitCtusY are derived as follows:

SubpicUnitCtusY = ( subpic_unit_num_ctus_y_minus1 +1 ) * CtbSizeY	(xx)

subpic_ctu_top_left_x[ i ] specifies horizontal position of top left CTU of i-th subpicture in unit of SubpicUnitCtusX CtbSizeY. The length of the syntax element is Ceil( Log2( ( pic_width_max_in_luma_samples + CtbSizeY  − 1 )  >>  CtbLog2SizeY ) ) Ceil( Log2( (pic_width_max_in_luma_samples + SubpicUnitCtusX − 1)  >>  Log2(SubpicUnitCtusX ) ) ) bits. When not present, the value of subpic_ctu_top_left_x[ i ] is inferred to be equal to 0.

subpic_ctu_top_left_y[ i ] specifies vertical position of top left CTU of i-th subpicture in unit of SubpicUnitCtusY CtbSizeY. The length of the syntax element is Ceil( Log2( ( pic_height_max_in_luma_samples + CtbSizeY − 1 )  >>  CtbLog2SizeY ) ) Ceil( Log2( (pic_height_max_in_luma_samples + SubpicUnitCtusY − 1)  >>  Log2(SubpicUnitCtusY ) ) )bits. When not present, the value of subpic_ctu_top_left_y[ i ] is inferred to be equal to 0.

subpic_width_minus1[ i ] plus 1 specifies the width of the i-th subpicture in units of SubpicUnitCtusX CtbSizeY. The length of the syntax element is Ceil( Log2( ( pic_width_max_in_luma_samples + CtbSizeY − 1 )  >>  CtbLog2SizeY ) ) Ceil( Log2( (pic_width_max_in_luma_samples + SubpicUnitCtusX − 1)  >>  Log2(SubpicUnitCtusX ) ) ) bits. When not present, the value of subpic_width_minus1[ i ] is inferred to be equal to ( ( pic_width_max_in_luma_samples + CtbSizeY− 1 )  >>  CtbLog2SizeY ) − subpic_ctu_top_left_x[ i ] − 1 ( (pic_width_max_in_luma_samples + SubpicUnitCtusX − 1)  >>  Log2(SubpicUnitCtusX ) ) - subpic_ctu_top_left_x[ i ] - 1.

subpic_height_minus1[ i ] plus 1 specifies the height of the i-th subpicture in units of SubpicUnitCtus CtbSizeY. The length of the syntax element is Ceil( Log2( ( pic_height_max_in_luma_samples + CtbSizeY − 1 )  >>  CtbLog2SizeY ) ) Ceil( Log2( (pic_width_height_in_luma_samples + SubpicUnitCtusY − 1)  >>  Log2(SubpicUnitCtusY ) ) )  bits. When not present, the value of subpic_height_minus1[ i ] is inferred to be equal to ( ( pic_height_max_in_luma_samples + CtbSizeY− 1 )  >>  CtbLog2SizeY ) − subpic_ctu_top_left_y[ i ] − 1 ( (pic_width_height_in_luma_samples + SubpicUnitCtusY − 1)  >>  Log2(SubpicUnitCtusY ) ) - subpic_ctu_top_left_y[ i ] - 1.

6.5.1

for( i = 0; i  <=  sps_num_subpics_minus1; i++ ) {
	leftX = subpic_ctu_top_left_x[ i ] * (subpic_unit_num_ctus_x_minus1+1)
	rightX = leftX + subpic_width_minus1[ i ] * (subpic_unit_num_ctus_x_minus1 +1)
	SubpicWidthInTiles[ i ] = ctbToTileColIdx[ rightX ] + 1 − ctbToTileColIdx[ leftX ]	(29)
	topY = subpic_ctu_top_left_y[ i ] * (subpic_unit_num_ctus_y_minus1 +1)
	bottomY = topY + subpic_height_minus1[ i ] * (subpic_unit_num_ctus_y_minus1 +1)
	SubpicHeightInTiles[ i ] = ctbToTileRowIdx[ bottomY ] + 1 − ctbToTileRowIdx[ topY ]
	if( SubpicHeightInTiles[ i ]  = =  1  &&
			subpic_height_minus1[ i ] + 1 < 	RowHeight[ ctbToTileRowIdx[ topY ] ] )
		subpicHeightLessThanOneTileFlag[ i ] = 1
else
		subpicHeightLessThanOneTileFlag[ i ] = 0
}

if( single_slice_per_subpic_flag ) {
	for( i = 0; i  <=   sps_num_subpics_minus1; i++ ) {
		NumCtusInSlice[ i ] = 0 
		if( subpicHeightLessThanOneTileFlag[ i ] ) /* The slice consists of a number of CTU rows in a tile. */
			AddCtbsToSlice( i, subpic_ctu_top_left_x[ i ] * (subpic_unit_num_ctus_x_minus1 +1),
				(subpic_ctu_top_left_x[ i ] + subpic_width_minus1[ i ] + 1) * (subpic_unit_num_ctus_x_minus1 +1), subpic_ctu_top_left_y[ i ] * (subpic_unit_num_ctus_y_minus1 +1),
				(subpic_ctu_top_left_y[ i ] + subpic_height_minus1[ i ] + 1) * (subpic_unit_num_ctus_y_minus1 +1))
		else { /* The slice consists of a number of complete tiles covering a rectangular region. */
			tileX = CtbToTileColBd[ subpic_ctu_top_left_x[ i ] * (subpic_unit_num_ctus_x_minus1 +1),]
			tileY = CtbToTileRowBd[ subpic_ctu_top_left_y[ i ] * (subpic_unit_num_ctus_y_minus1 +1),]
			for( j = 0; j < SubpicHeightInTiles[ i ]; j++ )
				for( k = 0; k < SubpicWidthInTiles[ i ]; k++ )
					AddCtbsToSlice( i, tileColBd[ tileX + k ], tileColBd[ tileX + k + 1 ], tileRowBd[ tileY + j ],
							tileRowBd[ tileY + j + 1 ] )
		}
	}
} else {

for( i = 0; i  <=  sps_num_subpics_minus1; i++ ) {
	NumSlicesInSubpic[ i ] = 0
	for( j = 0; j  <=  num_slices_in_pic_minus1; j++ ) {
		posX = CtbAddrInSlice[ j ][ 0 ] % PicWidthInCtbsY
		posY = CtbAddrInSlice[ j ][ 0 ] / PicWidthInCtbsY
		if( ( posX  >=  subpic_ctu_top_left_x[ i ] * (subpic_unit_num_ctus_x_minus1 +1))  &&	(32)
				( posX < (subpic_ctu_top_left_x[ i ] + subpic_width_minus1[ i ] + 1) * (subpic_unit_num_ctus_x_minus1 +1))  &&
				( posY  >=  subpic_ctu_top_left_y[ i ] * (subpic_unit_num_ctus_y_minus1 +1))  &&
				( posY < (subpic_ctu_top_left_y[ i ] + subpic_height_minus1[ i ] + 1 ) * (subpic_unit_num_ctus_y_minus1 +1) ) )
			NumSlicesInSubpic[ i ]++
}

7.4.3.14

The variable subpicSizeInMinCbsY for the subpicture with subpicture index subpicIdxA is derived to be equal to ( ( subpic_width_minus1[ subpicIdxA ] + 1) *SubpicUnitCtusX CtbSizeY / MinCbSizeY * ( subpic_height_minus1[ subpicIdxA ] + 1) * SubpicUnitCtusY CtbSizeY / MinCbSizeY ).

7.4.8.1

The variables SubpicLeftBoundaryPos, SubpicTopBoundaryPos, SubpicRightBoundaryPos, and SubpicBotBoundaryPos are derived as follows:

if( subpic_treated_as_pic_flag[ CurrSubpicIdx ] ) {
	SubpicLeftBoundaryPos = subpic_ctu_top_left_x[ CurrSubpicIdx ] * SubpicUnitCtusX CtbSizeY
	SubpicRightBoundaryPos = Min( pic_width_max_in_luma_samples − 1,
			( subpic_ctu_top_left_x[ CurrSubpicIdx ] +
			subpic_width_minus1[ CurrSubpicIdx ] + 1 ) * SubpicUnitCtusX CtbSizeY − 1 )
	SubpicTopBoundaryPos = subpic_ctu_top_left_y[ CurrSubpicIdx ] * SubpicUnitCtusY CtbSizeY	(117)
	SubpicBotBoundaryPos = Min( pic_height_max_in_luma_samples − 1,
			( subpic_ctu_top_left_y[ CurrSubpicIdx ] +
			subpic_height_minus1[ CurrSubpicIdx ] + 1 ) * SubpicUnitCtusY CtbSizeY − 1 )
}

D.7.2

The variable SubpicSizeY[ j ] is set equal to ( subpic_width_minus1[ j ] + 1 ) *  SubpicUnitCtusX CtbSizeY * ( subpic_height_minus1[ j ] + 1 ) *  SubpicUnitCtusY CtbSizeY .

Option3:

This proposal is to combine the Option1 with Option2 and two options can be selected.

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … | u(4) |
| sps_log2_ctu_size_minus5 | u(2) |
| subpic_info_present_flag | u(1) |
| if( subpic_info_present_flag ) { |  |
| sps_num_subpics_minus1 | ue(v) |
| sps_independent_subpics_flag | u(1) |
| subpic_unit_num_ctus_present_flag | u(1) |
| if(subpic_unit_num_ctus_present_flag) |  |
| ctu_x_same_as_ctu_y_flag | u(1) |
| for( i = 0; i < ctu_x_same_as_ctu_y_flag ? 1 : 2; i++ ) |  |
| subpic_unit_num_ctus_minus1[ iI ] | ue(v) |
| } |  |
| for( i = 0; sps_num_subpics_minus1 > 0 && i <= sps_num_subpics_minus1; i++ ) { |  |
| if( i > 0 && pic_width_max_in_luma_samples > CtbSizeY ) |  |
| subpic_ctu_top_left_x[ i ] | u(v) |
| if( i > 0 && pic_height_max_in_luma_samples > CtbSizeY ) { |  |
| subpic_ctu_top_left_y[ i ] | u(v) |
| if( i < sps_num_subpics_minus1 && pic_width_max_in_luma_samples > CtbSizeY ) |  |
| subpic_width_minus1[ i ] | u(v) |
| if( i < sps_num_subpics_minus1 && pic_height_max_in_luma_samples > CtbSizeY ) |  |
| subpic_height_minus1[ i ] | u(v) |
| if( !sps_independent_subpics_flag) { |  |
| subpic_treated_as_pic_flag[ i ] | u(1) |
| loop_filter_across_subpic_enabled_flag[ i ] | u(1) |
| } |  |
| } |  |
| … |  |

subpic_unit_num_ctus_present_flag equal to 1 specifies that subpic_unit_num_ctus_minus1[i] syntax element are present. subpic_unit_num_ctus_present_flag equal to 0 specifies that subpic_unit_num_ctus_minus1[i] syntax element are not present. When not present, the value of subpic_unit_num_ctus_present_flag is inferred to be equal to 0.

ctu_x_same_as_ctu_y_flag equal to 1 specifies that subpic_unit_num_ctus_minus1[0] and subpic_unit_num_ctus_minus1[1] are the same value and only subpic_unit_num_ctus_minus1[0] is present. ctu_x_same_as_ctu_y_flag equal to 0 specifies that subpic_unit_num_ctus_minus1[0] and subpic_unit_num_ctus_minus1[1] may be different value. When not present, the value of ctu_x_same_as_ctu_y_flag is inferred to be equal to 1.

subpic_unit_num_ctus_minus1[0] is the value to determine a unit of the subpic_ctu_top_left_x[ i ] and subpic_width_minus1[ i ].  The unit of these syntax elements is (subpic_unit_num_ctus_minus1[0] +1) * CtbSizeY.  When not present, the value of subpic_unit_num_ctus_minus1[0] is inferred to be equal to 0.

The variables SubpicUnitCtusX are derived as follows:

SubpicUnitCtusX = ( subpic_unit_num_ctus_minus1[0] +1 ) * CtbSizeY	(xx)

subpic_unit_num_ctus_minus1[1] is the value to determine a unit of the subpic_ctu_top_left_y[ i ] and subpic_height_minus1[ i ].  The unit of these syntax elements is subpic_unit_num_ctus_minus1[1]  +1) * CtbSizeY.  When not present, the value of subpic_unit_num_ctus_minus1[1] is inferred to be equal to subpic_unit_num_ctus_minus1[0].

The variables SubpicUnitCtusY are derived as follows:

SubpicUnitCtusY = (subpic_unit_num_ctus_minus1[1] +1 ) * CtbSizeY	(xx)

subpic_ctu_top_left_x[ i ] specifies horizontal position of top left CTU of i-th subpicture in unit of SubpicUnitCtusX CtbSizeY. The length of the syntax element is Ceil( Log2( ( pic_width_max_in_luma_samples + CtbSizeY  − 1 )  >>  CtbLog2SizeY ) ) Ceil( Log2( (pic_width_max_in_luma_samples + SubpicUnitCtusX − 1)  >>  Log2(SubpicUnitCtusX ) ) )  bits. When not present, the value of subpic_ctu_top_left_x[ i ] is inferred to be equal to 0.

subpic_ctu_top_left_y[ i ] specifies vertical position of top left CTU of i-th subpicture in unit of SubpicUnitCtusY CtbSizeY. The length of the syntax element is Ceil( Log2( ( pic_height_max_in_luma_samples + CtbSizeY − 1 )  >>  CtbLog2SizeY ) ) Ceil( Log2( (pic_height_max_in_luma_samples + SubpicUnitCtusY− 1)  >>  Log2(SubpicUnitCtusY ) ) ) bits. When not present, the value of subpic_ctu_top_left_y[ i ] is inferred to be equal to 0.

subpic_width_minus1[ i ] plus 1 specifies the width of the i-th subpicture in units of SubpicUnitCtusX CtbSizeY. The length of the syntax element is Ceil( Log2( ( pic_width_max_in_luma_samples + CtbSizeY − 1 )  >>  CtbLog2SizeY ) ) Ceil( Log2( (pic_width_max_in_luma_samples + SubpicUnitCtusX − 1)  >>  Log2(SubpicUnitCtusX ) ) )  bits. When not present, the value of subpic_width_minus1[ i ] is inferred to be equal to ( ( pic_width_max_in_luma_samples + CtbSizeY− 1 )  >>  CtbLog2SizeY ) − subpic_ctu_top_left_x[ i ] − 1 ( (pic_width_max_in_luma_samples + SubpicUnitCtusX− 1)  >>  Log2(SubpicUnitCtusX ) ) - subpic_ctu_top_left_x[ i ] - 1.

subpic_height_minus1[ i ] plus 1 specifies the height of the i-th subpicture in units of SubpicUnitCtus CtbSizeY. The length of the syntax element is Ceil( Log2( ( pic_height_max_in_luma_samples + CtbSizeY − 1 )  >>  CtbLog2SizeY ) ) Ceil( Log2( (pic_height_max_in_luma_samples + SubpicUnitCtusY− 1)  >>  Log2(SubpicUnitCtusY ) ) )  bits. When not present, the value of subpic_height_minus1[ i ] is inferred to be equal to ( ( pic_height_max_in_luma_samples + CtbSizeY− 1 )  >>  CtbLog2SizeY ) − subpic_ctu_top_left_y[ i ] − 1 ( (pic_height_max_in_luma_samples + SubpicUnitCtusY− 1)  >>  Log2(SubpicUnitCtusY ) ) - subpic_ctu_top_left_y[ i ] - 1.

6.5.1

for( i = 0; i  <=  sps_num_subpics_minus1; i++ ) {
	leftX = subpic_ctu_top_left_x[ i ] * (subpic_unit_num_ctus_minus1[0]+1)
	rightX = leftX + subpic_width_minus1[ i ] * (subpic_unit_num_ctus_minus1[0] +1)
	SubpicWidthInTiles[ i ] = ctbToTileColIdx[ rightX ] + 1 − ctbToTileColIdx[ leftX ]	(29)
	topY = subpic_ctu_top_left_y[ i ] * (subpic_unit_num_ctus_minus1[1] +1)
	bottomY = topY + subpic_height_minus1[ i ] * (subpic_unit_num_ctus_minus1[1] +1)
	SubpicHeightInTiles[ i ] = ctbToTileRowIdx[ bottomY ] + 1 − ctbToTileRowIdx[ topY ]
	if( SubpicHeightInTiles[ i ]  = =  1  &&
			subpic_height_minus1[ i ] + 1 < 	RowHeight[ ctbToTileRowIdx[ topY ] ] )
		subpicHeightLessThanOneTileFlag[ i ] = 1
else
		subpicHeightLessThanOneTileFlag[ i ] = 0
}

if( single_slice_per_subpic_flag ) {
	for( i = 0; i  <=   sps_num_subpics_minus1; i++ ) {
		NumCtusInSlice[ i ] = 0 
		if( subpicHeightLessThanOneTileFlag[ i ] ) /* The slice consists of a number of CTU rows in a tile. */
			AddCtbsToSlice( i, subpic_ctu_top_left_x[ i ] * (subpic_unit_num_ctus_minus1[0] +1),
				(subpic_ctu_top_left_x[ i ] + subpic_width_minus1[ i ] + 1) * (subpic_unit_num_ctus_minus1[0] +1), subpic_ctu_top_left_y[ i ] * (subpic_unit_num_ctus_minus1[1] +1),
				(subpic_ctu_top_left_y[ i ] + subpic_height_minus1[ i ] + 1) * (subpic_unit_num_ctus_minus1[1] +1))
		else { /* The slice consists of a number of complete tiles covering a rectangular region. */
			tileX = CtbToTileColBd[ subpic_ctu_top_left_x[ i ] * (subpic_unit_num_ctus_minus1[0] +1)]
			tileY = CtbToTileRowBd[ subpic_ctu_top_left_y[ i ] * (subpic_unit_num_ctus_minus1[1] +1)]
			for( j = 0; j < SubpicHeightInTiles[ i ]; j++ )
				for( k = 0; k < SubpicWidthInTiles[ i ]; k++ )
					AddCtbsToSlice( i, tileColBd[ tileX + k ], tileColBd[ tileX + k + 1 ], tileRowBd[ tileY + j ],
							tileRowBd[ tileY + j + 1 ] )
		}
	}
} else {

for( i = 0; i  <=  sps_num_subpics_minus1; i++ ) {
	NumSlicesInSubpic[ i ] = 0
	for( j = 0; j  <=  num_slices_in_pic_minus1; j++ ) {
		posX = CtbAddrInSlice[ j ][ 0 ] % PicWidthInCtbsY
		posY = CtbAddrInSlice[ j ][ 0 ] / PicWidthInCtbsY
		if( ( posX  >=  subpic_ctu_top_left_x[ i ] * (subpic_unit_num_ctus_minus1[0] +1))  &&	(32)
				( posX < (subpic_ctu_top_left_x[ i ] + subpic_width_minus1[ i ] + 1) * (subpic_unit_num_ctus_minus1[0] +1))  &&
				( posY  >=  subpic_ctu_top_left_y[ i ] * (subpic_unit_num_ctus_minus1[1] +1))  &&
				( posY < (subpic_ctu_top_left_y[ i ] + subpic_height_minus1[ i ] + 1 ) * (subpic_unit_num_ctus_minus1[1] +1) ) )
			NumSlicesInSubpic[ i ]++
}

7.4.3.14

The variable subpicSizeInMinCbsY for the subpicture with subpicture index subpicIdxA is derived to be equal to ( ( subpic_width_minus1[ subpicIdxA ] + 1) *SubpicUnitCtusX CtbSizeY / MinCbSizeY * ( subpic_height_minus1[ subpicIdxA ] + 1) * SubpicUnitCtusY CtbSizeY / MinCbSizeY ).

7.4.8.1

The variables SubpicLeftBoundaryPos, SubpicTopBoundaryPos, SubpicRightBoundaryPos, and SubpicBotBoundaryPos are derived as follows:

if( subpic_treated_as_pic_flag[ CurrSubpicIdx ] ) {
	SubpicLeftBoundaryPos = subpic_ctu_top_left_x[ CurrSubpicIdx ] * SubpicUnitCtusX CtbSizeY
	SubpicRightBoundaryPos = Min( pic_width_max_in_luma_samples − 1,
			( subpic_ctu_top_left_x[ CurrSubpicIdx ] +
			subpic_width_minus1[ CurrSubpicIdx ] + 1 ) * SubpicUnitCtusX CtbSizeY − 1 )
	SubpicTopBoundaryPos = subpic_ctu_top_left_y[ CurrSubpicIdx ] * SubpicUnitCtusY CtbSizeY	(117)
	SubpicBotBoundaryPos = Min( pic_height_max_in_luma_samples − 1,
			( subpic_ctu_top_left_y[ CurrSubpicIdx ] +
			subpic_height_minus1[ CurrSubpicIdx ] + 1 ) * SubpicUnitCtusY CtbSizeY − 1 )
}

D.7.2

The variable SubpicSizeY[ j ] is set equal to ( subpic_width_minus1[ j ] + 1 ) *  SubpicUnitCtusX CtbSizeY * ( subpic_height_minus1[ j ] + 1 ) *  SubpicUnitCtusY CtbSizeY .

Example

Using the subpicture layout of that Fig. 2 is example of Figure 7 in the VVC specification, it compare the bit length of subpicture layout signalling among JVET-Q2001-vE, option1, option2,and option3.  As a precondition, CtbSizeY is 128 and subpic_unit_num_ctus_minus1, subpic_unit_num_ctus_x_minus1, subpic_unit_num_ctus_y_minus1, subpic_unit_num_ctus_minus1[0], subpic_unit_num_ctus_minus1[1] is 1,respectively.

pic_width_max_in_luma_samples =2560

CtbSizeY=128

pic_height_max_in_luma_samples =1536

Fig. 2 Figure 7 in VVC specification

CtbSizeY=128, 24 subpictures

Table. 1 shows the bit length of syntax elements. Option 3 (1) is in case that ctu_x_same_as_ctu_y_flag  is 1. Option 3 (2) is in case that ctu_x_same_as_ctu_y_flag is 0. “-“ indicates the syntax element is not present.

Table. 1  bit length of the syntax element

| Syntax element | JVET-Q2001-vE | Option1 | Option2 | Option3 (1) | Option3 (2) | Remarks |
| --- | --- | --- | --- | --- | --- | --- |
| subpic_unit_num_ctus_present_flag | - | 1 | 1 | 1 | 1 |  |
| subpic_unit_num_ctus_minus1 | - | 5 | - | - | - |  |
| subpic_unit_num_ctus_x_minus1 | - | - | 5 | - | - |  |
| subpic_unit_num_ctus_y_minus1 | - | - | 5 | - | - |  |
| ctu_x_same_as_ctu_y_flag | - | - | - | 1 | 1 |  |
| subpic_unit_num_ctus_minus1[0] | - | - | - | 5 | 5 |  |
| subpic_unit_num_ctus_minus1[1] | - | - | - | - | 5 |  |
| subpic_ctu_top_left_x[ i ] | 5 | 4 | 4 | 4 | 4 | i=0 not signalled |
| subpic_ctu_top_left_y[ i ] | 4 | 3 | 3 | 3 | 3 | i=0 not signalled |
| subpic_width_minus1[ i ] | 5 | 4 | 4 | 4 | 4 | i=23 not signalled |
| subpic_height_minus1[ i ] | 4 | 3 | 3 | 3 | 3 | i=23 not signalled |
| Subpicture layout bit length | 414 | 328 | 333 | 329 | 334 |  |

Conclusion

It prefers the option3. It is the flexible signalling and almost same bit saving

Patent rights declaration(s)

Sony Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).