| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0136 |
| --- | --- |

| Title: | AHG/9AHG12: Improvements on sps_independent_subpics_flag and nal_unit_type constraint |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Mitsuru Katsumata Mitsuhiro Hirabayashi Teruhiko Suzuki 2-10-1 Osaki Shinagawa-ku, Tokyo, 141-8610, Japan | Tel: Email: | +81-50-3750-7393 Mitsuru.Katsumata@sony.com Mitsuhiro.Hirabayashi@sony.com Teruhiko.S@sony.com |
| Source: | Sony Corporation |  |  |

_____________________________

- Abstract

This contribution proposes two improvements, including

- when sps_independent_subpics_flag is not present, sps_independent_subpics_flag should be inferred to be equal to 1 (=treat as picture).

- to apply the current nal_unit_type constraints to a bitstream of a subpicture sequence from single-layer bitstream which mixed_nalu_types_in_pic_flag is equal to 1.

improvement of sps_independent_subpics_flag semantics

Introduction

VVC specification draft in JVET-Q2001-vE includes support for sps_independent_subpics_flag syntax element.  This flag and related syntax elements specify below.

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| subpic_info_present_flag | u(1) |
| if( subpic_info_present_flag ) { |  |
| sps_num_subpics_minus1 | ue(v) |
| sps_independent_subpics_flag | u(1) |
| for( i = 0; sps_num_subpics_minus1 > 0 && i <= sps_num_subpics_minus1; i++ ) { |  |
| … |  |
| if( !sps_independent_subpics_flag) { |  |
| subpic_treated_as_pic_flag[ i ] | u(1) |
| loop_filter_across_subpic_enabled_flag[ i ] | u(1) |
| } |  |
| } |  |
| … |  |

subpic_info_present_flag equal to 1 specifies that subpicture information is present for the CLVS and there may be one or more than one subpicture in each picture of the CLVS. subpic_info_present_flag equal to 0 specifies that subpicture information is not present for the CLVS and there is only one subpicture in each picture of the CLVS.

sps_independent_subpics_flag equal to 1 specifies that all subpicture boundaries in the CLVS are treated as picture boundaries and there is no loop filtering across the subpicture boundaries. sps_independent_subpics_flag equal to 0 does not impose such a constraint. When not present, the value of sps_independent_subpics_flag is inferred to be equal to 0.

subpic_treated_as_pic_flag[ i ] equal to 1 specifies that the i-th subpicture of each coded picture in the CLVS is treated as a picture in the decoding process excluding in-loop filtering operations. subpic_treated_as_pic_flag[ i ] equal to 0 specifies that the i-th subpicture of each coded picture in the CLVS is not treated as a picture in the decoding process excluding in-loop filtering operations. When not present, the value of subpic_treated_as_pic_flag[ i ] is inferred to be equal to sps_independent_subpics_flag.

loop_filter_across_subpic_enabled_flag[ i ] equal to 1 specifies that in-loop filtering operations may be performed across the boundaries of the i-th subpicture in each coded picture in the CLVS. loop_filter_across_subpic_enabled_flag[ i ] equal to 0 specifies that in-loop filtering operations are not performed across the boundaries of the i-th subpicture in each coded picture in the CLVS. When not present, the value of loop_filter_across_subpic_enabled_pic_flag[ i ] is inferred to be equal to 1 − sps_independent_subpics_flag.

When sps_independent_subpics_flag is not present, the value of subpic_info_present_flag is equal to 0 and there is only one subpicture in each picture of the CLVS.  In other words, the subpicture is equal to picture.  In this case, as sps_independent_subpics_flag is inffered to be equal 0, subpic_treated_as_pic_flag[ 0 ] is inffered to 0 and subpicture is not treated as a picture.

To fix this problem, this contribution proposes to change the inferred value of sps_independent_subpics_flag.

Proposal text

The proposed syntax and semantics are marked in yellow with removed text in red strike through on top of the current VVC draft (JVET-Q2001-vE)

sps_independent_subpics_flag equal to 1 specifies that all subpicture boundaries in the CLVS are treated as picture boundaries and there is no loop filtering across the subpicture boundaries. sps_independent_subpics_flag equal to 0 does not impose such a constraint. When not present, the value of sps_independent_subpics_flag is inferred to be equal to 1 0.

imporovement of nal_unit_type constraint

Introduction

VVC specification draft in JVET-Q2001-vE includes the below nal_unit_type constraint.  These constraints are for pictures in bitstream.

For a single-layer bitstream, the following constraints apply: [Ed. (YK): Further study the following constraints for mixed_nalu_types_in_pic_flag is equal to 1 and for multi-layer bitstreams.]

–	Each picture, other than the first picture in the bitstream in decoding order, is considered to be associated with the previous IRAP picture in decoding order.

–	When a picture is a leading picture of an IRAP picture, it shall be a RADL or RASL picture.

….

As noted in the editor's comments, we considered the case which mixed_nalu_types_in_pic_flag is equal to 1.

On the other hand, the following semantics of mixed_nalu_types_in_pic_flag specify that a subpicture of the picture which mixed_nalu_types_in_pic_flag is equal to 1 is treated as a picture and the subpicture has all VCL NAL unit with the same nal_unit_type.

For each slice with a nal_unit_type value nalUnitTypeA in the range of IDR_W_RADL to CRA_NUT, inclusive, in a picture picA that also contains one or more slices with another value of nal_unit_type (i.e., the value of mixed_nalu_types_in_pic_flag for the picture picA is equal to 1), the following applies:

–	The slice shall belong to a subpicture subpicA for which the value of the corresponding subpic_treated_as_pic_flag[ i ] is equal to 1.

–	The slice shall not belong to a subpicture of picA containing VCL NAL units with nal_unit_type not equal to nalUnitTypeA

As described above, when a subpicture is extracted from bitstream with mixed_nalu_types_in_pic_flag equal to 1, this resulting bitstream asserts to be a bitstream with mixed_nalu_types_in_pic_flag equal to 0.  Therefore, current nal_unit_type constrains could be applied to the resulting bitstream.

This contribution proposes to apply the current nal_unit_type constraints to a bitstream of a subpicture sequence from single-layer bitstream which mixed_nalu_types_in_pic_flag is equal to 1.

proposal text

The proposed syntax and semantics are marked in yellow with removed text in red strike through on top of the current VVC draft (JVET-Q2001-vE)

For a single-layer bitstream which include all picture with mixed_nalu_types_in_pic_flag equal to 0 the following constraints apply:

Or for a single-layer bitstream which include one or more pictures with mixed_nalu_types_in_pic_flag equal to 1, the following constrains apply to a bitstream of a subpicture sequence which could be extracted:

–	Each picture, other than the first picture in the bitstream in decoding order, is considered to be associated with the previous IRAP picture in decoding order.

–	When a picture is a leading picture of an IRAP picture, it shall be a RADL or RASL picture.

…

Patent rights declaration(s)

Sony Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).