| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 |  |  | Document: JVET-R0139_r1 |
| --- | --- | --- | --- |
| Title: | AHG9: High-level control flag for lossless coding |  |  |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tianyang Zhou Eiichi Sasaki Tomohiro Ikai 1-9-2 Nakase, Mihama-ku, Chiba, Japan | Tel: Email: | +81-50-5213-6338 zhou.tianyang@sharp.co.jp |
| Source: | Sharp Corporation |  |  |

_____________________________

- Abstract

This contribution proposes a high-level control flag for lossless coding in VVC. It includes two options:

Option 1: A sequence parameter set (SPS) level flag for lossless mode

Option 2: A picture parameter set (PPS) level flag for lossless mode

In both cases, some applicable flags that are always constant in lossless mode are not signaled when the proposed lossless flag is equal to 1. It is asserted that the proposed syntax can be beneficial for sequence or picture level lossless case and it avoids useless flag signalling in lossless coding case.

Introduction

In the current VVC design, lossless coding is achieved with a CU level flag, transform_skip_mode and dedicated lossless configuration where loopfilter and dependent quantization, etc. are disabled and QPs are equal to or less than the lossless level.

It is asserted that a large number of applicable flags are not used in the lossless encoding mode.

Proposal

In this contribution, it is proposed to use a high-level lossless coding control flag in SPS or PPS level.

Option 1 SPS level lossless mode flag

In option 1, we introduce sps_lossless_mode_enabled_flag in SPS.

When the value of sps_lossless_mode_enabled_flag is equal to 1, the applicable flags for the following tools are not signaled but inferred.

- Transform skip (always enabled)

- SAO / ALF / DF (always disabled)

- DQ, SBT, LMCS, ISP, MTS, LFNST, LADF, JCCR, VB (always disabled)

To ensure the transform skip available in all TUs, the max TU size and the max transform skip block size is inferred as follows.

- sps_max_luma_transform_size_64_flag (always 0)

- log2_transform_skip_max_size_minus2 (always 3)

This way, both of the max TU size and max transform unit size becomes 32 where all possible TUs can be coded as transform skip blocks.

Secondly, when the value of sps_lossless_mode enabled_flag is equal to 1, the applicable flags for the following tools are constrained as a bitstream constraint.

- DF (always disabled)

Lastly when the value of sps_lossless_mode enabled_flag is equal to 1, the the following syntax is not signaled but inferred.

- transform_skip_flag (always equal to 1)

The specific inferred value and bitstream constraints are shown in Table 1.

Table 1. Proposed inferred value

| Where | Syntax element | Infered value or Bitstream constraint |
| --- | --- | --- |
| SPS | sps_dep_quant_enabled_flag | 0 |
|  | sps_sbt_enabled_flag | 0 |
|  | sps_lmcs_enabled_flag | 0 |
|  | sps_isp_enabled_flag | 0 |
|  | sps_mts_enabled_flag | 0 |
|  | sps_lfnst_enabled_flag | 0 |
|  | sps_ladf_enabled_flag | 0 |
|  | sps_joint_cbcr_enabled_flag | 0 |
|  | sps_sao_enabled_flag | 0 |
|  | sps_alf_enabled_flag | 0 |
|  | sps_ccalf_enabled_flag | 0 |
|  | sps_dmvr_enabled_flag | 0 |
|  | sps_bdof_enabled_flag | 0 |
|  | sps_affine_prof_enabled_flag | 0 |
|  | sps_max_luma_transform_size_64_flag | 0 |
|  | sps_transform_skip_enabled_flag | 1 |
|  | log2_transform_skip_max_size_minus2 | 3 |
|  | sps_virtual_boundaries_enabled_flag | 0 |
| PPS | deblocking_filter_control_present_flag | When sps_lossless_mode_enabled_flag is equal to 1, the value of deblocking_filter_control_present_flag shall be equal to 0. |
|  | deblocking_filter_disabled_flag | sps_lossless_mode_enabled_flag ? 1 : 0 |
|  | cu_qp_delta_enabled_flag | When sps_lossless_mode_enabled_flag is equal to 1, the value of cu_qp_delta_enabled_flag shall be equal to 0. |
|  | pps_chroma_tool_offsets_present_flag | When sps_lossless_mode_enabled_flag is equal to 1, the value of pps_chroma_tool_offsets_present_flag shall be equal to 0. |
| CU | transform_skip_flag[ x0 ][ y0 ][ cIdx ] | – If sps_lossless_mode_enabled_flag is equal to 1 or BdpcmFlag[ x0 ][ y0 ][ cIdx ] is equal to 1, transform_skip_flag[ x0 ][ y0 ][ cIdx ] is inferred to be equal to 1. – Otherwise (BdpcmFlag[ x0 ][ y0 ][ cIdx ] is equal to 0), transform_skip_flag[ x0 ][ y0 ][ cIdx ] is inferred to be equal to 0. |

Proposed syntax change

We propose the following syntax changes. The added parts are highlighted in yellow.

      - Sequence parameter set RBSP syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| sps_seq_parameter_set_id | u(4) |
| … |  |
| sps_lossless_mode_enabled_flag | u(1) |
| … |  |
| if (!sps_lossless_mode_enabled_flag) { |  |
| sps_max_luma_transform_size_64_flag | u(1) |
| if( ChromaArrayType != 0 ) { |  |
| sps_joint_cbcr_enabled_flag | u(1) |
| same_qp_table_for_chroma | u(1) |
| numQpTables = same_qp_table_for_chroma ? 1 : ( sps_joint_cbcr_enabled_flag ? 3 : 2 ) |  |
| for( i = 0; i < numQpTables; i++ ) { |  |
| qp_table_start_minus26[ i ] | se(v) |
| num_points_in_qp_table_minus1[ i ] | ue(v) |
| for( j = 0; j <= num_points_in_qp_table_minus1[ i ]; j++ ) { |  |
| delta_qp_in_val_minus1[ i ][ j ] | ue(v) |
| delta_qp_diff_val[ i ][ j ] | ue(v) |
| } |  |
| } |  |
| } |  |
| sps_sao_enabled_flag | u(1) |
| sps_alf_enabled_flag | u(1) |
| if( sps_alf_enabled_flag && ChromaArrayType != 0 ) |  |
| sps_ccalf_enabled_flag | u(1) |
| sps_transform_skip_enabled_flag | u(1) |
| if( sps_transform_skip_enabled_flag ) { |  |
| log2_transform_skip_max_size_minus2 | ue(v) |
| sps_bdpcm_enabled_flag | u(1) |
| } |  |
| } |  |
| … |  |
| if (!sps_lossless_mode_enabled_flag) { |  |
| sps_bdof_enabled_flag | u(1) |
| if( sps_bdof_enabled_flag ) |  |
| sps_bdof_pic_present_flag | u(1) |
| } |  |
| sps_smvd_enabled_flag | u(1) |
| if (!sps_lossless_mode_enabled_flag) { |  |
| sps_dmvr_enabled_flag | u(1) |
| if( sps_dmvr_enabled_flag) |  |
| sps_dmvr_pic_present_flag | u(1) |
| } |  |
| sps_mmvd_enabled_flag | u(1) |
| if (!sps_lossless_mode_enabled_flag) { |  |
| sps_isp_enabled_flag | u(1) |
| } |  |
| … |  |
| if (!sps_lossless_mode_enabled_flag) { |  |
| sps_mts_enabled_flag | u(1) |
| … |  |
| if (!sps_lossless_mode_enabled_flag) { |  |
| sps_sbt_enabled_flag | u(1) |
| … |  |
| if (!sps_lossless_mode_enabled_flag) { |  |
| sps_affine_prof_enabled_flag | u(1) |
| … |  |
| if (!sps_lossless_mode_enabled_flag) { |  |
| sps_lmcs_enabled_flag | u(1) |
| sps_lfnst_enabled_flag | u(1) |
| sps_ladf_enabled_flag | u(1) |
| if( sps_ladf_enabled_flag ) { |  |
| sps_num_ladf_intervals_minus2 | u(2) |
| sps_ladf_lowest_interval_qp_offset | se(v) |
| for( i = 0; i < sps_num_ladf_intervals_minus2 + 1; i++ ) { |  |
| sps_ladf_qp_offset[ i ] | se(v) |
| sps_ladf_delta_threshold_minus1[ i ] | ue(v) |
| } |  |
| } |  |
| } |  |
| … |  |
| if (!sps_lossless_mode_enabled_flag) { |  |
| sps_explicit_scaling_list_enabled_flag | u(1) |
| sps_dep_quant_enabled_flag | u(1) |
| if( !sps_dep_quant_enabled_flag ) |  |
| sps_sign_data_hiding_enabled_flag | u(1) |
| } |  |
| … |  |

      - Transform unit syntax

| transform_unit( x0, y0, tbWidth, tbHeight, treeType, subTuIndex, chType ) { | Descriptor |
| --- | --- |
| … |  |
| if( tu_y_coded_flag[ x0 ][ y0 ] && treeType != DUAL_TREE_CHROMA ) { |  |
| if( sps_transform_skip_enabled_flag && !BdpcmFlag[ x0 ][ y0 ][ 0 ] && tbWidth <= MaxTsSize && tbHeight <= MaxTsSize && ( IntraSubPartitionsSplitType = = ISP_NO_SPLIT ) && !cu_sbt_flag && !sps_lossless_mode_enabled_flag) |  |
| transform_skip_flag[ x0 ][ y0 ][ 0 ] | ae(v) |
| if( !transform_skip_flag[ x0 ][ y0 ][ 0 ] ¦ ¦ slice_ts_residual_coding_disabled_flag ) |  |
| residual_coding( x0, y0, Log2( tbWidth ), Log2( tbHeight ), 0 ) |  |
| else |  |
| residual_ts_coding( x0, y0, Log2( tbWidth ), Log2( tbHeight ), 0 ) |  |
| } |  |
| if( tu_cb_coded_flag[ xC ][ yC ] && treeType != DUAL_TREE_LUMA ) { |  |
| if( sps_transform_skip_enabled_flag && !BdpcmFlag[ x0 ][ y0 ][ 1 ] && wC <= MaxTsSize && hC <= MaxTsSize && !cu_sbt_flag && !sps_lossless_mode_enabled_flag) |  |
| transform_skip_flag[ xC ][ yC ][ 1 ] | ae(v) |
| if( !transform_skip_flag[ xC ][ yC ][ 1 ] ¦ ¦ slice_ts_residual_coding_disabled_flag ) |  |
| residual_coding( xC, yC, Log2( wC ), Log2( hC ), 1 ) |  |
| else |  |
| residual_ts_coding( xC, yC, Log2( wC ), Log2( hC ), 1 ) |  |
| } |  |
| if( tu_cr_coded_flag[ xC ][ yC ] && treeType != DUAL_TREE_LUMA && !( tu_cb_coded_flag[ xC ][ yC ] && tu_joint_cbcr_residual_flag[ xC ][ yC ] ) ) { |  |
| if( sps_transform_skip_enabled_flag && !BdpcmFlag[ x0 ][ y0 ][ 2 ] && wC <= MaxTsSize && hC <= MaxTsSize && !cu_sbt_flag && !sps_lossless_mode_enabled_flag) |  |
| transform_skip_flag[ xC ][ yC ][ 2 ] | ae(v) |
| if( !transform_skip_flag[ xC ][ yC ][ 2 ] ¦ ¦ slice_ts_residual_coding_disabled_flag ) |  |
| residual_coding( xC, yC, Log2( wC ), Log2( hC ), 2 ) |  |
| else |  |
| residual_ts_coding( xC, yC, Log2( wC ), Log2( hC ), 2 ) |  |
| } |  |
| } |  |

Option 2 PPS level lossless mode flag

In option 2, we introduce pps_lossless_mode_enabled_flag in PPS.

When the value of pps_lossless_mode enabled_flag is equal to 1, the applicable flags for the following tools are not signaled but inferred.

- DF (always disabled)

Lastly when the value of pps_lossless_mode enabled_flag is equal to 1, the following tools are not signaled but inferred.

- transform_skip_flag (always equal to 1)

Table 2. Proposed inferred value

| Where | Syntax element | Infered value or Bitstream constraint |
| --- | --- | --- |
| PPS | deblocking_filter_control_present_flag | 0 |
|  | deblocking_filter_disabled_flag | pps_lossless_mode_enabled_flag ? 1 : 0 |
|  | cu_qp_delta_enabled_flag | 0 |
|  | pps_chroma_tool_offsets_present_flag | 0 |
|  | sao_info_in_ph_flag | 1 |
|  | alf_info_in_ph_flag | 1 |
| PH | ph_alf_enabled_flag | 0 |
|  | ph_explicit_scaling_list_enabled_flag | 0 |
|  | ph_virtual_boundaries_present_flag | 0 |
|  | ph_lmcs_enabled_flag | 0 |
|  | ph_disable_bdof_flag | When ph_disable_bdof_flag is not present, the following applies: - If sps_bdof_enabled_flag is equal to 1 and pps_lossless_mode_enabled_flag is equal to 0, the value of ph_disable_bdof_flag is inferred to be equal to 0. - Otherwise (sps_bdof_enabled_flag is equal to 0 or pps_lossless_mode_enabled_flag is equal to 1), the value of ph_disable_bdof_flag is inferred to be equal to 1. |
|  | ph_disable_dmvr_flag | - If sps_dmvr_enabled_flag is equal to 1 and pps_lossless_mode_enabled_flag is equal to 0, the value of ph_disable_dmvr_flag is inferred to be equal to 0. - Otherwise (sps_dmvr_enabled_flag is equal to 0 or pps_lossless_mode_enabled_flag is equal to 1), the value of ph_disable_dmvr_flag is inferred to be equal to 1. |
|  | ph_disable_prof_flag | When ph_disable_prof_flag is not present, the following applies: - If sps_affine_prof_enabled_flag is equal to 1 and pps_lossless_mode_enabled_flag is equal to 0, the value of ph_disable_prof_flag is inferred to be equal to 0. - Otherwise (sps_affine_prof_enabled_flag is equal to 0 or pps_lossless_mode_enabled_flag is equal to 1), the value of ph_disable_prof_flag is inferred to be equal to 1. |
|  | ph_sao_luma_enabled_flag | 0 |
|  | ph_sao_chroma_enabled_flag | 0 |
|  | ph_dep_quant_enabled_flag | 0 |
|  | pic_sign_data_hiding_enabled_flag | 0 |
| CU | tu_joint_cbcr_residual_flag[ xC ][ yC ] | 0 |
|  | transform_skip_flag[ x0 ][ y0 ][ cIdx ] | – If pps_lossless_mode_enabled_flag is equal to 1 or BdpcmFlag[ x0 ][ y0 ][ cIdx ] is equal to 1, transform_skip_flag[ x0 ][ y0 ][ cIdx ] is inferred to be equal to 1. – Otherwise (BdpcmFlag[ x0 ][ y0 ][ cIdx ] is equal to 0), transform_skip_flag[ x0 ][ y0 ][ cIdx ] is inferred to be equal to 0. |

    - Proposed syntax change

We propose the following syntax changes. The added parts are highlighted in yellow.

      - Picture parameter set RBSP syntax

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| pps_pic_parameter_set_id | ue(v) |
| … |  |
| pps_lossless_mode_enabled_flag | u(1) |
| … |  |
| if( !pps_lossless_mode_enabled_flag ) { |  |
| cu_qp_delta_enabled_flag | u(1) |
| pps_chroma_tool_offsets_present_flag | u(1) |
| if( pps_chroma_tool_offsets_present_flag ) { |  |
| pps_cb_qp_offset | se(v) |
| pps_cr_qp_offset | se(v) |
| pps_joint_cbcr_qp_offset_present_flag | u(1) |
| if( pps_joint_cbcr_qp_offset_present_flag ) |  |
| pps_joint_cbcr_qp_offset_value | se(v) |
| pps_slice_chroma_qp_offsets_present_flag | u(1) |
| pps_cu_chroma_qp_offset_list_enabled_flag | u(1) |
| } |  |
| if( pps_cu_chroma_qp_offset_list_enabled_flag ) { |  |
| chroma_qp_offset_list_len_minus1 | ue(v) |
| for( i = 0; i <= chroma_qp_offset_list_len_minus1; i++ ) { |  |
| cb_qp_offset_list[ i ] | se(v) |
| cr_qp_offset_list[ i ] | se(v) |
| if( pps_joint_cbcr_qp_offset_present_flag ) |  |
| joint_cbcr_qp_offset_list[ i ] | se(v) |
| } |  |
| } |  |
| } |  |
| pps_weighted_pred_flag | u(1) |
| pps_weighted_bipred_flag | u(1) |
| if( !pps_lossless_mode_enabled_flag ) { |  |
| deblocking_filter_control_present_flag | u(1) |
| if( deblocking_filter_control_present_flag ) { |  |
| deblocking_filter_override_enabled_flag | u(1) |
| pps_deblocking_filter_disabled_flag | u(1) |
| if( !pps_deblocking_filter_disabled_flag ) { |  |
| pps_beta_offset_div2 | se(v) |
| pps_tc_offset_div2 | se(v) |
| pps_cb_beta_offset_div2 | se(v) |
| pps_cb_tc_offset_div2 | se(v) |
| pps_cr_beta_offset_div2 | se(v) |
| pps_cr_tc_offset_div2 | se(v) |
| } |  |
| } |  |
| } |  |
| rpl_info_in_ph_flag | u(1) |
| if( deblocking_filter_override_enabled_flag ) |  |
| dbf_info_in_ph_flag | u(1) |
| if( !pps_lossless_mode_enabled_flag ) { |  |
| sao_info_in_ph_flag | u(1) |
| alf_info_in_ph_flag | u(1) |
| } |  |
| … |  |

      - Picture header structure syntax

| picture_header_structure( ) { | Descriptor |
| --- | --- |
| … |  |
| if( sps_alf_enabled_flag && alf_info_in_ph_flag && !pps_lossless_mode_enabled_flag) { |  |
| ph_alf_enabled_flag | u(1) |
| if( ph_alf_enabled_flag ) { |  |
| ph_num_alf_aps_ids_luma | u(3) |
| for( i = 0; i < ph_num_alf_aps_ids_luma; i++ ) |  |
| ph_alf_aps_id_luma[ i ] | u(3) |
| if( ChromaArrayType != 0 ) |  |
| ph_alf_chroma_idc | u(2) |
| if( ph_alf_chroma_idc > 0 ) |  |
| ph_alf_aps_id_chroma | u(3) |
| if( sps_ccalf_enabled_flag ) { |  |
| ph_cc_alf_cb_enabled_flag | u(1) |
| if( ph_cc_alf_cb_enabled_flag ) |  |
| ph_cc_alf_cb_aps_id | u(3) |
| ph_cc_alf_cr_enabled_flag | u(1) |
| if( ph_cc_alf_cr_enabled_flag ) |  |
| ph_cc_alf_cr_aps_id | u(3) |
| } |  |
| } |  |
| } |  |
| if( sps_lmcs_enabled_flag && !pps_lossless_mode_enabled_flag) { |  |
| ph_lmcs_enabled_flag | u(1) |
| if( ph_lmcs_enabled_flag ) { |  |
| ph_lmcs_aps_id | u(2) |
| if( ChromaArrayType != 0 ) |  |
| ph_chroma_residual_scale_flag | u(1) |
| } |  |
| } |  |
| if( sps_explicit_scaling_list_enabled_flag && !pps_lossless_mode_enabled_flag) { |  |
| ph_explicit_scaling_list_enabled_flag | u(1) |
| if( ph_explicit_scaling_list_enabled_flag ) |  |
| ph_scaling_list_aps_id | u(3) |
| } |  |
| if( sps_virtual_boundaries_enabled_flag && !sps_virtual_boundaries_present_flag && !pps_lossless_mode_enabled_flag) { |  |
| ph_virtual_boundaries_present_flag | u(1) |
| if( ph_virtual_boundaries_present_flag ) { |  |
| ph_num_ver_virtual_boundaries | u(2) |
| for( i = 0; i < ph_num_ver_virtual_boundaries; i++ ) |  |
| ph_virtual_boundaries_pos_x[ i ] | u(13) |
| ph_num_hor_virtual_boundaries | u(2) |
| for( i = 0; i < ph_num_hor_virtual_boundaries; i++ ) |  |
| ph_virtual_boundaries_pos_y[ i ] | u(13) |
| } |  |
| } |  |
| … |  |
| if( sps_bdof_pic_present_flag && !pps_lossless_mode_enabled_flag) |  |
| ph_disable_bdof_flag | u(1) |
| if( sps_dmvr_pic_present_flag && !pps_lossless_mode_enabled_flag) |  |
| ph_disable_dmvr_flag | u(1) |
| if( sps_prof_pic_present_flag && !pps_lossless_mode_enabled_flag) |  |
| ph_disable_prof_flag | u(1) |
| … |  |
| if( sps_joint_cbcr_enabled_flag && !pps_lossless_mode_enabled_flag) |  |
| ph_joint_cbcr_sign_flag | u(1) |
| if( sps_sao_enabled_flag && sao_info_in_ph_flag && !pps_lossless_mode_enabled_flag) { |  |
| ph_sao_luma_enabled_flag | u(1) |
| if( ChromaArrayType != 0 ) |  |
| ph_sao_chroma_enabled_flag | u(1) |
| } |  |
| if( sps_dep_quant_enabled_flag && !pps_lossless_mode_enabled_flag ) |  |
| ph_dep_quant_enabled_flag | u(1) |
| if( sps_sign_data_hiding_enabled_flag && !ph_dep_quant_enabled_flag ) |  |
| pic_sign_data_hiding_enabled_flag | u(1) |
| … |  |

      - Transform unit syntax

| transform_unit( x0, y0, tbWidth, tbHeight, treeType, subTuIndex, chType ) { | Descriptor |
| --- | --- |
| … |  |
| if( sps_joint_cbcr_enabled_flag && !pps_lossless_mode_enabled_flag && ( ( CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_INTRA && ( tu_cb_coded_flag[ xC ][ yC ] ¦ ¦ tu_cr_coded_flag[ xC ][ yC ] ) ) ¦ ¦ ( tu_cb_coded_flag[ xC ][ yC ] && tu_cr_coded_flag[ xC ][ yC ] ) ) && chromaAvailable ) |  |
| tu_joint_cbcr_residual_flag[ xC ][ yC ] | ae(v) |
| if( tu_y_coded_flag[ x0 ][ y0 ] && treeType != DUAL_TREE_CHROMA ) { |  |
| if( sps_transform_skip_enabled_flag && !BdpcmFlag[ x0 ][ y0 ][ 0 ] && tbWidth <= MaxTsSize && tbHeight <= MaxTsSize && ( IntraSubPartitionsSplitType = = ISP_NO_SPLIT ) && !cu_sbt_flag && !pps_lossless_mode_enabled_flag) |  |
| transform_skip_flag[ x0 ][ y0 ][ 0 ] | ae(v) |
| if( !transform_skip_flag[ x0 ][ y0 ][ 0 ] ¦ ¦ slice_ts_residual_coding_disabled_flag ) |  |
| residual_coding( x0, y0, Log2( tbWidth ), Log2( tbHeight ), 0 ) |  |
| else |  |
| residual_ts_coding( x0, y0, Log2( tbWidth ), Log2( tbHeight ), 0 ) |  |
| } |  |
| if( tu_cb_coded_flag[ xC ][ yC ] && treeType != DUAL_TREE_LUMA ) { |  |
| if( sps_transform_skip_enabled_flag && !BdpcmFlag[ x0 ][ y0 ][ 1 ] && wC <= MaxTsSize && hC <= MaxTsSize && !cu_sbt_flag && !pps_lossless_mode_enabled_flag) |  |
| transform_skip_flag[ xC ][ yC ][ 1 ] | ae(v) |
| if( !transform_skip_flag[ xC ][ yC ][ 1 ] ¦ ¦ slice_ts_residual_coding_disabled_flag ) |  |
| residual_coding( xC, yC, Log2( wC ), Log2( hC ), 1 ) |  |
| else |  |
| residual_ts_coding( xC, yC, Log2( wC ), Log2( hC ), 1 ) |  |
| } |  |
| if( tu_cr_coded_flag[ xC ][ yC ] && treeType != DUAL_TREE_LUMA && !( tu_cb_coded_flag[ xC ][ yC ] && tu_joint_cbcr_residual_flag[ xC ][ yC ] ) ) { |  |
| if( sps_transform_skip_enabled_flag && !BdpcmFlag[ x0 ][ y0 ][ 2 ] && wC <= MaxTsSize && hC <= MaxTsSize && !cu_sbt_flag && !pps_lossless_mode_enabled_flag) |  |
| transform_skip_flag[ xC ][ yC ][ 2 ] | ae(v) |
| if( !transform_skip_flag[ xC ][ yC ][ 2 ] ¦ ¦ slice_ts_residual_coding_disabled_flag ) |  |
| residual_coding( xC, yC, Log2( wC ), Log2( hC ), 2 ) |  |
| else |  |
| residual_ts_coding( xC, yC, Log2( wC ), Log2( hC ), 2 ) |  |
| } |  |
| } |  |

Conclusion

This contribution proposes a high-level control flag for lossless coding in VVC. It is recommended to adopt this lossless flag syntax and semantics in the next VVC WD and VTM software.

Reference

[1]	T. Nguyen, T. C. Ma, A. Nalci, “JVET common test conditions and software reference configurations for lossless, near lossless, and mixed lossy/lossless coding,” Joint Video Expert Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-Q2014, 17th Meeting: Brussels, BE, 7–17 January 2020.

[2]	B. Bross, J. Chen, S. Liu, Y.-K. Wang, “Versatile Video Coding (Draft 8),” Joint Video Expert Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-Q2001, 17th Meeting: Brussels, BE, 7–17 January 2020.

Patent rights declaration(s)

Sharp Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).