| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0142_r1 |
| --- | --- |

| Title: | AHG9: Slice header signaling clean up |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tomonori Hashimoto Tomoko Aono Tomohiro Ikai 1-9-2 Nakase, Mihama-ku, Chiba, Japan | Tel: Email: | +81-50-5213-6337 tomonori.hashimoto@sharp.co.jp |
| Source: | Sharp Corporation |  |  |

_____________________________

- Abstract

This contribution proposes not to signal slice_ts_residual_coding_disabled_flag when sps_transform_skip_enabled_flag is equal to 0. When transform skip is disabled, the regular residual coding mode for transform skip mode, i.e. slice_ts_residual_coding_disabled_flag, is never used. The modification can avoid a unnecessary slice header level signaling when transform skip is disabled in SPS level.

Introduction

In the previous meeting, a syntax element of slice_ts_residual_coding_disabled_flag [1] was adopted into the VVC specification [2]. When this flag is enabled, regular residual coding (RRC) is used instead of transform skip residual coding (TSRC), even if transform skip is applied.

However, if transform skip is disabled in SPS, slice_ts_residual_coding_disabled_flag is never used. Since slice level signaling overhead is relatively large, it would not be desirable to signal unused flags.

Proposed syntax change

We propose the following syntax changes. The added texts are highlighted in yellow, and deleted texts are marked by red strikethrough.

- 7.3.7.1		General slice header syntax

| slice_header( ) { | Descriptor |
| --- | --- |
| picture_header_in_slice_header_flag | u(1) |
| … |  |
| if( deblocking_filter_override_enabled_flag && !dbf_info_in_ph_flag ) |  |
| slice_deblocking_filter_override_flag | u(1) |
| if( slice_deblocking_filter_override_flag ) { |  |
| slice_deblocking_filter_disabled_flag | u(1) |
| if( !slice_deblocking_filter_disabled_flag ) { |  |
| slice_beta_offset_div2 | se(v) |
| slice_tc_offset_div2 | se(v) |
| slice_cb_beta_offset_div2 | se(v) |
| slice_cb_tc_offset_div2 | se(v) |
| slice_cr_beta_offset_div2 | se(v) |
| slice_cr_tc_offset_div2 | se(v) |
| } |  |
| } |  |
| if( sps_transform_skip_enabled_flag ) |  |
| slice_ts_residual_coding_disabled_flag | u(1) |
| slice_ts_residual_coding_disabled_flag | u(1) |
| if( ph_lmcs_enabled_flag ) |  |
| slice_lmcs_enabled_flag | u(1) |
| if( ph_explicit_scaling_list_enabled_flag ) |  |
| slice_explicit_scaling_list_used_flag | u(1) |
| if( NumEntryPoints > 0 ) { |  |
| offset_len_minus1 | ue(v) |
| for( i = 0; i < NumEntryPoints; i++ ) |  |
| entry_point_offset_minus1[ i ] | u(v) |
| } |  |
| if( slice_header_extension_present_flag ) { |  |
| slice_header_extension_length | ue(v) |
| for( i = 0; i < slice_header_extension_length; i++) |  |
| slice_header_extension_data_byte[ i ] | u(8) |
| } |  |
| byte_alignment( ) |  |
| } |  |

- 7.4.8.1		General slice header semantics

…

slice_ts_residual_coding_disabled_flag equal to 1 specifies that the residual_coding( ) syntax structure is used to parse the residual samples of a transform skip block for the current slice. slice_ts_residual_coding_disabled_flag equal to 0 specifies that the residual_ts_coding( ) syntax structure is used to parse the residual samples of a transform skip block for the current slice. When slice_ts_residual_coding_disabled_flag is not present, it is infered to be equal to 0.

…

Conclusion

This contribution proposes not to signal slice_ts_residual_coding_disabled_flag when sps_transform_skip_enabled_flag is equal to 0. It is recommended to adopt this fix in the next VVC specification.

Reference

[1]	A. Nalci, H. Wang, M. Coban, M. Karczewicz, T. Tsukuba, M. Ikeda, Y. Yagasaki, T. Suzuki, Z.-Y. Lin, T.-D. Chuang, C.-Y. Chen, C.-W. Hsu, Y.-W. Huang, S.-M. Lei, T.-C. Ma, H.-J. Jhu, X. Xiu, Y.-W. Chen, X. Wang, “CE3-2.4: Luma and Chroma BDPCM for Lossless Coding with Regular Residual Coding,” Joint Video Expert Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-Q0089, 17th Meeting: Brussels, BE, 7–17 January 2020.

[2]	B. Bross, J. Chen, S. Liu, Y.-K. Wang, “Versatile Video Coding (Draft 8),” Joint Video Expert Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-Q2001, 17th Meeting: Brussels, BE, 7–17 January 2020.

Patent rights declaration(s)

Sharp Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).