| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0143-v2 |
| --- | --- |

| Title: | AHG14: Configuration parameter to enable TSRC for lossless coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Christopher Hollmann, Mitra Damghanian, Lukasz Litwic, Mikael von Strauss Torshamnsgatan 23 16480 Stockholm Sweden | Tel: Email: | {christopher.hollmann, mitra.damghanian, lukasz.litwic, mikael.von.strauss} @ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

In this contribution it is reported that Transform Skip Residual Coding was disabled for lossless compression in VTM-8.0. This contribution claims that this change reduces compression efficiency in non-CTC test cases. It is proposed to add a configuration parameter TSRCdisableLL to the VTM reference software, which allows to control the usage of Transform Skip Residual Coding in the lossless configuration.

The results presented in this contribution were generated using a non-CTC dataset. The CityScapes data set is known by the authors to be used in machine learning applications. The impact of allowing Transform Skip Residual Coding by setting the proposed parameter to false was measured using both 4:2:0 and 4:4:4 chroma sampling modes, and an increased compression efficiency is reported compared to VTM-8.0:

- 4:2:0: 2.6 percentage points in standard configuration, 2.3 percentage points in SCC,

- 4:4:4: 29.5 percentage points in standard configuration, 15.5 percentage points in SCC.

Version 2 of this documents adds a reference to a MPEG contribution comparing the performance to HEVC.

# Introduction

At the 17th JVET meeting in Brussels, Belgium, a proposal with several changes to the lossless coding mode, JVET-Q0089 [1] was adopted. Among the changes was a modification to the use of Transform Skip Residual Coding (TSRC). In version 8.0 of VTM, when operating in lossless mode, TSRC is not used. A parameter indicating this is sent in the slice header.

However, this change has a significant negative impact on the compression performance when using a non-CTC dataset. In our test we used the CityScapes dataset, which contains segmentation maps and is for example used in machine learning applications. Compression aspects of this dataset are also being investigated by the MPEG Video Coding for Machines AhG. The detailed setup of the test is described in section 3.

# Proposal

We propose to add a configuration parameter to the VTM reference software that allows to control whether to use TSRC or not for lossless coding on a configuration level. The software changes besides the parameter handling are minimal. The proposed name for the parameter is TSRCdisableLL and setting it to true, which is also the default value if it is not explicitly set in the configuration or command line, produces bit-exact results compared to VTM-8.0.

As we only propose to add a parameter to the encoder configuration, there are no changes to the VVC specification or the decoding process.

# Dataset and test set up

In our test we compress the segmentation maps of a publicly available dataset. We use the ground truth of the training part of the CityScapes dataset [2], which contains segmentation maps in lossless PNG format. These segmentation maps are converted to YUV 4:2:0 and YUV 4:4:4 using FFmpeg version 4.2.2 [3]. Then the raw data is compressed using different versions of VTM. Segmentation maps, including those in the CityScapes dataset, have large areas of constant colors and very sharp edges. They are usually generated by an algorithm. Figure 1 shows an example of a segmentation map from the CityScapes dataset. All images in this dataset have a resolution of 2048x1024 pixels and are coded using VTM in lossless all-intra configuration.

As anchor we use VTM-7.3, while the tests are done using VTM-8.0 and the proposed version of VTM-8.0 where the configuration parameter TSRCdisableLL has been added and set to false.

Attached to this contribution in the scripts.zip file are:

- Python scripts that generate shell scripts to:

  - Convert the PNG sources to YUV 4:2:0 and YUV 4:4:4 (create_conv.py and create_conv_444.py)

  - Compress the YUV files using VTM-7.3, VTM-8.0 and the proposed modified version of VTM-8.0 (create_vtm_7.3.py, create_vtm_8.0.py and create_vtm_prop.py)

- The configuration files used to generate the results

  - Note that both VTM-8.0 and the proposal use the same configuration file, but the TSRCdisableLL parameter is added in the command line in the create_vtm_prop.py script.

Figure 1. Example of a segmentation map (aachen_000000_000019) from the CityScapes dataset

# Results

Table 1 shows the results comparing VTM-8.0 and the proposed version using 4:2:0 with two configurations: as-is (lossless) on the left side and in screen content coding configuration with additionally DualITree disabled on the right side. While there is not a large difference, adding this configuration parameter and setting it to false increases the gain for this test set by 2.6 percentage points in standard configuration and by 2.3 percentage points in SCC configuration.

Table 1. Performance compared to VTM-7.3 for 4:2:0 configurations

|  | 4:2:0, lossless |  | 4:2:0, SCC, lossless, DT off |  |
| --- | --- | --- | --- | --- |
|  | VTM-8.0 | Proposed | VTM-8.0 | Proposed |
| All | -12.8% | -15.4% | -3.2% | -5.5% |

In Table 2 the results for 4:4:4 are shown, using the same configurations as above. Here the importance of setting this parameter to false for this type of data has a substantial impact on the results, as there is a significant decrease in compression performance for VTM-8.0 compared to VTM-7.3, while the proposed version maintains the previously achieved compression performance. Compared to the unmodified version of VTM-8.0, the proposed version reduces the loss by 29.5 percentage points in standard configuration and by 15.5 percentage points in SCC configuration.

Table 2. Performance compared to VTM-7.3 for 4:4:4 configurations

|  | 4:4:4, lossless |  | 4:4:4, SCC, lossless, DT off |  |
| --- | --- | --- | --- | --- |
|  | VTM-8.0 | Proposed | VTM-8.0 | Proposed |
| All | 29.6% | 0.1% | 17.1% | 1.6% |

It is also noted that there is a related MPEG contribution m53555 [4], which compares the performance of VTM for the same use case as investigated in this contribution to HM-16.21 and its screen content coding extension SCM-8.8.

# Conclusion

As the results in section 4 indicate, disabling TSRC has a considerable negative impact on the compression performance, especially for 4:4:4 coding, for this dataset. We therefore propose to include the described TSRCdisableLL parameter in the next version of VTM, as this is a straight-forward solution to prevent the considerable loss in compression efficiency for some non-CTC datasets.

# References

- A. Nalci et al. CE3-2.4: Luma and Chroma BDPCM for Lossless Coding with Regular Residual Coding. JVET-Q0089, Brussels, Belgium. January 2020.

- Cordts, Marius et al. “The Cityscapes Dataset for Semantic Urban Scene Understanding”. In: 2016 IEEE Conference on Computer Vision and Pattern Recognition (CVPR). June 2016, pp. 3213–3223. DOI: 10.1109/CVPR.2016.350.

- FFmpeg. https://ffmpeg.org/, accessed 2020-03-27

- C. Hollmann et al. “[VCM] Study of compression methods for segmentation maps”. MPEG input contribution m53555, April 2020.

# Patent rights declaration(s)

Ericsson does not have any current or pending patent rights relating to the technology described in this contribution.