| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0144 |
| --- | --- |

| Title: | AHG14: On lossless operation with RRC |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | J. Gan, C. Rosewarne Building A, 5 Talavera Rd, Macquarie Park, 2113, NSW, Australia | Tel: Email: | +61 2 9805 2844 jonathan.gan@canon.com.au chris.rosewarne@canon.com.au |
| Source: | Canon Australia Pty Ltd |  |  |

_____________________________

# Abstract

This contribution asserts that “slice_ts_residual_coding_disabled_flag” introduces a bug for mixed lossy/lossless coding when either of the dependent quantization or sign bit hiding coding tools are also enabled. It is proposed to fix this bug for each of the DQ and SBH coding tools by additionally conditioning their application on the transform_skip_flag.

# Introduction

The adoption of Q0089 [1] introduced a slice-level flag “slice_ts_residual_coding_disabled_flag”. This flag was introduced because it was shown that the regular residual coding (RRC) process was generally found more efficient than transform skip residual coding (TSRC) for lossless mode.

With lossless mode enabled by a combination of setting QP and transform skip flag, it is intended that the VVC standard should have sufficient flexibility to allow mixed lossy/lossless coding. Lossy/lossless coding can be switched by QP delta signalling, which is achievable at quantization group granularity. However, lossless mode is not possible if either of the following combinations are set by the encoder:

- slice_ts_residual_coding_disabled_flag==1 and ph_dep_quant_enabled_flag==1

- slice_ts_residual_coding_disabled_flag==1 and pic_sign_data_hiding_enabled_flag==1

The DQ and SBH tools cannot be switched at QG granularity. Therefore, in the current working draft Q2001 a mixed lossy/lossless encoder would need to disable both DQ and SBH coding tools. It is asserted that this restriction causes a significant and unnecessary coding loss in lossy regions.

# Proposed method

## Aspect 1: Dependent quantisation

In the current working draft, in section 7.3.10.11 the TransCoeffLevel variable is reconstructed conditioned on

( ph_dep_quant_enabled_flag )

The proposed method changes this condition to

( ph_dep_quant_enabled_flag && !transform_skip_flag[ x0 ][ y0 ][ cIdx ] )

## Aspect 2: Sign bit hiding

In the current working draft, sign bit hiding is disabled if the following condition is true:

( ph_dep_quant_enabled_flag  | |  !pic_sign_data_hiding_enabled_flag )

The proposed method changes this condition to

( ph_dep_quant_enabled_flag  | |  !pic_sign_data_hiding_enabled_flag | |

transform_skip_flag[ x0 ][ y0 ][ cIdx ] )

# Experimental results

# References

- A. Nalci, H. Wang, M. Coban, M. Karczewicz, T. Tsukuba, M. Ikeda, Y. Yagasaki, T. Suzuki, Z.-Y. Lin, T.-D. Chuang, C.-Y. Chen, C.-W. Hsu, Y.-W. Huang, S.-M. Lei, T.-C. Ma, H.-J. Jhu, X. Xiu, Y.-W. Chen, X. Wang, “CE3-2.4: Luma and Chroma BDPCM for Lossless Coding with Regular Residual Coding”, JVET-Q0089, Brussels, BE, Jan 2020.

# Patent rights declaration(s)

Canon Inc may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Canon Australia Pty Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).