| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0144 |
| --- | --- |

| Title: | AHG14: On lossless operation with RRC |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | J. Gan, C. Rosewarne Building A, 5 Talavera Rd, Macquarie Park, 2113, NSW, Australia | Tel: Email: | +61 2 9805 2844 jonathan.gan@canon.com.au chris.rosewarne@canon.com.au |
| Source: | Canon Australia Pty Ltd |  |  |

_____________________________

# Abstract

This contribution asserts that “slice_ts_residual_coding_disabled_flag” introduces a bug for mixed lossy/lossless coding when either of the dependent quantization or sign data hiding coding tools are also enabled. It is proposed to fix this bug for each of the DQ and SDH coding tools by additionally conditioning their application on the transform_skip_flag.

# Introduction

The adoption of Q0089 [1] introduced a slice-level flag “slice_ts_residual_coding_disabled_flag”. This flag was introduced because it was shown that the regular residual coding (RRC) process was generally found more efficient than transform skip residual coding (TSRC) for lossless mode.

With lossless mode enabled by a combination of setting QP and transform skip flag, it is intended that the VVC standard should have sufficient flexibility to allow mixed lossy/lossless coding. Lossy/lossless coding can be switched by QP delta signalling, which is achievable at quantization group granularity. However, lossless mode is not possible if either of the following combinations are set by the encoder:

- slice_ts_residual_coding_disabled_flag==1 and ph_dep_quant_enabled_flag==1

- slice_ts_residual_coding_disabled_flag==1 and pic_sign_data_hiding_enabled_flag==1

The DQ and SDH tools cannot be switched at QG granularity. Therefore, in the current working draft Q2001 a mixed lossy/lossless encoder would need to disable both DQ and SDH coding tools. It is asserted that this restriction causes a significant and unnecessary coding loss in lossy regions.

# Proposed method

## Aspect 1: Dependent quantisation

In the current working draft, in section 7.3.10.11 the TransCoeffLevel variable is reconstructed conditioned on

( ph_dep_quant_enabled_flag )

The proposed method changes this condition to

( ph_dep_quant_enabled_flag && !transform_skip_flag[ x0 ][ y0 ][ cIdx ] )

## Aspect 2: Sign data hiding

In the current working draft, sign data hiding is disabled if the following condition is true:

( ph_dep_quant_enabled_flag  | |  !pic_sign_data_hiding_enabled_flag )

The proposed method changes this condition to

( !pic_sign_data_hiding_enabled_flag | | transform_skip_flag[ x0 ][ y0 ][ cIdx ] )

Aspect 2 includes the removal of ph_dep_quant_enabled_flag from the condition, which is asserted to be a purely editorial change. In working draft Q2001 [2], when ph_dep_quant_enabled_flag is 1, pic_sign_data_hiding_enabled_flag must be inferred as 0. Therefore, including ph_dep_quant_enabled_flag in the condition is redundant.

# Experimental results

Aspect 1 aligns the working draft with the behaviour of the VTM-8.0 software, so there is no corresponding test.

Aspect 2 was tested on lossless conditions defined by Q2014 [3] with the additional encoder setting “--SignHideFlag=1”.

Table 1: Aspect 2 over VTM-8.0 in JVET-Q2014 4:2:0 conditions

|  | All Intra |  |  | Random Access |  |  | Low delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | ref | test |  | ref | test |  | ref | test |  |
| Class A1 | 2.4 | 2.4 | 0.00% | 2.4 | 2.4 | 0.00% |  |  |  |
| Class A2 | 1.8 | 1.8 | 0.00% | 1.9 | 1.9 | 0.00% |  |  |  |
| Class B | 2.3 | 2.3 | 0.00% | 2.5 | 2.5 | 0.00% | 2.4 | 2.4 | 0.00% |
| Class C | 2.1 | 2.1 | 0.00% | 2.6 | 2.6 | 0.00% | 2.6 | 2.6 | 0.00% |
| Class D | 2.1 | 2.1 | 0.00% | 2.9 | 2.9 | 0.00% | 2.9 | 2.9 | 0.00% |
| Class E | 3.1 | 3.1 | 0.00% |  |  |  | 3.4 | 3.4 | 0.00% |
| Class F | 5.8 | 5.8 | 0.00% | 35.4 | 35.4 | 0.00% | 51.8 | 51.8 | 0.00% |
| TGM | 12.4 | 12.4 | 0.00% | 109.0 | 109.0 | 0.00% | 122.8 | 122.8 | 0.00% |
| Overall | 2.3 | 2.3 | 0.00% | 2.4 | 2.4 | 0.00% | 2.7 | 2.7 | 0.00% |
| Enc Time[%] | 100% |  |  | 101% |  |  | 101% |  |  |
| Dec Time[%] | 100% |  |  | 102% |  |  | 103% |  |  |

Table 2: Aspect 2 over VTM-8.0 in JVET-Q2014 4:4:4 conditions

| RGB | All Intra |  |  | Random Access |  |  | Low delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | ref | test |  | ref | test |  | ref | test |  |
| TGM 1080p | 10.9 | 10.9 | 0.00% | 21.6 | 0.0 | #N/A | 31.4 | 0.0 | #N/A |
| TGM 720p | 5.0 | 5.0 | 0.00% | 6.4 | 0.0 | #N/A | 11.3 | 0.0 | #N/A |
| A | 3.0 | 3.0 | 0.00% | 1.1 | 0.0 | #N/A | 0.9 | 0.0 | #N/A |
| M | 3.0 | 3.0 | 0.00% | 2.4 | 0.0 | #N/A | 2.6 | 0.0 | #N/A |
| CC | 0.9 | 0.9 | 0.00% | 0.1 | 0.0 | #N/A | 0.1 | 0.0 | #N/A |
| Overall | 5.2 | 5.2 | 0.00% | 7.7 | 0.0 | #N/A | 11.3 | 0.0 | #N/A |
| Enc Time[%] | 100% |  |  | 0% |  |  | 0% |  |  |
| Dec Time[%] | 103% |  |  | 653% |  |  | 651% |  |  |
| YUV | All Intra |  |  | Random Access |  |  | Low delay B |  |  |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | ref | test |  | ref | test |  | ref | test |  |
| TGM 1080p | 10.7 | 10.7 | 0.00% | 21.8 | 0.0 | #N/A | 31.6 | 0.0 | #N/A |
| TGM 720p | 6.4 | 6.4 | 0.00% | 8.1 | 0.0 | #N/A | 14.5 | 0.0 | #N/A |
| A | 3.5 | 3.5 | 0.00% | 1.0 | 0.0 | #N/A | 1.0 | 0.0 | #N/A |
| M | 3.9 | 3.9 | 0.00% | 3.3 | 0.0 | #N/A | 3.6 | 0.0 | #N/A |
| CC | 1.0 | 1.0 | 0.00% | 0.1 | 0.0 | #N/A | 0.1 | 0.0 | #N/A |
| Overall | 5.8 | 5.8 | 0.00% | 8.3 | 0.0 | #N/A | 12.4 | 0.0 | #N/A |
| Enc Time[%] | 100% |  |  | 0% |  |  | 0% |  |  |
| Dec Time[%] | 101% |  |  | 683% |  |  | 710% |  |  |

# References

- A. Nalci, H. Wang, M. Coban, M. Karczewicz, T. Tsukuba, M. Ikeda, Y. Yagasaki, T. Suzuki, Z.-Y. Lin, T.-D. Chuang, C.-Y. Chen, C.-W. Hsu, Y.-W. Huang, S.-M. Lei, T.-C. Ma, H.-J. Jhu, X. Xiu, Y.-W. Chen, X. Wang, “CE3-2.4: Luma and Chroma BDPCM for Lossless Coding with Regular Residual Coding”, JVET-Q0089, Brussels, BE, Jan 2020.

- B. Bross, J. Chen, S. Liu, Y.-K. Wang, “Versatile Video Coding (Draft 8)”, JVET-Q2001, Brussels, BE, Jan 2020.

- T.-C. Ma, A. Nalci, T. Nguyen, “JVET common test conditions and software reference configurations for lossless, near lossless, and mixed lossy/lossless coding”, JVET-Q2014, Brussels, BE, Jan 2020.

# Patent rights declaration(s)

Canon Inc may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Canon Australia Pty Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).