| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0146 |
| --- | --- |

| Title: | AHG11: Context coded bin limits for palette coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | J. Gan, C. Rosewarne Building A, 5 Talavera Rd, Macquarie Park, 2113, NSW, Australia | Tel: Email: | +61 2 9805 2844 jonathan.gan@canon.com.au chris.rosewarne@canon.com.au |
| Source: | Canon Australia Pty Ltd |  |  |

_____________________________

# Abstract

To limit the required throughput of the CABAC engine, the number of context coded bins is limited to 1.75 bins per sample. This limit is explicitly calculated and enforced in the residual coding process. In palette coding, the number of context coded bins is not explicitly limited. With the adoption of JVET-Q0504, an “evil” bitstream can be constructed that would reach 1.984 context coded bins/sample. This contribution proposes a number of options for solving this bug.

# Introduction

In working draft Q2001 [1], the complexity of the CABAC engine is constrained by limiting the number of context coded bins in a transform block to 1.75 bins per sample. This limit is explicitly calculated and enforced in the residual coding process (section 7.3.10.11) by switching to bypass coding when the limit is reached.

In palette coding, the number of context coded bins is not explicitly limited. There are two syntax elements (run_copy_flag and copy_above_flag) which may each be signalled once per “palette coefficient”. Each “palette coefficient” may correspond to multiple samples, depending on the chroma format and the colour components corresponding to the palette coding CU. However, with the adoption of Q0504 [2], in the worst case each “palette coefficient” corresponds to just one sample. The worst case may be reached in:

- Palette mode CU in DUAL_TREE_LUMA

- Palette mode CU in SINGLE_TREE_LUMA on a 4:0:0 video sequence

Then, an “evil” bitstream can be constructed that would result in a palette mode CU exceeding the context coded bins limit defined for residual coding. For example:

For a 64x64 palette CU in DUAL_TREE_LUMA, the CU consists of 4096 samples

- Other than the first sample, run_copy_flag is context coded with the value 0. (4095 bins)

- Other than the first row/column, copy_above_flag is context coded with the value 0. (4032 bins)

- Context coded bins/sample rate = 1.984

# Proposed methods

## Option 1: Explicitly track context coded bin budget in palette mode

Unchanged behaviour for run_copy_flag, ie: always decode run_copy_flag by context coding (apart from first coefficient). Initialise the remaining context coded bins as 1.75 bins/sample – (cbWidth * cbHeight). Decrement the remaining bins whenever copy_above_flag is decoded by context coding. When the remaining bins are exhausted:

- Option 1a: (Preferred method) Infer copy_above_flag as 0.

- Option 1b: Decode copy_above_flag with bypass bin.

## Option 2: Disable palette mode in worst cases

The problem is eliminated if palette mode is disabled for DUAL_TREE_LUMA, and completely disabled for 4:0:0 video sequences.

It is unclear whether this solution is desirable. The run coding of palette mode may be highly efficient in certain 4:0:0 use cases (eg bi-level video sequence).

# Alternative solutions (not preferred)

- Because palette coding has a natural limit of 2 context coded bins/sample, the problem can be eliminated by also raising the limit for residual coding to 2 bins per sample. This solution is not recommended because it would increase throughput requirements of the CABAC coder by 14% at a very late stage of the standardisation process.

- Any bitstream containing a palette CU using more context coded bins than 1.75 bins/sample could be declared illegal, by adding a conformance statement. This solution is not preferred because despite declaring such bitstreams as illegal, in practice an illegal bitstream could still choke a real-time decoder unless the context coded bins are counted.

# Experimental results

# Additional results (informative)

# References

- B. Bross, J. Chen, S. Liu, Y.K. Wang, “Versatile Video Coding (Draft 8)”, JVET-Q2001, Brussels, BE, Jan 2020.

- R.-L. Liao, M. Sarwer, J. Chen, Y. Ye, J. Luo, Y.-H. Chao, C.-H. Hung, W.-J. Chien, V. Seregin, M. Karczewicz, “CE2-related: Palette mode for non 4:4:4 color format”, JVET-Q0504, Brussels, BE, Jan 2020.

# Patent rights declaration(s)

Canon Inc may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Canon Australia Pty Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).