| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0148-v1 |
| --- | --- |

| Title: | AHG12: Subpicture merging experiments |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Antti Hallapuro, Maryam Homayouni, Alireza Aminlou, Miska M. Hannuksela Nokia Technologies, Tampere, Finland | Email: | firstname.lastname@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

This contribution reports successful experiments carried out with the following subpicture-based system:

- Encoding several bitstreams, each with one subpicture per picture, using the VTM encoder.

- Merging selected encoded bitstreams into a bitstream with multiple subpictures, using a merger software developed by the authors.

- Decoding the bitstream having multiple subpictures, using the VTM decoder.

The implemented merger software and the performed experiments give the authors high confidence that both the subpicture feature in the VVC specification and the subpicture implementation in the VTM software are mature.

# Introduction

This contribution provides information on encoding and bitstream merging that can be used for selective subpicture-based system operation. The system setup is illustrated in Figure 1 and described briefly below:

- Splitting to subpicture sequences
An original uncompressed video clip is spatially split into uncompressed subpicture sequences.

- VVC encoding
Each uncompressed subpicture sequence is encoded independently with an encoding configuration that generates only one subpicture per picture. The VTM encoder was used in the performed experiments.

- Bitstream selection
Any subset of the encoded bitstreams is selected. For example, in viewport-dependent streaming, the bitstreams covering the viewport could be selected.

- Merging
The selected bitstreams are merged into one bitstream with multiple subpictures. Each input bitstream corresponds to a certain subpicture in the subpicture layout of the output bitstream. A merger software was developed for performing this step.

- VVC decoding
The bitstream with multiple subpictures is decoded. The VTM decoder was used in the performed experiments.

Figure 1. System for selective subpicture-based operation.

Our motivation and asserted benefits for the illustrated system setup include the following:

- It is envisioned that there could be VVC encoder implementations that do not provide support for multiple subpictures. An intent of the performed experiments was to verify that any encoders can be used to generate encoded content that can be merged as subpictures into a VVC bitstream.

- It is assumed that parallel encoding of subpictures, e.g. in separate processing cores, can be more easily achieved with the tested encoding setup than with an encoder that natively supports encoding multiple subpictures per picture. With the given approach, the encoder implementation does not need to include handling of separate processes or threads per each subpicture. The encoding approach used in the experiments could be applied e.g. in a cloud-based encoding setup.

- One of the goals of the subpicture design in the VVC standard is to enable straightforward merging of subpictures. The merger software intends to serve as evidence that this design goal has been successfully met.

- The merger software can be regarded as an independent implementation that generates bitstreams with multiple subpictures. However, it is remarked that merger software shares some VTM source code modules for high-level syntax parsing and generation.

- The system setup can also be used to verify that the VTM decoder is capable of decoding bitstreams that have multiple selectable subpictures and originate from another source than directly from the VTM encoder.

- The merger software implements operations that are needed in players that select a subset of available subpictures to be received and/or decoded. For example, players that support viewport-dependent 360° streaming are expected to include subpicture merging. The merger software can be considered to be a building block in such players.

- The approach where the video clip is split into subpicture sequences before encoding was practical for our purposes, since the work was started already before the VTM encoder supported multiple subpictures.

# Merger software

The merger tool takes multiple subpicture bitstreams, along with their dimensions (width and height) and target on-screen locations, as an input and generates a merged bitstream. The tool integrates with VTM encoder and decoder libraries so that useful parsing and coding functions from VTM can be used easily.

The tool parses all parameter sets, picture header NAL units (if they exist) and slice headers of all subpictures for the current time instance. It is assumed that parameter sets and picture headers of subpicture bitstreams have the same features enabled so that common parameter sets can be shared in the merged stream. It is also assumed that APS NAL units having the same ID and type have identical content.

The merger tool makes the following modifications to the NAL units:

- All parameter set and picture header NAL units of the input bitstreams are discarded except for the those of the first subpicture that are used as the basis for the merged stream.

- Subpicture information is written to the SPS(s) of the merged stream.

- Picture partitioning information is written to the PPS(s) of the merged stream. Each subpicture is to have one tile and one slice.

- Picture headers are changed to be coded as PH NAL units, if they were originally coded within slice header.

- Subpicture IDs are written to the slice headers, if the slice headers did not originally contain them.

- All SEI NAL units are discarded in the current implementation.

- All other non-VCL NAL units are taken from the bitstream of the first subpicture only.

Modified NAL units, slice headers, remaining non-VCL NAL units and VCL NAL slice data sections from the subpictures are then written to the target bitstream in the correct decoding order.

# Performed experiments

Experiments were carried out using the SkateboardInLot 8K (8192×4096) 360° video test sequence, defined in [2]. As described in JVET-M0870 [1], the test sequence was first converted to the CMP format with a face size of 2048×2048 using JVET 360Lib-10.0 software ‎[3]. The CMP video was then partitioned to individual subpictures of size 512×512 before encoding. In other words, the 2048×2048 cube faces were split to 4×4 subpictures, resulting in 96 individual subpictures.

The subpicture sequences were encoded with the VTM master branch dated 2020-03-25, which was fixed with merge request #966 (https://jvet.hhi.fraunhofer.de/trac/vvc/ticket/966). The random access (RA) common test configuration was used except that the length of the bitstreams was limited to 17 or 64 frames in different tests and the following parameters were turned off in order to enable subpicture merging:

- ALF

- LMCS

- Scaling list

- Joint chroma coding residual sign flag was forced to a constant value (with a software change in encoder)

- SAO

- AMaxBT: since max BT is signalled in the picture header, having this parameter configured to 1 sometimes causes maximum BT size to be different for different subpictures.

The resulting 96 subpicture bitstreams were then combined with the merger tool into a single merged bitstream with multiple subpictures. The merged bitstream was decoded with the VTM decoder (of the master branch dated 2020-03-25, which was fixed with merge request #966). The decoded sequences were verified to be correct in content. The experiment was repeated with different QP values.

# References

- M. Coban, R. Skupin, “AHG12: Proposed JVET common test conditions and evaluation procedures for MCTS and sub-pictures with boundary padding,” JVET-M0870, Marrakech, MA, Jan. 2019.

- P. Hanhart, J. Boyce, K. Choi, J.-L. Lin, “JVET common test conditions and evaluation procedures for 360°,” JVET-L1012, Oct. 2018.

- 360Lib-10.0. [Online]. Available: https://jvet.hhi.fraunhofer.de/svn/svn_360Lib/tags/360Lib-10.0/