| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0149-v1 |
| --- | --- |

| Title: | AHG9/AHG12: Relaxing an APS constraint |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Miska M. Hannuksela, Maryam Homayouni, Antti Hallapuro, Alireza Aminlou Nokia Technologies, Tampere, Finland | Email: | firstname.lastname@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

VVC Draft 8 includes a constraint that the content of all APS NAL units with a particular value of adaptation_parameter_set_id and a particular value of aps_params_type within a picture unit shall have the same content. This contribution asserts that, because of this constraint, ALF needs to be turned off in many applications using adaptive subpicture-based delivery or decoding. To enable the use of ALF in such applications, the contribution proposes:

- To change the constraint on APS NAL units to have the same content within a picture unit to apply within a subpicture.

- To disallow interleaving of APS NAL units of different subpictures.

# Problem Statement

## APS related constraints in the VVC draft text

VVC Draft 8 constrains the content of APS NAL units with the following restriction:

All APS NAL units with a particular value of adaptation_parameter_set_id and a particular value of aps_params_type within a PU, regardless of whether they are prefix or suffix APS NAL units, shall have the same content.

This restriction originates from the following adoption in the JVET-M meeting (bolding added):

Decision: Adopt an adaptation parameter set (APS) to carry ALF parameters. The tile group header contains an aps_id which is conditionally present when ALF is enabled. The APS contains an aps_id and the ALF parameters. A new NUT value is assigned for APS (from JVET-M0132). For the CTC, we will just use aps_id = 0 and send the APS with each picture. For now, the range of APS ID values will be 0..31 and APSs can be shared across pictures (and can be different in different tile groups within a picture). The ID value should be fixed-length coded when present. ID values cannot be re-used with different content within the same picture.

Further study was encouraged for making use of shared values across pictures and different APSs in the same picture. Further study was also encouraged on whether to relax the constraint on re-using ID values.

The maximum number of different types of APSs was constrained in the JVET-O meeting. In the understanding of the authors of this contribution, the motivation for specifying these maximums was bounding the "local" memory usage.

## Subpicture extraction and merging

Either of the following encoding strategies are possible for use cases involving subpicture extraction and merging:

- Encoding several versions of the content for different bitrates and/or resolutions, each version with multiple subpictures. Extraction of the coded subpicture sequences from the bitstreams for selective delivery and/or decoding.

- Splitting the uncompressed video into subpicture sequences before encoding, and encoding several versions of each subpicture sequence, each for different bitrates and/or resolutions. This approach has been discussed in more details in JVET-R0148.

In both encoding approaches, the content of ALF APSs is selected adaptively based on the content and it is typically not possible to avoid using the same ALF APS ID values in different bitstreams due to the limited value range (0..7). Consequently, to enable merging of subpictures originating from different bitstreams, ALF needs to be turned off, which may cause a remarkable loss in rate-distortion performance. For example, an average loss of 4.04% in luma BD-bitrate was observed in the experiments for 360° video discussed in Section 4.

# Proposal

The proposed changes are summarized as follows:

- Change the constraint on APS NAL units to have the same content within a picture unit to apply within a subpicture.

- Disallow interleaving of APS NAL units of different subpictures.

The following specification text changes are proposed:

All APS NAL units with a particular value of adaptation_parameter_set_id and a particular value of aps_params_type and associated with the VCL NAL units of a single subpicturewithin a PU, regardless of whether they are prefix or suffix APS NAL units, shall have the same content.

A prefix APS NAL unit is associated with the subpicture that contains the next VCL NAL unit in decoding order. A postfix APS NAL unit is associated with the subpicture that contains the previous VCL NAL unit in decoding order. When both one or more prefix APS NAL units and one or more postfix APS NAL units are present between two consecutive VCL NAL units in decoding order, the VCL NAL units shall belong to different subpictures and all the postfix APS NAL units shall precede, in decoding order, all the prefix APS NAL units.

# Further information: envisioned system operation

MPEG N19049 contains a working draft for the carriage of VVC in ISOBMFF. Among other things it specifies subpicture tracks, which are meant for storing coded subpicture sequences. It is envisioned that subpicture tracks are used in streaming when spatial adaptation of the content is needed in the application. For example, OMAF already specifies viewport-dependent delivery with HEVC tiling profiles, where tile/sub-picture tracks are selectively streamed by an OMAF player. Thus, it is expected that immersive video applications store VVC content in subpicture tracks and use a system like illustrated in Figure 1.

If the subpicture tracks use overlapping APS ID values and the VVC standard is modified as proposed in this contribution, the merging process in Figure 1 can operate as follows:

- The merging process keeps a copy of all APS NAL units of the received subpicture tracks.

- Just prior to inserting a subpicture from a subpicture track to the merged bitstream, the merging process inserts the APS NAL unit(s) applicable to that subpicture into the merged bitstream.

While the process above could result into many APS NAL units being inserted into the merged bitstream, the delivery bitrate is not increased, since the subpicture merging takes place in the player. MPEG N19049 also allows to share APS NAL units among many subpicture tracks by storing them into a non-VCL track.

Figure 1. Envisioned system operation for subpicture-based delivery.

# Further information: impact of ALF in 360° video with subpictures

Simulations were carried out using 300 frames for each of 360° video test sequences listed in Table 1. As described in JVET-M0870 ‎[2], the 6K and 8K 360° video test sequences, defined in ‎[3], were first converted to the CMP format with a face size of 1536x1536 and 2048x2048, respectively, using JVET 360Lib-10.0 software ‎[4].

Table 1. Test sequences used in this study

| Class | Sequence name | ORG ERP Resolution@FPS | CMP Resolutinos | Frame count | Bit depth |
| --- | --- | --- | --- | --- | --- |
| S1 | SkateboardInLot | 8192×4096@30 | 6144×4096 | 300 | 10 |
| S1 | ChairLift | 8192×4096@30 | 6144×4096 | 300 | 10 |
| S1 | KiteFlite | 8192×4096@30 | 6144×4096 | 300 | 8 |
| S1 | Harbor | 8192×4096@30 | 6144×4096 | 300 | 8 |
| S1 | Trolley | 8192×4096@30 | 6144×4096 | 300 | 8 |
| S1 | GasLamp | 8192×4096@30 | 6144×4096 | 300 | 8 |
| S2 | Balboa | 6144×3072@60 | 4608×3072 | 300 | 8 |
| S2 | Broadway | 6144×3072@60 | 4608×3072 | 300 | 8 |
| S2 | Landing2 | 6144×3072@60 | 4608×3072 | 300 | 8 |
| S2 | BranCastle2 | 6144×3072@60 | 4608×3072 | 300 | 8 |

Uncompressed video was then partitioned to individual subpictures of size 512×512 before encoding and were encoded individually. In other words, each 1536×1536 cube face of a 6K sequence was split to 3×3 subpictures, while the 2048×2048 cube faces of 8K sequences were split to 4×4 subpictures, resulting in 54 and 96 individual subpictures, respectively. The subpicture sequences were then coded with VTM 8.0 ‎[1], using random access (RA) configuration. LMCS in SPS, scaling list in SPS, SAO and Joint Chroma sign flag were disabled for all tests. This subpicture encoding approach has been discussed in more details and found valid in JVET-R0148.

All the subpictures/sequences were encoded with quantization values of 20, 23, 26, and 29. This range of values was selected to obtain acceptable picture quality for viewing on a head-mounted display.

The effect of ALF was investigated by encoding each sequence twice, once with ALF turned on and another time with ALF turned off. The results are presented in Table 2. As shown in Table 2, turning off ALF causes an average loss of 4.04 % in luma BD-bitrate.

Table 2. VTM-8.0 SubPicture coding ALF off vs ALF on

|  | VTM-8.0 SubPicture coding |  |  |
| --- | --- | --- | --- |
|  | ALF on vs. ALF on |  |  |
|  | Y | U | V |
| Class S1 | 3.56% | 4.99% | 4.86% |
| Class S2 | 4.52% | 3.86% | 3.54% |
| Overall | 4.04% | 4.42% | 4.20% |

# References

- VTM8.0, reference software for VVC. [Online]. Available: https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/-/tags/VTM-8.0

- M. Coban, R. Skupin, “AHG12: Proposed JVET common test conditions and evaluation procedures for MCTS and sub-pictures with boundary padding,” JVET-M0870, Marrakech, MA, Jan. 2019.

- P. Hanhart, J. Boyce, K. Choi, J.-L. Lin, “JVET common test conditions and evaluation procedures for 360°,” JVET-L1012, Oct. 2018.

- 360Lib-10.0. [Online]. Available: https://jvet.hhi.fraunhofer.de/svn/svn_360Lib/tags/360Lib-10.0/

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).