| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0154_v1 |
| --- | --- |

| Title: | AHG9/16 : On sign data hiding for BDPCM blocks |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sunmi Yoo Jungah Choi Jaehyun Lim 19 Yangjaedaero-11gil Seocho-gu, Seoul 06772 South Korea Seunghwan Kim 10225 Willow Creek Rd San Diego, CA 92131 United States | Tel: Email: | +82-2-6912-6561 sunmi.yoo@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

- Abstract

The current VVC draft specification could allow both the sign data hiding (SDH) and BDPCM to be enabled. However, the levels of the coefficients can be modified by BDPCM, and it can affect the SDH decision in the residual coding. This document proposes two control mechanisms to prevent the unintended usage of sign data hiding (SDH) when BDPCM is enabled and transform skip residual coding is disabled. First, a flag to enable sign data hiding (sps_sign_data_hiding_enabled_flag) is not signalled when BDPCM is enabled. Second, sign data hiding is disabled in each residual coding block when corresponding block is BDPCM coded.

Problem Statement

At the Brussels meeting, slice_ts_residual_coding_disabled_flag is introduced to determine which residual coding method is used when transform_skip_flag is equal to 1[1]. In the case that the syntax element slice_ts_residual_coding_disabled_flag is set equal to 1, transform skip residuals can be coded using the same syntax of regular residual coding (RRC). Therefore, it is possible to enable SDH to the BDPCM coded block. However, it can cause mismatches between the encoder and decoder because BDPCM changes the level of the residuals which affects the SDH decision in the residual coding.

Proposed spec

Method 1 : High level change

To prevent SDH for the BDPCM block, high level switch for SDH can be not transmitted and implicitly set equal to 0 when sps_bdpcm_enabled_flag is equal to 1. The proposed method can be specified underlined like below :

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … | u(1) |
| if( !( sps_dep_quant_enabled_flag ¦ ¦ sps_bdpcm_enabled_flag ) ) |  |
| sps_sign_data_hiding_enabled_flag | u(1) |
| … |  |
| } |  |

Alternatively, the syntax element sps_sign_data_hiding_enabled_flag can be restricted by the conformance requirement.

- 7.3.2.3 Sequence parameter set RBSP syntax … sps_sign_data_hiding_enabled_flag equal to 0 specifies that sign bit hiding is disabled for pictures referring to the SPS. sps_sign_data_hiding_enabled_flag equal to 1 specifies that sign bit hiding may be enabled for pictures referring to the SPS. When sps_sign_data_hiding_enabled_flag is not present, it is inferred to be equal to 0. It is a requirement of bitstream conformance that when the value of sps_bdpcm_enabled_flag is equal to 1, the value of sps_sign_data_hiding_enabled_flag shall be equal to 0. …

Method 2 : Low level change

To fix the undesirable case described above, it is proposed to check whether the current block is coded with BDPCM when using SDH is determined. The proposed method can be specified underlined like below :

| residual_coding( x0, y0, log2TbWidth, log2TbHeight, cIdx ) { | Descriptor |  |
| --- | --- | --- |
| … |  |
| if( ph_dep_quant_enabled_flag ¦ ¦ !pic_sign_data_hiding_enabled_flag ¦ ¦ BdpcmFlag[ x0 ][ y0 ][ cIdx ] ) |  |
| signHidden = 0 |  |
| else |  |
| signHidden = ( lastSigScanPosSb − firstSigScanPosSb > 3 ? 1 : 0 ) |  |
| … |  |
| } |  |  |

Reference(s)

- B. Bross, J. Chen, S. Liu, and Y. Wang “Versatile Video Coding (Draft 8),” Document of Joint Video Experts Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-Q2001, 17th Meeting: Brussels, BE, 7–17 Jan. 2020.

Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).