| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0158-v1 |
| --- | --- |

| Title: | Semantic bug fixes for syntax elements in VPS and SPS |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Biao Wang, Semih Esenlik, Han Gao, Anand Meher Kotra, Elena Alshina | Tel: Email: | biao.wang@huawei.com |
| Source: | Huawei Technologies Co., Ltd |  |  |

_____________________________

# Abstract

This contribution proposes following semantics bug fixes for VPS and SPS syntax elements:

- infer vps_layer_id [0] equal to nuh_layer_id of the first VCL NAL unit in a bitstream when vps_layer_id [0] is not signaled

- fix the semantics of sps_max_sublayers_minus1 when vps_max_sublayers_minus1 is not present

# Proposed semantics bug fixes

For the latest working draft of VVC [1], the following semantics bug fixes are proposed.

## Semantics of vps_max_layers_minus1

It is asserted that an inferred value of vps_layer_id [0] is required. vps_layer_id [0] is always signaled in VPS but VPS is not necessarily present in a bitstream. Meanwhile, the value of vps_layer_id [0] is used  to check bitstream complaince. Missing inferred value of vps_layer_id [0] resulting undefined behavior in such compliance checking.

For example, in clause 7.4.2.2 for checking the constraint of AUD NAL units

‘‘‘

The value of nuh_layer_id for AUD, PH, EOS, and FD NAL units is constrained as follows:

–	If nal_unit_type is equal to AUD_NUT, nuh_layer_id shall be equal to vps_layer_id[ 0 ].

’’’

However, when VPS is not present, the vps_layer_id [0] is undefined, and such a constraint cannot be checked.

According to the constraint of sps_video_parameter_set_id in specification

‘‘‘

When sps_video_parameter_set_id is equal to 0, the following applies:

–	The SPS does not refer to a VPS.

–	No VPS is referred to when decoding each CLVS referring to the SPS.

–	The value of vps_max_layers_minus1 is inferred to be equal to 0.

–	The CVS shall contain only one layer (i.e., all VCL NAL unit in the CVS shall have the same value of nuh_layer_id).

–	The value of GeneralLayerIdx[ nuh_layer_id ] is inferred to be equal to 0.

–	The value of vps_independent_layer_flag[ GeneralLayerIdx[ nuh_layer_id ] ] is inferred to be equal to 1.

’’’

And definition of GeneralLayerIdx

‘‘‘

for( i = 0; i  <=  vps_max_layers_minus1; i++ )		(38)
	GeneralLayerIdx[ vps_layer_id[ i ] ] = i

’’’

When VPS is not present, the sps_video_parameter_set_id can be set to 0 and therefore there is only one layer in CVS (i.e. vps_max_layers_minus1 is inferred to be equal to 0). GeneralLayerIdx[ vps_layer_id[ 0] ] is equal to 0 and vps_layer_id[ 0] shall be equal to nuh_layer_id. Since there is only one layer, the nuh_layer_id shal be equal to the layer id of a VCL NAL unit in the CVS, since in specification containing one layer means “all VCL NAL unit in the CVS shall have the same value of nuh_layer_id”.

Therefore, it is proposed to add the following highlighted text for semantics of vps_layer_id[i]:

‘‘‘

vps_layer_id[ i ] specifies the nuh_layer_id value of the i-th layer. For any two non-negative integer values of m and n, when m is less than n, the value of vps_layer_id[ m ] shall be less than vps_layer_id[ n ]. When not present, vps_layer_id[0] is inferred to be equal to the value of nuh_layer_id of the first VCL NAL unit.

’’’

## Semantics of sps_max_sublayers_minus1

The semantics of vps_max_sublayers_minus1 is defined as follows:

‘‘‘

sps_max_sublayers_minus1 plus 1 specifies the maximum number of temporal sublayers that may be present in each CLVS referring to the SPS. The value of sps_max_sublayers_minus1 shall be in the range of 0 to vps_max_sublayers_minus1, inclusive.

’’’

However, vps_max_sublayers_minus1 might not be defined since VPS might not be present in a bitstream and no inferred value is specified in the semantics for vps_max_sublayers_minus1. Meanwhile, except the reference of constraining the range of sps_max_sublayers_minus1, vps_max_sublayers_minus1 referenced in other places all apply to the VPS syntax elements.

Therefore, the following semantics change for sps_max_sublayers_minus1 is proposed:

‘‘‘

sps_max_sublayers_minus1 plus 1 specifies the maximum number of temporal sublayers that may be present in each CLVS referring to the SPS. When VPS is present, the value of sps_max_sublayers_minus1 shall be in the range of 0 to vps_max_sublayers_minus1, inclusive. When VPS is not present, the value of sps_max_sublayers_minus1 shall be in the range of 0 to 6, inclusive.

’’’

# Conclusions

This contribution proposes two bug fixes for syntax elements in VPS and SPS. Without them, the definition in current working draft is broken. The source code modification based on the latest commit (7257fba5) is attached for the proposed aspects. It is suggested to adopt the proposed bug fixes to the next version of VVC specification.

# Patent rights declaration(s)

Huawei Technologies Co., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- “Versatile Video Coding (Draft 8)”, JVET-Q2001-vE, B. Bross, J. Chen, S. Liu, Y.-K. Wang