| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0159-v1 |
| --- | --- |

| Title: | AHG9: On high level syntax of deblocking filter |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jie Chen, Jiancong Luo, Yan Ye, Ru-Ling Liao Radiance Jin Hui Tower, Building No.6, The 4 District Wangjing Dong Yuan, Chaoyang District, Beijing, China | Tel: Email: | jiechen.cj@alibaba-inc.com |
| Source: | Alibaba group |  |  |

_____________________________

- Abstract

This contribution includes two syntax and semantics changes of deblocking filter (DBF) as follows.

- It is proposed to signal an SPS enabled flag for DBF as other loop filters

- Fix the bugs of the semantics of DBF control related syntax elements.

Proposal 1: SPS enabled flag for DBF

It is noted that in VVC draft 8 [1] all the loop filters including ALF, SAO and LMCS have an SPS enabled flag except deblocking filter (DBF).

Thus, it is proposed to add a SPS enabled flag for DBF. The spec. changes are given below

Table 1 SPS syntax change of proposal 1

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| sps_deblocking_filter_disabled_flag | u(1) |
| sps_sao_enabled_flag | u(1) |
| sps_alf_enabled_flag | u(1) |

sps_deblocking_filter_disabled_flag equal to 1 specifies the deblocking filter is not applied for the slices referring to the SPS. sps_deblocking_filter_disabled_flag equal to 0 specifies the deblocking filter may be applied for the slices referring to the SPS

deblocking_filter_control_present_flag equal to 1 specifies the presence of deblocking filter control syntax elements in the PPS. deblocking_filter_control_present_flag equal to 0 specifies the absence of deblocking filter control syntax elements in the PPS. When the value of sps_deblocking_filter_disabled_flag equal to 1, the value of deblocking_filter_control_present_flag shall be 0.

Proposal 2: DBF semantics fix

Table 2: DBF syntax in PPS

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| deblocking_filter_control_present_flag | u(1) |
| if( deblocking_filter_control_present_flag ) { |  |
| deblocking_filter_override_enabled_flag | u(1) |
| pps_deblocking_filter_disabled_flag | u(1) |
| if( !pps_deblocking_filter_disabled_flag ) { |  |
| pps_beta_offset_div2 | se(v) |
| pps_tc_offset_div2 | se(v) |
| pps_cb_beta_offset_div2 | se(v) |
| pps_cb_tc_offset_div2 | se(v) |
| pps_cr_beta_offset_div2 | se(v) |
| pps_cr_tc_offset_div2 | se(v) |
| } |  |
| } |  |
| … |  |
| if( deblocking_filter_override_enabled_flag ) |  |
| dbf_info_in_ph_flag | u(1) |

Table 3:DBF syntax in the picture header

| picture_header_structure( ) { | Descriptor |
| --- | --- |
| … |  |
| if( deblocking_filter_override_enabled_flag && dbf_info_in_ph_flag ) { |  |
| ph_deblocking_filter_override_flag | u(1) |
| if( ph_deblocking_filter_override_flag ) { |  |
| ph_deblocking_filter_disabled_flag | u(1) |
| if( !ph_deblocking_filter_disabled_flag ) { |  |
| ph_beta_offset_div2 | se(v) |
| ph_tc_offset_div2 | se(v) |
| ph_cb_beta_offset_div2 | se(v) |
| ph_cb_tc_offset_div2 | se(v) |
| ph_cr_beta_offset_div2 | se(v) |
| ph_cr_tc_offset_div2 | se(v) |
| } |  |
| } |  |
| } |  |

Table 4: DBF syntax in the slice header

| slice_header( ) { | Descriptor |
| --- | --- |
| … |  |
| if( deblocking_filter_override_enabled_flag && !dbf_info_in_ph_flag ) |  |
| slice_deblocking_filter_override_flag | u(1) |
| if( slice_deblocking_filter_override_flag ) { |  |
| slice_deblocking_filter_disabled_flag | u(1) |
| if( !slice_deblocking_filter_disabled_flag ) { |  |
| slice_beta_offset_div2 | se(v) |
| slice_tc_offset_div2 | se(v) |
| slice_cb_beta_offset_div2 | se(v) |
| slice_cb_tc_offset_div2 | se(v) |
| slice_cr_beta_offset_div2 | se(v) |
| slice_cr_tc_offset_div2 | se(v) |
| } |  |
| } |  |

First, In VVC draft 8 [1], as shown in Table 2, pps_deblocking_filter_disabled_flag is signaled in PPS to indicate whether the operation of deblocking filter is applied for slices referring to the PPS. However, when deblocking_filter_override_enabled_flag is equal to 1, it can be overridden by ph_deblocking_filter_disabled_flag signaled in PH as shown in Table 3 or slice_deblocking_filter_disabled_flag in SH as shown in Table 4.

Thus, the current semantics of pps_deblocking_filter_disabled_flag is not accurate. It is proposed to change the semantics as follows.

pps_deblocking_filter_disabled_flag equal to 1 specifies that the operation of deblocking filter is not applied for slices referring to the PPS of in which slice_deblocking_filter_disabled_flag and ph_deblockig_filter_disabled_flag are is not present. pps_deblocking_filter_disabled_flag equal to 0 specifies that the operation of the deblocking filter is applied for slices referring to the PPS of in which slice_deblocking_filter_disabled_flag and ph_deblockig_filter_disabled_flag are is not present. When not present, the value of pps_deblocking_filter_disabled_flag is inferred to be equal to 0 sps_deblocking_filter_disabled_flag.

Or

pps_deblocking_filter_disabled_flag equal to 1 specifies that the operation of deblocking filter is not applied for slices referring to the PPS when deblocking_filter_override_enabled_flag is equal to 0 in which slice_deblocking_filter_disabled_flag is not present. pps_deblocking_filter_disabled_flag equal to 0 specifies that the operation of the deblocking filter is applied for slices referring to the PPS when deblocking_filter_override_enabled_flag is equal to 0 in which slice_deblocking_filter_disabled_flag is not present. When not present, the value of pps_deblocking_filter_disabled_flag is inferred to be equal to 0.

Second, as shown in Table 3, ph_deblocking_filter_disabled_flag is signaled PH to indicate whether the operation of the deblocking filter is applied for the slices associated with the PH. When ph_deblocking_filter_disabled_flag is not present, it is inferred to be equal to pps_deblocking_filter_disabled_flag. However, in the case that pps_deblocking_filter_disabled_flag is overridden by slice_deblocking_filter_disabled_flag (in this case, ph_deblocking_filter_disabled_flag is not signaled), the value of pps_deblocking_filter_disabled_flag may not by applicable to the slices referring to PPS. So the inferred value of ph_deblocking_filter_disabled_flag is also not applicable to the slices referring to the PH.

Based on this, it is proposed to change the semantics of ph_deblocking_filter_disabled_flag as follows to make it more accurate.

ph_deblocking_filter_disabled_flag equal to 1 specifies that the operation of the deblocking filter is not applied for the slices associated with the PH in which slice_deblocking_filter_disabled_flag is not present. ph_deblocking_filter_disabled_flag equal to 0 specifies that the operation of the deblocking filter is applied for the slices associated with the PH in which slice_deblocking_filter_disabled_flag is not present. When ph_deblocking_filter_disabled_flag is not present, it is inferred to be equal to pps_deblocking_filter_disabled_flag.

Conclusion

First, a SPS enabled flag is proposed for deblocking filter in this contribution to provide the encoder the flexibility to disable the DBF. Second, the semantics of deblocking filter high level control related syntax element in VVC draft 8 is not accurate. The fix of these semantics is provided in this contribution. It is suggested to adopt this contribution.

Reference

B. Bross, J. Chen, S. Liu, Y.-K. Wang, “Versatile Video Coding (Draft 8)”, JVET-Q2001, October 2019.

Patent rights declaration(s)

Alibaba Group may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).