| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0164_v2 |
| --- | --- |

| Title: | AHG10: Mean-scaled SATD for VTM encoder |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jani Lainema Antti Hallapuro | Email: | first.lastname@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

This contribution reports that a coding efficiency improvement is achieved by scaling down the DC coefficients in the SATD (Sum of Absolute Transformed Differences) calculations performed by the VTM encoder. It is proposed to apply a weight of one quarter for the absolute value of the DC coefficient resulting from the Hadamard transform of the SATD process, while keep applying a weight of one for the rest of the coefficients.

Coding efficiency impact is reportedly on average -0.12 %, -0.29 % and -0.37 % for AI, RA and LD-B configurations, respectively. It is further reported that the gains in coding efficiency tend to be larger for the higher resolution categories. For example, in the case of Class A1, the impact in RA configuration is reported as -0.57 %.

[v2 added LD-P results and a new section on output image analysis]

# Introduction

VTM encoder uses SATD (Sum of Absolute Transformed Differences) as a cost measure in certain mode and parameter decisions. Such decisions include, for example, fractional pel motion estimation and selecting a subset of intra prediction modes for the full RDO stage.

The SATD cost function of VTM provides better coding efficiency than e.g. SAD (Sum of Absolute Differences) based cost functions, as SATD can model the frequency content of a prediction error block and provides a penalty for the cost when a block has content that is challenging for transform coding. However, SATD model assumes that each transform coefficient contributes equally to the cost of a block which is not necessarily optimal considering lower frequency residual, and especially the DC coefficient, can be coded more efficiently than the higher frequency components of the residual.

# Proposal

It is proposed to modify the SATD calculations in the VTM encoder by applying a weight of one quarter for the absolute value of the DC coefficient resulting from the SATD transform process. This is asserted to compensate for the relative easiness of coding the DC components with respect to the AC coefficients.

The weight of one quarter is selected empirically as it provides on average larger gains than other weights that can be implemented with simple bit shifting. Modifying the weights of lower frequency AC coefficients was also studied but extending scaling operations for those appeared to have negligible impact on coding efficiency.

Implementation of the proposed change is provided as an attachment to this contribution.

# Coding efficiency results

Simulation results following the JVET CTC can be found below.

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | VTM-8.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.19 % | -0.13 % | -0.15 % | 99 % | 99 % |
| Class A2 | -0.11 % | -0.09 % | -0.06 % | 99 % | 99 % |
| Class B | -0.10 % | -0.02 % | -0.03 % | 98 % | 101 % |
| Class C | -0.08 % | -0.10 % | -0.07 % | 99 % | 101 % |
| Class E | -0.13 % | -0.09 % | -0.08 % | 99 % | 98 % |
| Overall | -0.12 % | -0.08 % | -0.07 % | 99 % | 100 % |
| Class D | -0.09 % | -0.15 % | 0.04 % | 100 % | 100 % |
| Class F | -0.08 % | -0.04 % | -0.07 % | 99 % | 98 % |
|  | Random access |  |  |  |  |
|  |  |  | VTM-8.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.57 % | -0.07 % | -0.16 % | 100 % | 99 % |
| Class A2 | -0.26 % | -0.23 % | 0.01 % | 100 % | 100 % |
| Class B | -0.25 % | -0.09 % | -0.22 % | 101 % | 100 % |
| Class C | -0.14 % | -0.03 % | -0.10 % | 100 % | 100 % |
| Class E |  |  |  |  |  |
| Overall | -0.29 % | -0.10 % | -0.13 % | 100 % | 100 % |
| Class D | -0.05 % | 0.18 % | -0.07 % | 99 % | 99 % |
| Class F | -0.10 % | -0.07 % | -0.05 % | 101 % | 99 % |
|  | Low delay B |  |  |  |  |
|  |  |  | VTM-8.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.34 % | -0.04 % | -0.46 % | 102 % | 100 % |
| Class C | -0.50 % | -0.44 % | -0.43 % | 99 % | 100 % |
| Class E | -0.24 % | 0.27 % | -0.43 % | 98 % | 99 % |
| Overall | -0.37 % | -0.09 % | -0.44 % | 100 % | 99 % |
| Class D | -0.09 % | -0.36 % | -0.12 % | 99 % | 99 % |
| Class F | -0.20 % | 0.05 % | -0.01 % | 99 % | 99 % |

|  | Low delay P |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | VTM-8.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.29 % | -0.43 % | -0.24 % | 100 % | 100 % |
| Class C | -0.49 % | -0.32 % | -0.22 % | 100 % | 100 % |
| Class E | -0.21 % | -0.70 % | -0.70 % | 100 % | 100 % |
| Overall | -0.34 % | -0.46 % | -0.35 % | 100 % | 100 % |
| Class D | -0.12 % | 0.03 % | -0.02 % | 100 % | 101 % |
| Class F | -0.36 % | -0.29 % | -1.11 % | 101 % | 100 % |

# Output image frequency analysis

In order to analyse the impact of the proposed change on output images, a simple spatial frequency analysis was performed using the CTC RA configuration. In this analysis all output luma images were split into 4x4 blocks, a forward 4x4 DCT was performed on those and an average DCT block was calculated. Same analysis was performed for both VTM-8 anchor and for the proposed approach. DCT-2 implementation of the VTM software was used as the forward transform. The tables below illustrate the resulting average 4x4 transform coefficients for the two cases tested.

Table 2: Frequency analysis of the output pictures comparing VTM-8 and the proposal

| VTM-8 |  |  |  |  | Proposed |  |  |  |  | Difference |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 13619.17 | 370.63 | 176.09 | 91.71 |  | 13619.14 | 370.68 | 176.10 | 91.74 |  | 0.00 % | 0.01 % | 0.01 % | 0.03 % |
| 367.64 | 162.63 | 96.67 | 50.67 |  | 367.67 | 162.64 | 96.66 | 50.67 |  | 0.01 % | 0.01 % | 0.00 % | 0.00 % |
| 168.59 | 93.31 | 61.95 | 36.10 |  | 168.60 | 93.31 | 61.95 | 36.11 |  | 0.01 % | 0.00 % | 0.00 % | 0.03 % |
| 91.63 | 51.07 | 37.00 | 23.74 |  | 91.66 | 51.07 | 37.00 | 23.76 |  | 0.04 % | 0.02 % | 0.02 % | 0.09 % |

The test indicates that compared to the anchor, the propose approach is producing somewhat higher frequency output images. This may indicate potential improvements in visual quality.

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).