| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0168 |
| --- | --- |

| Title: | Issue on bS derivation of deblocking filter |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kiyofumi Abe Tadamasa Toma 3-1-1 Yagumo-naka-machi, Moriguchi, Osaka, Japan | Tel: Email: | +81-80-3083-1171 abe.kiyo@jp.panasonic.com toma.tadamasa@jp.panasonic.com |
| Source: | Panasonic |  |  |

_____________________________

- Abstract

This contribution points out the mismatch between VVC text and VTM on bS derivation process of deblocking filter. In VVC text, bS is set equal to 1 for the boundary between IBC and inter block on both luma edge and chroma edge. On the other hand, in VTM, bS is set equal to 1 for the boundary between IBC and inter block only on luma edge. The proponent of this contribution thinks there are two solutions, solution1: fix VTM to align to VVC text, solution2: fix VVC text to align to VTM. This contribution shows the difference of coding performance and text changes for both solutions.

Introduction

The following is the specification of bS derivation of deblocking filter described in VVC draft of JVET-Q2001-vE[1]. According to the description enclosed in a red square, bS is set equal to 1 for the boundary between IBC and inter block. This condition is applied to both luma edge and chroma edge. On the other hand, in the implementation of VTM, this condition is applied only to luma edge. The proponent of this contribution believes that both methods can be possible, and it needs to discuss which is appropriate by comparing the effect on the specification and the performance.

8.8.3.5	Derivation process of boundary filtering strength

….

- The variable bS[ xDi ][ yDj ] is derived as follows:

    - If cIdx is equal to 0 and both samples p0 and q0 are in a coding block with intra_bdpcm_luma_flag equal to 1, bS[ xDi ][ yDj ] is set equal to 0.

    - Otherwise, if cIdx is greater than 0 and both samples p0 and q0 are in a coding block with intra_bdpcm_chroma_flag equal to 1, bS[ xDi ][ yDj ] is set equal to 0.

    - Otherwise, if the sample p0 or q0 is in the coding block of a coding unit coded with intra prediction mode, bS[ xDi ][ yDj ] is set equal to 2.

    - Otherwise, if the block edge is also a coding block edge and the sample p0 or q0 is in a coding block with ciip_flag equal to 1, bS[ xDi ][ yDj ] is set equal to 2.

    - Otherwise, if the block edge is also a transform block edge and the sample p0 or q0 is in a transform block which contains one or more non-zero transform coefficient levels, bS[ xDi ][ yDj ] is set equal to 1.

    - Otherwise, if the prediction mode of the coding subblock containing the sample p0 is different from the prediction mode of the coding subblock containing the sample q0 (i.e. one of the coding subblock is coded in IBC prediction mode and the other is coded in inter prediction mode), bS[ xDi ][ yDj ] is set equal to 1.

    - Otherwise, if cIdx is equal to 0, edgeFlags[ xDi ][ yDj ] is equal to 2, and one or more of the following conditions are true, bS[ xDi ][ yDj ] is set equal to 1:

      - The coding subblock containing the sample p0 and the coding subblock containing the sample q0 are both coded in IBC prediction mode, and the absolute difference between the horizontal or vertical component of the block vectors used in the prediction of the two coding subblocks is greater than or equal to 8 in units of 1/16 luma samples.

      - For the prediction of the coding subblock containing the sample p0 different reference pictures or a different number of motion vectors are used than for the prediction of the coding subblock containing the sample q0.

….

Solutions

The proponent of this contribution believes there are two solutions. Figure1 shows the bS derivation comparing solution1 and solutino2.

Figure1: comparison of solution1 and solution2

Solution1:

In this solution, it is proposed to align to VVC text and modify VTM implementation. bS is set equal to 1 for the boundary between IBC and inter block on both luma edge and chroma edge. It means that the strength of IBC boundary is treated same as non-zero coefficient block boundary. This solution has some impact on coding results and subjective evaluation.

Solution2:

In this solution, it is proposed to align to VTM and modify VVC text. bS is set equal to 1 for the boundary between IBC and inter block only on luma edge. It means that the strength of IBC boundary is treated same as inter boundary. This solution has no change in coding result.

Simulation results of solution1

Table1 shows the simulation results of solution1. The simulation is conducted using VTM-8.0 with the test condition described in JVET-N1010[2] adding TGM. The difference of coding results can be seen only classF and TGM on RA and LDB, since the boundary between IBC and inter block occurs only in these conditions. This result shows that the impact of coding performance is very small but there is possibility to have some affect on subjective image quality for some contents.

Table1: Simulation results of solution1

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 101% | 100% |
| Class A2 | 0.00% | 0.00% | 0.00% | 100% | 99% |
| Class B | 0.00% | 0.00% | 0.00% | 101% | 100% |
| Class C | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class E | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Overall | 0.00% | 0.00% | 0.00% | 101% | 100% |
| Class D | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class F | 0.00% | 0.00% | 0.00% | 100% | 100% |
| TGM | 0.00% | 0.00% | 0.00% | 100% | 100% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 100% | 99% |
| Class A2 | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class B | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class C | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class D | 0.00% | 0.00% | 0.00% | 100% | 98% |
| Class F | 0.00% | 0.06% | 0.00% | 100% | 99% |
| TGM | 0.01% | 0.03% | 0.05% | 100% | 100% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.00% | 0.00% | 100% | 99% |
| Class C | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class E | 0.00% | 0.00% | 0.00% | 100% | 99% |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 99% |
| Class D | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class F | -0.04% | 0.60% | 0.23% | 100% | 99% |
| TGM | 0.03% | 0.10% | 0.15% | 100% | 101% |

Specification text changes of solution2

The suggested specification changes of solution2 are described on top of VVC draft of JVET-Q2001-vE. The newly added parts are highlighted in yellow and the text to be removed is marked as red.

8.8.3.5	Derivation process of boundary filtering strength

….

- The variable bS[ xDi ][ yDj ] is derived as follows:

    - If cIdx is equal to 0 and both samples p0 and q0 are in a coding block with intra_bdpcm_luma_flag equal to 1, bS[ xDi ][ yDj ] is set equal to 0.

    - Otherwise, if cIdx is greater than 0 and both samples p0 and q0 are in a coding block with intra_bdpcm_chroma_flag equal to 1, bS[ xDi ][ yDj ] is set equal to 0.

    - Otherwise, if the sample p0 or q0 is in the coding block of a coding unit coded with intra prediction mode, bS[ xDi ][ yDj ] is set equal to 2.

    - Otherwise, if the block edge is also a coding block edge and the sample p0 or q0 is in a coding block with ciip_flag equal to 1, bS[ xDi ][ yDj ] is set equal to 2.

    - Otherwise, if the block edge is also a transform block edge and the sample p0 or q0 is in a transform block which contains one or more non-zero transform coefficient levels, bS[ xDi ][ yDj ] is set equal to 1.

    - Otherwise, if the prediction mode of the coding subblock containing the sample p0 is different from the prediction mode of the coding subblock containing the sample q0 (i.e. one of the coding subblock is coded in IBC prediction mode and the other is coded in inter prediction mode), bS[ xDi ][ yDj ] is set equal to 1.

    - Otherwise, if cIdx is equal to 0, edgeFlags[ xDi ][ yDj ] is equal to 2, and one or more of the following conditions are true, bS[ xDi ][ yDj ] is set equal to 1:

      - The prediction mode of the coding subblock containing the sample p0 is different from the prediction mode of the coding subblock containing the sample q0 (i.e. one of the coding subblock is coded in IBC prediction mode and the other is coded in inter prediction mode).

      - The coding subblock containing the sample p0 and the coding subblock containing the sample q0 are both coded in IBC prediction mode, and the absolute difference between the horizontal or vertical component of the block vectors used in the prediction of the two coding subblocks is greater than or equal to 8 in units of 1/16 luma samples.

      - For the prediction of the coding subblock containing the sample p0 different reference pictures or a different number of motion vectors are used than for the prediction of the coding subblock containing the sample q0.

….

Conclusion

This contribution points out the mismatch between VVC text and VTM on bS derivation process of deblocking filter. In VVC text, bS is set equal to 1 for the boundary between IBC and inter block on both luma edge and chroma edge. On the other hand, in VTM, bS is set equal to 1 for the boundary between IBC and inter block only on luma edge. The proponent of this contribution thinks there are two solutions, solution1: fix VTM to align to VVC text, solution2: fix VVC text to align to VTM. This contribution shows the difference of coding performance and text changes for both solutions. The proponent of this contribution proposes to fix VVC text to align to VTM (solution2), since this solution has no impact on coding result and subjective evaluation, and the concept of the processing makes sense.

References

- B. Bross, J. Chen, S. Liu, Y.-K. Wang, “Versatile Video Coding (Draft 8)”, JVET-Q2001-vE, JVET 17th Meeting: Brussels, BE, 7–17 January 2020.

- F. Bossen, J. Boyce, K. Suehring, X. Li, V. Seregin, “JVET common test conditions and software reference configurations for SDR video”, JVET-N1010, JVET 14th Meeting: Geneva, CH, 19–27 Mar. 2019.

Patent rights declaration(s)

Panasonic Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).