| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0169 |
| --- | --- |

| Title: | AHG14: Report of CABAC skip mode results on VTM-8.0 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Kiyofumi Abe Tadamasa Toma Virginie Drugeon 3-1-1 Yagumo-naka-machi, Moriguchi, Osaka, Japan | Tel: Email: | +81-80-3083-1171 abe.kiyo@jp.panasonic.com toma.tadamasa@jp.panasonic.com virginie.drugeon@eu.panasonic.com |
| Source: | Panasonic |  |  |

_____________________________

- Abstract

This contribution is an information contribution which reports the coding results of CABAC skip mode on VTM-8.0. CABAC skip mode was proposed by JVET-M0089, JVET-N0207, and JVET-O0308. It directly outputs binarized bins as a bitstream without CABAC processing, and can avoid CABAC throughput issue without any additional building blocks. Simulation results reportedly show that the CABAC skip mode can guarantee the fixed processing delay with the cost of 16%, 19%, and 23% bits increasing for AI, RA, and LDB on VTM-8.0, and 8%, 7%, and 10% bits increasing for AI, RA, and LDB on VTM-8.0-lossless.

Introduction

This contribution is an information contribution which reports the coding results of CABAC skip mode on VTM-9.0. CABAC skip mode was proposed by JVET-M0089 [1], JVET-N0207 [2], and JVET-O0308 [3]. It directly outputs binarized bins as a bitstream without CABAC processing, and can avoid CABAC throughput issue without any additional building blocks.

Figure1 shows the overview of CABAC skip mode. The binarization module can be implemented in the pipeline structure without additional buffer, so this mode can guarantee fixed processing delay regardless of bits amount in a block. This mode decreases the coding efficiency, but it can avoid CABAC throughput issue without any additional building blocks.

Figure1: CABAC skip mode.

Figure2 shows the detailed explanation about the modification of binarization process. The residual binarization which is used after exceeding the maximum CCB is always reused for CABAC skip mode. Specifically, threshold value for CCB is always set equal to 0, and select the residual binarization for the condition where CCB exceeds the limitation. This residual binarization tends to be a small number of bins compared to normal residual binarization. Therefore, it can be expected to reduce the bit increasing of CABAC skip mode.

Figure2: Residual binarization of CABAC skip mode.

Simulation results

Table1 shows the simulation results of CABAC skip mode. The simulation is conducted using VTM-8.0 with the test condition described in JVET-N1010 [6]. Table1 also shows the simulation results of CABAC skip mode on VTM-7.0 that is reported by JVET-Q0149 [5] at previous meeting. This result shows that the coding performance is similar to that of VTM-7.0 reported at previous meeting.

Table1: Simulation results of CABAC skip mode

|  | All Intra Main 10 |  |  |  |  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 (JVET-Q0149) |  |  |  |  | Over VTM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 20.35% | 8.73% | 10.27% | 116% | 92% | 20.32% | 9.04% | 10.25% | 118% | 95% |
| Class A2 | 17.71% | 11.23% | 12.86% | 97% | 91% | 17.72% | 11.45% | 12.42% | 99% | 94% |
| Class B | 15.09% | 12.18% | 10.31% | 98% | 92% | 15.09% | 11.52% | 10.40% | 100% | 96% |
| Class C | 12.62% | 12.13% | 11.22% | 88% | 94% | 12.65% | 12.04% | 11.03% | 88% | 98% |
| Class E | 13.27% | 11.30% | 10.44% | 96% | 95% | 13.31% | 10.49% | 10.10% | 97% | 97% |
| Overall | 15.55% | 11.29% | 10.95% | 98% | 93% | 15.56% | 11.04% | 10.80% | 99% | 96% |
| Class D | 12.09% | 10.79% | 10.36% | 85% | 93% | 12.09% | 10.81% | 10.43% | 87% | 106% |
| Class F | 19.22% | 17.83% | 17.32% | 98% | 92% | 19.67% | 17.65% | 16.84% | 102% | 95% |
| TGM | 21.27% | 23.83% | 23.73% | 92% | 102% | 21.76% | 22.02% | 21.76% | 95% | 105% |

|  | Random Access Main 10 |  |  |  |  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 (JVET-Q0149) |  |  |  |  | Over VTM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 21.27% | 10.52% | 9.93% | 97% | 96% | 20.42% | 10.26% | 9.37% | 98% | 99% |
| Class A2 | 20.23% | 9.69% | 12.88% | 91% | 94% | 19.33% | 9.06% | 12.18% | 91% | 98% |
| Class B | 19.58% | 8.87% | 6.20% | 94% | 97% | 19.02% | 8.00% | 5.90% | 95% | 99% |
| Class C | 16.48% | 9.30% | 8.02% | 92% | 98% | 15.96% | 8.83% | 7.24% | 93% | 101% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | 19.22% | 9.48% | 8.77% | 93% | 97% | 18.55% | 8.89% | 8.21% | 94% | 99% |
| Class D | 15.63% | 8.03% | 7.43% | 91% | 98% | 15.30% | 8.05% | 6.99% | 91% | 108% |
| Class F | 21.45% | 18.81% | 18.28% | 98% | 100% | 21.29% | 17.47% | 16.78% | 98% | 110% |
| TGM | 27.54% | 27.70% | 28.06% | 95% | 109% | 27.19% | 25.99% | 26.03% | 96% | 122% |

|  | Low delay B Main10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 (JVET-Q0149) |  |  |  |  | Over VTM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | 25.39% | 5.90% | 1.01% | 90% | 100% | 24.64% | 5.32% | 1.95% | 90% | 102% |
| Class C | 21.88% | 8.76% | 6.03% | 89% | 102% | 21.13% | 8.01% | 5.17% | 89% | 103% |
| Class E | 23.18% | 9.33% | 6.34% | 99% | 110% | 20.90% | 7.82% | 5.13% | 98% | 118% |
| Overall | 23.67% | 7.71% | 4.02% | 92% | 103% | 22.54% | 6.84% | 3.82% | 91% | 106% |
| Class D | 21.10% | 4.14% | 2.37% | 90% | 100% | 20.74% | 3.20% | 2.54% | 88% | 100% |
| Class F | 28.29% | 19.45% | 19.25% | 99% | 112% | 27.30% | 18.57% | 17.07% | 98% | 126% |
| TGM | 32.68% | 31.05% | 31.11% | 98% | 119% | 31.72% | 28.76% | 28.66% | 97% | 137% |

Table2 shows the simulation results of CABAC skip mode on the loss less condition. The simulation is conducted using VTM-8.0 with the lossless test condition described in JVET-Q2014 [7]. This result shows that the coding performance loss of CABAC skip mode is not so significant when considering the use cases.

Table2: Simulation results of CABAC skip mode on the loss less setting

|  | All Intra |  |  | Random Access |  |  | Low delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | VTM-8.0-lossless | Proposal |  | VTM-8.0-lossless | Proposal |  | VTM-8.0-lossless | Proposal |  |
| Class A1 | 2.4 | 2.3 | 6.39% | 2.4 | 2.3 | 6.02% |  |  |  |
| Class A2 | 1.8 | 1.7 | 4.17% | 1.9 | 1.8 | 3.22% |  |  |  |
| Class B | 2.3 | 2.2 | 6.94% | 2.5 | 2.3 | 6.83% | 2.4 | 2.3 | 6.97% |
| Class C | 2.1 | 1.9 | 7.89% | 2.6 | 2.4 | 9.10% | 2.6 | 2.4 | 9.16% |
| Class D | 2.1 | 1.9 | 7.57% | 2.9 | 2.6 | 13.77% | 2.9 | 2.5 | 13.65% |
| Class E | 3.1 | 2.7 | 13.26% |  |  |  | 3.4 | 3.0 | 15.08% |
| Class F | 5.8 | 4.8 | 18.05% | 35.4 | 25.4 | 31.93% | 51.8 | 34.1 | 35.49% |
| TGM | 12.4 | 9.7 | 26.86% | 109.0 | 89.6 | 23.75% | 122.8 | 102.5 | 22.38% |
| Overall | 2.3 | 2.1 | 7.65% | 2.4 | 2.2 | 6.55% | 2.7 | 2.5 | 9.72% |
| EncT | 90% |  |  | 99% |  |  | 99% |  |  |
| DecT | 58% |  |  | 72% |  |  | 75% |  |  |

Conclusion

This contribution is an information contribution which reports the coding results of CABAC skip mode on VTM-8.0. CABAC skip mode was proposed by JVET-M0089, JVET-N0207, and JVET-O0308. It directly outputs binarized bins as a bitstream without CABAC processing, and can avoid CABAC throughput issue without any additional building blocks. Simulation results reportedly show that the CABAC skip mode can guarantee the fixed processing delay with the cost of 16%, 19%, and 23% bits increasing for AI, RA, and LDB on VTM-8.0, and 8%, 7%, and 10% bits increasing for AI, RA, and LDB on VTM-8.0-lossless.

References

[1] K. Abe, T. Toma, “Non-CE5: CABAC skip mode for super low delay”, JVET-M0089, JVET 13th Meeting: Marrakech, MA, 9–18 Jan. 2019.

[2] K. Abe, T. Toma, “AHG14: CABAC skip mode”, JVET-N0207, JVET 14th Meeting: Geneva, CH, 19–27 March 2019.

[3] K. Abe, T. Toma, D.-Y. Kim, “CABAC skip mode”, JVET-O0308, JVET 15th Meeting: Gothenburg, SE, 3–12 July 2019.

[4] K. Abe, T. Toma, “Report of CABAC skip mode results on VTM-6.0”, JVET-P0939, JVET 16th Meeting: Geneva, CH, 1–11 October 2019.

[5] K. Abe, T. Toma, V. Drugeon, “Report of CABAC skip mode results on VTM-7.0”, JVET-Q0149, JVET 17th Meeting: Brussels, BE, 7–17 January 2020.

[6]	F. Bossen, J. Boyce, K. Suehring, X. Li, V. Seregin, “JVET common test conditions and software reference configurations for SDR video”, JVET-N1010, JVET 14th Meeting: Geneva, CH, 19–27 Mar. 2019.

[7]	T. Nguyen, T. C. Ma, A. Nalci, “JVET common test conditions and software reference configurations for lossless, near lossless, and mixed lossy/lossless coding”, JVET-Q2014, JVET 17th Meeting: Brussels, BE, 7–17 January 2020.

Patent rights declaration(s)

Panasonic Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).