| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0172-r1 |
| --- | --- |

| Title: | AhG 9: Removed Redundant Coding of Chroma Deblocking Filter Parameters |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Karam Naser Fabrice Le Léannec Tangi Poirier Franck Galpin | Tel: Email: | karam.naser@interdigital.com |
| Source: | InterDigital |  |  |

_____________________________

# Abstract

In JVET-Q2001, the chroma related HLS elements are coded depending on the variable ChromaArrayType. When it is zero, all chroma tools are deactivated and chroma syntax shall not be coded. However, the chroma deblocking filter parameters are coded at PPS, PH and SH regardless of ChromaArrayType. This leads to redundant coding as well as inconsistency with the rest of the design. This contribution proposes to remove this redundant coding by conditioning the presence of these parameters on ChromaArrayType.

# Introduction

The deblocking parameters are coded in Picture Parameters Set (PPS), slice header (SH) or picture header (PH). This is controlled by a PPS flag named deblocking_filter_override_enabled_flag. If this flag equals to one, the parameters are coded in either in PH or SH, otherwise they are coded only in the PPS. The corresponding specification text is shown below:

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| deblocking_filter_control_present_flag | u(1) |
| if( deblocking_filter_control_present_flag ) { |  |
| deblocking_filter_override_enabled_flag | u(1) |
| pps_deblocking_filter_disabled_flag | u(1) |
| if( !pps_deblocking_filter_disabled_flag ) { |  |
| pps_beta_offset_div2 | se(v) |
| pps_tc_offset_div2 | se(v) |
| pps_cb_beta_offset_div2 | se(v) |
| pps_cb_tc_offset_div2 | se(v) |
| pps_cr_beta_offset_div2 | se(v) |
| pps_cr_tc_offset_div2 | se(v) |
| } |  |
| } |  |
| … |  |
| if( deblocking_filter_override_enabled_flag ) |  |
| dbf_info_in_ph_flag | u(1) |

In the picture header, the parameters are coded in the following manner:

| picture_header_structure( ) { | Descriptor |
| --- | --- |
| … |  |
| if( deblocking_filter_override_enabled_flag && dbf_info_in_ph_flag ) { |  |
| ph_deblocking_filter_override_flag | u(1) |
| if( ph_deblocking_filter_override_flag ) { |  |
| ph_deblocking_filter_disabled_flag | u(1) |
| if( !ph_deblocking_filter_disabled_flag ) { |  |
| ph_beta_offset_div2 | se(v) |
| ph_tc_offset_div2 | se(v) |
| ph_cb_beta_offset_div2 | se(v) |
| ph_cb_tc_offset_div2 | se(v) |
| ph_cr_beta_offset_div2 | se(v) |
| ph_cr_tc_offset_div2 | se(v) |
| } |  |
| } |  |
| } |  |
| … |  |

And in the slice header, the parameters are coded in the following manner:

| slice_header( ) { | Descriptor |
| --- | --- |
| … |  |
| if( deblocking_filter_override_enabled_flag && !dbf_info_in_ph_flag ) |  |
| slice_deblocking_filter_override_flag | u(1) |
| if( slice_deblocking_filter_override_flag ) { |  |
| slice_deblocking_filter_disabled_flag | u(1) |
| if( !slice_deblocking_filter_disabled_flag ) { |  |
| slice_beta_offset_div2 | se(v) |
| slice_tc_offset_div2 | se(v) |
| slice_cb_beta_offset_div2 | se(v) |
| slice_cb_tc_offset_div2 | se(v) |
| slice_cr_beta_offset_div2 | se(v) |
| slice_cr_tc_offset_div2 | se(v) |
| } |  |
| } |  |
| … |  |

In all of the three levels (PPS, PH and HS), the deblocking parameters that are coded are:

- beta_offset_div2

- tc_offset_div2

- cb_beta_offset_div2

- cb_tc_offset_div2

- cr_beta_offset_div2

- cr_tc_offset_div2

They represent the beta and tc offsets used for deblocking. They are coded for luma and chroma (cb and cr) components. Unlike many other syntax elements, the chroma parameters are coded regardless of the chroma format.

Compared to other syntax elements, deblocking filter parameters related to chroma are coded regardless of the value of ChromaArrayType. On one hand, this is useless signaling that costs some bits that are never used. On the other hand, it makes it more complicated for the encoder designer to set irrelevant values. Therefore, this contribution proposes to remove this useless coding.

# Proposal 1

To solve the useless coding problem, the coding of chroma deblocking parameters is conditioned on the chroma format. That is, if ChromaArrayType is zero, the parameters shall not be coded but inferred to be zero. This makes a consistency with the design of VVC, where all redundant flags are not coded but inferred.

The corresponding specification change is given below (added part in blue):

PPS level:

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| deblocking_filter_control_present_flag | u(1) |
| if( deblocking_filter_control_present_flag ) { |  |
| deblocking_filter_override_enabled_flag | u(1) |
| pps_deblocking_filter_disabled_flag | u(1) |
| if( !pps_deblocking_filter_disabled_flag ) { |  |
| pps_beta_offset_div2 | se(v) |
| pps_tc_offset_div2 | se(v) |
| if (ChromaArrayType != 0){ |  |
| pps_cb_beta_offset_div2 | se(v) |
| pps_cb_tc_offset_div2 | se(v) |
| pps_cr_beta_offset_div2 | se(v) |
| pps_cr_tc_offset_div2 | se(v) |
| } |  |
| } |  |
| } |  |
| … |  |
| if( deblocking_filter_override_enabled_flag ) |  |
| dbf_info_in_ph_flag | u(1) |

PH level:

| picture_header_structure( ) { | Descriptor |
| --- | --- |
| … |  |
| if( deblocking_filter_override_enabled_flag && dbf_info_in_ph_flag ) { |  |
| ph_deblocking_filter_override_flag | u(1) |
| if( ph_deblocking_filter_override_flag ) { |  |
| ph_deblocking_filter_disabled_flag | u(1) |
| if( !ph_deblocking_filter_disabled_flag ) { |  |
| ph_beta_offset_div2 | se(v) |
| ph_tc_offset_div2 | se(v) |
| if (ChromaArrayType != 0){ |  |
| ph_cb_beta_offset_div2 | se(v) |
| ph_cb_tc_offset_div2 | se(v) |
| ph_cr_beta_offset_div2 | se(v) |
| ph_cr_tc_offset_div2 | se(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| … |  |

SH level:

| slice_header( ) { | Descriptor |
| --- | --- |
| … |  |
| if( deblocking_filter_override_enabled_flag && !dbf_info_in_ph_flag ) |  |
| slice_deblocking_filter_override_flag | u(1) |
| if( slice_deblocking_filter_override_flag ) { |  |
| slice_deblocking_filter_disabled_flag | u(1) |
| if( !slice_deblocking_filter_disabled_flag ) { |  |
| slice_beta_offset_div2 | se(v) |
| slice_tc_offset_div2 | se(v) |
| if (ChromaArrayType != 0){ |  |
| slice_cb_beta_offset_div2 | se(v) |
| slice_cb_tc_offset_div2 | se(v) |
| slice_cr_beta_offset_div2 | se(v) |
| slice_cr_tc_offset_div2 | se(v) |
| } |  |
| } |  |
| } |  |
| … |  |

# Proposal 2

For PPS, a different method is proposed here to remove the interaction between SPS and PPS. Specifically, ChromaArrayType depends on the SPS syntax elements chroma_format_idc and separate_colour_plane_flag. Therefore, it might be preferable to not use this value inside the PPS. For this reason, the PPS flag pps_chroma_tool_offsets_present_flag is used. This flag is meant for enabling/disabling all chroma QP offset.

In this proposal, the same flag is used to check if chroma components are available. The following modifications are proposed:

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| deblocking_filter_control_present_flag | u(1) |
| if( deblocking_filter_control_present_flag ) { |  |
| deblocking_filter_override_enabled_flag | u(1) |
| pps_deblocking_filter_disabled_flag | u(1) |
| if( !pps_deblocking_filter_disabled_flag ) { |  |
| pps_beta_offset_div2 | se(v) |
| pps_tc_offset_div2 | se(v) |
| if (pps_chroma_tool_offsets_present_flag { |  |
| pps_cb_beta_offset_div2 | se(v) |
| pps_cb_tc_offset_div2 | se(v) |
| pps_cr_beta_offset_div2 | se(v) |
| pps_cr_tc_offset_div2 | se(v) |
| } |  |
| } |  |
| } |  |
| … |  |
| if( deblocking_filter_override_enabled_flag ) |  |
| dbf_info_in_ph_flag | u(1) |

Editorially, the name of the flag can be changed to pps_chroma_tool_offsets_dbf_present_flag. And its semantic is changed to:

pps_chroma_tool_offsets_dbf_present_flag equal to 1 specifies that chroma tool offsets related syntax elements and choma deblocking  filer parameters are present in the PPS RBSP syntax structure. pps_chroma_tool_offsets_dbp_present_flag equal to 0 specifies that chroma tool offsets related syntax elements and chroma deblocking filter parameters are not present in in the PPS RBSP syntax structure. When ChromaArrayType is equal to 0, the value of pps_chroma_tool_offsets_dbf_present_flag shall be equal to 0.

The same modifications for PH and SH are used as in Proposal 1.

# Proposal 3

It consists of inserting a specific flag in the PPS, dedicated to signaling that chroma deblocking filter parameters are signaled in the considered PPS.

This takes the form of the pps_chroma_deblocking_filter_parms_present_flag syntax element.

The proposed modified PPS syntax table is illustrated in the table below.

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| deblocking_filter_control_present_flag | u(1) |
| if( deblocking_filter_control_present_flag ) { |  |
| deblocking_filter_override_enabled_flag | u(1) |
| pps_deblocking_filter_disabled_flag | u(1) |
| if( !pps_deblocking_filter_disabled_flag ) { |  |
| pps_beta_offset_div2 | se(v) |
| pps_tc_offset_div2 | se(v) |
| pps_chroma_deblocking_filter_params_present_flag | u(1) |
| if (pps_chroma_deblocking_filter_params_present_flag) { |  |
| pps_cb_beta_offset_div2 | se(v) |
| pps_cb_tc_offset_div2 | se(v) |
| pps_cr_beta_offset_div2 | se(v) |
| pps_cr_tc_offset_div2 | se(v) |
| } |  |
| } |  |
| } |  |
| … |  |
| if( deblocking_filter_override_enabled_flag ) |  |
| dbf_info_in_ph_flag | u(1) |

The semantics of the new introduced PPS level flag is as follows.

pps_chroma_deblocking_filter_params_present_flag equal to 1 specifies that chroma decoded picture parameters are present in the PPS RBSP syntax structure. pps_chroma_deblocking_filter_params_present_flag equal to 0 specifies that chroma deblocking filter parameters are not present in in the PPS RBSP syntax structure. When ChromaArrayType is equal to 0, the value of pps_chroma_deblocking_filter_params_present_flag shall be equal to 0.

# The same modifications for PH and SH are used as in Proposal 1.

# Proposal 4

In this proposal, the chroma deblocking filter parameters shall not be present in the PPS. Instead, a new PPS flag “pps_chroma_available_flag” is coded to indicate if there is chroma available. If it is true, then the flag “pps_chroma_tool_offsets_present_flag” is coded to indicate if chroma offset is available and the deblocking parameters of chroma are signaled. The following modifications are made:

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| pps_chroma_available_flag | u(1) |
| if( pps_chroma_available_flag ) { |  |
| pps_chroma_tool_offsets_present_flag | u(1) |
| if( pps_chroma_tool_offsets_present_flag ) { |  |
| pps_cb_qp_offset | se(v) |
| pps_cr_qp_offset | se(v) |
| pps_joint_cbcr_qp_offset_present_flag | u(1) |
| if( pps_joint_cbcr_qp_offset_present_flag ) |  |
| pps_joint_cbcr_qp_offset_value | se(v) |
| pps_slice_chroma_qp_offsets_present_flag | u(1) |
| pps_cu_chroma_qp_offset_list_enabled_flag | u(1) |
| } |  |
| } |  |
| … |  |
| deblocking_filter_control_present_flag | u(1) |
| if( deblocking_filter_control_present_flag ) { |  |
| deblocking_filter_override_enabled_flag | u(1) |
| pps_deblocking_filter_disabled_flag | u(1) |
| if( !pps_deblocking_filter_disabled_flag ) { |  |
| pps_beta_offset_div2 | se(v) |
| pps_tc_offset_div2 | se(v) |
| if (pps_chroma_available_flag) { |  |
| pps_cb_beta_offset_div2 | se(v) |
| pps_cb_tc_offset_div2 | se(v) |
| pps_cr_beta_offset_div2 | se(v) |
| pps_cr_tc_offset_div2 | se(v) |
| } |  |
| } |  |
| } |  |
| … |  |
| if( deblocking_filter_override_enabled_flag ) |  |
| dbf_info_in_ph_flag | u(1) |

The same modifications for PH and SH are used as in Proposal 1.

# Proposal 5

In this proposal, two flags are added: pps_chroma_available_flag and pps_chroma_deblocking_filter_params_present_flag. Like the previous proposals, the first is to indicate if chroma is available, and the second is about the presence of chroma deblocking filter. The following text modification are proposed:

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| pps_chroma_available_flag | u(1) |
| if( pps_chroma_available_flag ) { |  |
| pps_chroma_tool_offsets_present_flag | u(1) |
| if( pps_chroma_tool_offsets_present_flag ) { |  |
| pps_cb_qp_offset | se(v) |
| pps_cr_qp_offset | se(v) |
| pps_joint_cbcr_qp_offset_present_flag | u(1) |
| if( pps_joint_cbcr_qp_offset_present_flag ) |  |
| pps_joint_cbcr_qp_offset_value | se(v) |
| pps_slice_chroma_qp_offsets_present_flag | u(1) |
| pps_cu_chroma_qp_offset_list_enabled_flag | u(1) |
| } |  |
| } |  |
| … |  |
| deblocking_filter_control_present_flag | u(1) |
| if( deblocking_filter_control_present_flag ) { |  |
| deblocking_filter_override_enabled_flag | u(1) |
| pps_deblocking_filter_disabled_flag | u(1) |
| if( !pps_deblocking_filter_disabled_flag ) { |  |
| pps_beta_offset_div2 | se(v) |
| pps_tc_offset_div2 | se(v) |
| if( pps_chroma_available_flag ) { |  |
| pps_chroma_deblocking_filter_params_present_flag | u(1) |
| if (pps_chroma_deblocking_filter_params_present_flag) { |  |
| pps_cb_beta_offset_div2 | se(v) |
| pps_cb_tc_offset_div2 | se(v) |
| pps_cr_beta_offset_div2 | se(v) |
| pps_cr_tc_offset_div2 | se(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| … |  |
| if( deblocking_filter_override_enabled_flag ) |  |
| dbf_info_in_ph_flag | u(1) |

The same modifications for PH and SH are used as in Proposal 1.

# Patent rights declaration(s)

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).