| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0173 |
| --- | --- |

| Title: | AhG 9: Cleanup of Constraint Flags |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Karam Naser Fabrice Le Léannec Michel Kerdranvat Philippe de Lagrange | Tel: Email: | karam.naser@interdigital.com |
| Source: | InterDigital |  |  |

_____________________________

# Abstract

The draft VVC specification provides several constraint flags to profiles where certain coding tools are activated/deactivated. It is asserted that several redundant constraint flags are signaled when either monochrome coding or intra-only profiles are used. This contribution proposes cleaning up the constraint flags such that redundant coding of constraint flags is removed.

# Introduction

Several constraint flags are defined in VVC. The inter-related ones are:

| general_constraint_info( ) { | Descriptor |
| --- | --- |
| … |  |
| no_ref_wraparound_constraint_flag | u(1) |
| no_temporal_mvp_constraint_flag | u(1) |
| no_sbtmvp_constraint_flag | u(1) |
| no_amvr_constraint_flag | u(1) |
| no_bdof_constraint_flag | u(1) |
| no_dmvr_constraint_flag | u(1) |
| … |  |
| no_sbt_constraint_flag | u(1) |
| no_affine_motion_constraint_flag | u(1) |
| no_bcw_constraint_flag | u(1) |
| … |  |
| no_ciip_constraint_flag | u(1) |
| no_fpel_mmvd_constraint_flag | u(1) |
| no_gpm_constraint_flag | u(1) |
| … |  |

Similarly, the chroma related ones are:

| general_constraint_info( ) { | Descriptor |
| --- | --- |
| … |  |
| no_ccalf_constraint_flag | u(1) |
| no_joint_cbcr_constraint_flag | u(1) |
| … |  |
| no_cclm_constraint_flag | u(1) |
| } |  |

The problem is that when dealing with intra-only profile, 12 redundant flags are coded. For monochrome profile, 3 color-related flags are unnecessarily coded. This contribution proposes removing this redundancy by conditionally coding these flags depending on the current profile.

# Proposal

At the constraint information level, the chroma type and inter coding can be checked to remove redundant coding. This is done in the following way:

| general_constraint_info( ) { | Descriptor |
| --- | --- |
| … |  |
| intra_only_constraint_flag | u(1) |
| … |  |
| max_chroma_format_constraint_idc | u(2) |
| … |  |
| if (max_chroma_format_constraint_idc){ |  |
| no_ccalf_constraint_flag | u(1) |
| no_joint_cbcr_constraint_flag | u(1) |
| no_cclm_constraint_flag | u(1) |
| } |  |
| if (intra_only_constraint_flag !=1){ |  |
| no_ref_wraparound_constraint_flag | u(1) |
| no_temporal_mvp_constraint_flag | u(1) |
| no_sbtmvp_constraint_flag | u(1) |
| no_amvr_constraint_flag | u(1) |
| no_bdof_constraint_flag | u(1) |
| no_dmvr_constraint_flag | u(1) |
| no_sbt_constraint_flag | u(1) |
| no_affine_motion_constraint_flag | u(1) |
| no_bcw_constraint_flag | u(1) |
| no_ciip_constraint_flag | u(1) |
| no_fpel_mmvd_constraint_flag | u(1) |
| no_gpm_constraint_flag | u(1) |
| } |  |
| no_cclm_constraint_flag | u(1) |
| no_mts_constraint_flag | u(1) |
| no_sbt_constraint_flag | u(1) |
| no_affine_motion_constraint_flag | u(1) |
| no_bcw_constraint_flag | u(1) |
| no_ibc_constraint_flag | u(1) |
| no_ciip_constraint_flag | u(1) |
| no_fpel_mmvd_constraint_flag | u(1) |
| no_gpm_constraint_flag | u(1) |
| … |  |

That is, when max_chroma_format_constraint_idc is not zero (chroma format is yuv400), the chroma related constraint flags are not coded. Similarly, when intra_only_constraint_flag is not equal to one (not intra only coding), the inter-related constraint flags are coded.

The semantics of these flags are modified to have inferred values if not coded:

no_ccalf_constraint_flag equal to 1 specifies that sps_ccalf_enabled_flag shall be equal to 0. no_ccalf_constraint_flag equal to 0 does not impose such a constraint. When not present, the value of no_ccalf_constraint_flag is inferred to be 1.

no_joint_cbcr_constraint_flag equal to 1 specifies that sps_joint_cbcr_enabled_flag shall be equal to 0.  no_joint_cbcr_constraint_flag equal to 0 does not impose such a constraint. When not present, the value of no_joint_cbcr_constraint_flag is inferred to be 1.

no_cclm_constraint_flag equal to 1 specifies that sps_cclm_enabled_flag shall be equal to 0. no_cclm_constraint_flag equal to 0 does not impose such a constraint. When not present, the value of no_cclm_constraint_flag is inferred to be 1.

no_ref_wraparound_constraint_flag equal to 1 specifies that sps_ref_wraparound_enabled_flag shall be equal to 0. no_ref_wraparound_constraint_flag equal to 0 does not impose such a constraint. When not present, the value of no_ref_wraparound_constraint_flag is inferred to be 1.

no_temporal_mvp_constraint_flag equal to 1 specifies that sps_temporal_mvp_enabled_flag shall be equal to 0. no_temporal_mvp_constraint_flag equal to 0 does not impose such a constraint. When not present, the value of no_temporal_mvp_constraint_flag is inferred to be 1.

no_sbtmvp_constraint_flag equal to 1 specifies that sps_sbtmvp_enabled_flag shall be equal to 0. no_sbtmvp_constraint_flag equal to 0 does not impose such a constraint. When not present, the value of no_sbtmvp_constraint_flag is inferred to be 1.

no_amvr_constraint_flag equal to 1 specifies that sps_amvr_enabled_flag shall be equal to 0. no_amvr_constraint_flag equal to 0 does not impose such a constraint. When not present, the value of no_amvr_constraint_flag is inferred to be 1.

no_bdof_constraint_flag equal to 1 specifies that sps_bdof_enabled_flag shall be equal to 0. no_bdof_constraint_flag equal to 0 does not impose such a constraint. When not present, the value of no_bdof_constraint_flag is inferred to be 1.

no_dmvr_constraint_flag equal to 1 specifies that sps_dmvr_enabled_flag shall be equal to 0. no_dmvr_constraint_flag equal to 0 does not impose such a constraint. When not present, the value of no_dmvr_constraint_flag is inferred to be 1.

no_sbt_constraint_flag equal to 1 specifies that sps_sbt_enabled_flag shall be equal to 0. no_sbt_constraint_flag equal to 0 does not impose such a constraint. When not present, the value of no_sbt_constraint_flag is inferred to be 1.

no_affine_motion_constraint_flag equal to 1 specifies that sps_affine_enabled_flag  shall be equal to 0. no_affine_motion_constraint_flag equal to 0 does not impose such a constraint. When not present, the value no_affine_motion_constraint_flag of is inferred to be 1.

no_bcw_constraint_flag equal to 1 specifies that sps_bcw_enabled_flag shall be equal to 0. no_bcw_constraint_flag equal to 0 does not impose such a constraint. When not present, the value of no_bcw_constraint_flag is inferred to be 1.

no_ciip_constraint_flag equal to 1 specifies that sps_ciip_enabled_flag shall be equal to 0. no_cipp_constraint_flag equal to 0 does not impose such a constraint. When not present, the value of no_ciip_constraint_flag is inferred to be 1.

no_fpel_mmvd_constraint_flag equal to 1 specifies that sps_fpel_mmvd_enabled_flag shall be equal to 0. no_fpel_mmvd_constraint_flag equal to 0 does not impose such a constraint. When not present, the value of no_fpel_mmvd_constraint_flag is inferred to be 1.

no_gpm_constraint_flag equal to 1 specifies that sps_gpm_enabled_flag shall be equal to 0. no_gpm_constraint_flag equal to 0 does not impose such a constraint. When not present, the value of no_gpm_constraint_flag is inferred to be 1.

# Patent rights declaration(s)

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).